Fm: 	ESS Marketing							Date: 09/30/97

To: 	Maestro-2 customers

Subj:	Maestro-2 & ES1918 Power Management Characteristics

NOTE:

The following measurements are based on the Maestro-2 engineering samples and ES1918 AC97 codec rev. D parts.  These are preliminary numbers and may be changed later with driver fine-tuning. Please check with ESS FAEs for the update.

Maestro-2:

| Operating Mode | Typical IDDD at 3.3V | Power consumption: (IDDD X 3.3V) Watt |
| --- | --- | --- |
| Fully on | 75 mA | 0.248 Watt |
| ASSP clock stopped (1) | 55 mA | 0.182 Watt |
| Every module is clock-stopped except the PCI interface module | 4 mA | 0.013 Watt |
| Every module is clock-stopped and PCI I/F is stopped via CLKRUN# | 700uA | 0.002 Watt |

- ASSP is responsible for FM hardware emulation and digital-filtering algorithm. The clock is controlled by the driver and could be dynamically stopped and resumed, depending on if the ASSP is in use.

ES1918 Rev. D AC97 codec:

| Operating Mode | Typical IDDD at 5.0V | Typical IDDA at 5.0V | D+A combined (IDDD+IDDA) X 5.0V |
| --- | --- | --- | --- |
| Fully On | 9.3mA | 34.1mA | 0.217 Watt |
| Fully Power Down | 1.1mA | 0.00mA | 0.006 Watt |