Trident 4DWAVE-DX / NX

Linux

Software Programmers Guide (SPG)

NOTICE

The information in this document is subject to change, as the Company may make changes to product in order to improve reliability, design, or function, without prior written notice. No part of this manual may be reproduced or transmitted in any form or by any means without the written permission of the company.

In no event will the Company be liable for special incidental or consequential damages, whether arising directly or indirectly, such as loss of profit or good will, that may be suffered in connection with the purchase of this product or from the breach of any representation or warranty.

February 1999, Rev 0.2

Limited Warranty

THE COMPANY DISCLAIMS ALL OTHER EXPRESS, IMPLIED OR STATUTORY WARRANTIES, INCLUDING THE IMPLIED WARRANTIES OF FITNESS FOR A PARTICULAR PURPOSE AND MERCHANTABILITY.

IF SUCH DISCLAIMER OF ANY IMPLIED WARRANTY IS NOT PERMITTED BY LAW, THE DURATION OF ANY SUCH IMPLIED WARRANTIES IS LIMITED TO 90 DAYS FROM THE DATE OF DELIVERY. SOME JURISDICTIONS DO NOT ALLOW THE EXCLUSION OF IMPLIED WARRANTIES OR LIMITATIONS ON HOW LONG AN IMPLIED WARRANTY MAY LAST, OR THE EXCLUSION OR LIMITATION OF INCIDENTAL OR CONSEQUENTIAL DAMAGES, SO SUCH LIMITATIONS OR EXCLUSIONS MAY NOT APPLY TO YOU. THIS WARRANTY GIVES YOU SPECIFIC LEGAL RIGHTS AND YOU MAY ALSO HAVE OTHER RIGHTS WHICH VARY FROM JURISDICTION TO JURISDICTION.

Trident Microsystems, Inc. assumes no responsibility for the use of any circuit other than circuits embodied in a Trident Microsystems, Inc. product.

LICENSE

The Company grants the customer a non-exclusive, non-transferable license to use the software, if any, accompanying this product for internal use on a single computer system. The end user may make a single copy of the software solely for backup purposes; otherwise, no copies may be made of the software or any part thereof. No other license of any kind is granted to any part of the product or any of the intellectual property therein.

TRADEMARK ACKNOWLEDGMENTS

Trident 4DWAVE Linux Software Programmers Guide, Trident™ Microsystems, Inc. 1999. All rights reserved.

Trident Microsystems, Inc. is a registered trademark of Trident Microsystems, Inc.

OPL3 is a registered trademark of Yahama Corporation.

SoundBlaster and SoundBlaster Pro are trademarks of Creative Labs, Inc.

All other product names or trademarks are the property of their respective owners.

Copyright protection claimed includes all forms and matters of copyright table material and information now allowed by statutory or judicial law or hereinafter granted, including without limitation, material generated from the software programs which are displayed on the screen such as icons, screen display looks, etc. Reproduction or disassembly of embedded computer programs or algorithms is prohibited.

Table of Contents

1	Introduction	2

1.1	Advanced 64-Voice PCI Accelerator for Desktop & Notebook	2

1.2	Feature Highlights	3

1.2.1	Advanced PCI Bus-Mastering Accelerator	3

1.2.2	High Quality Wavetable Synthesizer	3

1.2.3	High Quality Audio and AC ’97 Support	3

1.2.4	Advanced Desktop, Notebook PC and Docking Features	3

1.2.5	Advanced Streaming Architecture	3

1.2.6	Multi-Voice Support	4

1.2.7	Extras	4

1.2.8	Power Management	4

1.3	Reference Documents	4

2	DX - System Overview	5

DX - Block Overviews	6

3.1	DX - PCI Interface	6

3.2	DX - Legacy	6

3.3	DX - Stream Buffer	7

3.4	DX - Address Engine	7

3.5	DX - Envelope Engine	7

3.6	DX - Mixer	7

3.7	DX - Capture Channel	7

3.8	DX - AC’97 Interface	8

4	DX - Architecture Constraints	8

4.1	Legacy	8

4.2	64 Channels	8

4.3	PCI Bus Clock	8

4.4	Programming Capture Channels	9

4.5	Using Delay Start Mode	9

4.6	Using SBDELTA/DELTA_R	9

5	DX - Address Map and Register Description	10

5.1	DX - PCI Configuration Space	10

5.1.1	DX - General Configuration	11

5.1.1.1	Vendor ID (Offset = 00h)	11

5.1.1.2	Device ID (Offset = 02h)	11

5.1.1.3	Command (Offset = 04h)	11

5.1.1.4	Status (Offset = 06h)	12

5.1.1.5	Revision ID (Offset = 08h)	12

5.1.1.6	Class Code (Offset = 10h)	12

5.1.1.7	Cache Line Size (Offset = 0Ch)	13

5.1.1.8	Latency Timer (Offset = 0Dh)	13

5.1.1.9	Header Type (Offset = 0Eh)	13

5.1.1.10	BIST (Offset = 0Fh)	13

5.1.1.11	I/O Base Address (Offset = 10h)	13

5.1.1.12	Memory Base Address (Offset = 14h)	13

5.1.1.13	Subsystem Vendor ID (Offset = 2Ch)	14

5.1.1.14	Subsystem ID (Offset = 2Eh)	14

5.1.1.15	Capabilities Pointer (Offset = 34h)	14

5.1.1.16	Interrupt Line (Offset = 3Ch)	14

5.1.1.17	Interrupt Pin (Offset = 3Dh)	14

5.1.1.18	Minimum Grant (Offset = 3Eh)	14

5.1.1.19	Maximum Latency (Offset = 3Fh)	14

5.1.2	DX - Legacy Configuration	15

5.1.2.1	Distributed DMA Configuration (Offset = 40h)	15

5.1.2.2	Legacy I/O Base (Offset = 44h)	15

5.1.2.3	Legacy DMA (Offset = 45h)	16

5.1.2.4	Legacy Control (Offset = 46h)	16

5.1.3	DX - Power Management Configuration	17

5.1.3.1	Capabilities ID (Offset = 48h)	17

5.1.3.2	Next Item Pointer (Offset = 49h)	17

5.1.3.3	Power Management Capabilities (Offset = 4Ah)	17

5.1.3.4	Power Management Control/Status (Offset = 4Ch)	18

5.1.4	DX - Interrupt Snooping Configuration	18

5.1.4.1	Interrupt Snooping Control (Offset = 50h)	18

5.2	4DWave-DX I/O or Memory Space	19

5.3	DX - Wave Register Definition	23

5.3.1	DX - DMA	23

5.3.1.1	DMAR0 (Legacy DMA Playback Buffer Base Register Port1)	23

5.3.1.2	DMAR1 (Legacy DMA Playback Buffer Base Register Port2)	23

5.3.1.3	DMAR2 (Legacy DMA Playback Buffer Base Register Port3)	23

5.3.1.4	DMAR3 (Legacy DMA Playback Buffer Base Register Port4 )	24

5.3.1.5	DMAR4 (Legacy DMA Playback Byte Count Register 1)	24

5.3.1.6	DMAR5 (Legacy DMA Playback Byte Count Register 2)	24

5.3.1.7	DMAR6 (Legacy DMA Playback Byte Count Register 3)	25

5.3.1.8	DMAR8 Legacy DMA Controller Command / Status Register	25

5.3.1.9	DMAR10 Legacy DMA Single Channel Mask Port	25

5.3.1.10	DMAR11 Legacy DMA Channel Operation Mode Register	25

5.3.1.11	DMAR12 Legacy DMA Controller First/Last Flag Clear Port	26

5.3.1.12	DMAR13 Legacy DMA Controller Master Clear Port	27

5.3.1.13	DMAR14 Legacy DMA Controller Clear Mask Port	27

5.3.1.14	DMAR15 Legacy DMA Controller Multi-Channel Mask Register	27

5.3.2	DX - Sound Blaster / Adlib	27

5.3.2.1	SBR0 (Legacy FmMusic Bank 0 Register Index / Legacy FmMusic Status)	27

5.3.2.2	SBR1/SBR3 (Legacy FmMusic Bank 0/1 Register Data Port)	28

5.3.2.3	SBR2 (Legacy FmMusic Bank 1 Register Index)	28

5.3.2.4	SBR4 (Legacy Sound Blaster Mixer Register Index)	28

5.3.2.5	SBR5 (Legacy Sound Blaster Mixer Register Data Port)	28

5.3.2.6	SBR6 (Legacy Sound Blaster ESP Reset Port)	29

5.3.2.7	SBR7 (Legacy Sound Blaster ESP Data Port)	29

5.3.2.8	SBR8 (Legacy Sound Blaster Command / Status Port)	29

5.3.2.9	SBR9 (Legacy Sound Blaster ESP Data Ready / IRQ Acknowledge Port 1)	30

5.3.2.10	SBR10 (Legacy Sound Blaster ESP Data Ready / IRQ Acknowledge Port 2 )	30

5.3.3	DX - MPU-401 UART	30

5.3.3.1	MPUR0 (Legacy MPU-401 Data Port / IRQ Acknowledge Port)	30

5.3.3.2	MPUR1 (Legacy MPU-401 Command / Status Port)	31

5.3.3.3	MPUR2 (MPU-401 Operation Control  / Status Register)	32

5.3.3.4	MPUR3 (MPU-401 MIDI-IN FIFO Access Port)	33

5.3.4	DX - Game Port	33

5.3.4.1	GAMER0 (Gameport Control Register)	33

5.3.4.2	GAMER1 (Legacy Gameport I/O Register)	33

5.3.4.3	GAMER2 (Enhanced Gameport Position Register 1)	34

5.3.4.4	GAMER3 (Enhanced Gameport Position Register 2)	35

5.3.5	DX - AC’97 Registers	35

5.3.5.1	ACR0 AC’97 Codec Write Register	35

5.3.5.2	ACR1 AC’97 Codec Read Register	35

5.3.5.3	ACR2 AC’97 Command/Status Register	36

5.3.6	DX - General	37

5.3.6.1	ASR0 4D Wave Status Register	37

5.3.6.2	ASR1  Legacy Sound Blaster Frequency Read Back Register	38

5.3.6.3	ASR2 Legacy Sound Blaster Time Constant Read Back Register	39

5.3.6.4	ASR3 4D Wave Scratch Register	39

5.3.6.5	ASR4 4D Wave Version Control Register	39

5.3.6.6	ASR5 SB ESP Version High Byte Control Register	39

5.3.6.7	ASR6 SB ESP Version Low Byte Control Register	39

5.3.7	OPL3 Channel Status Register	40

5.3.7.1	AOPLSR0 OPL3 Emulation Channel Keyon/off Trace Register	40

5.3.8	DX - Channel Registers	40

5.3.8.1	CI0, RCI1, RCI2 Record Channel Index 0 (Mixer), 1 (Reverb), & 2 (Chorus)	40

5.3.8.2	PSBVLD_A Bank A PCI Stream Buffer Valid Flags	41

5.3.8.3	PSBVLD_B Bank A PCI Stream Buffer Valid Flags	41

5.3.8.4	START_A Bank A Start Command and Status Register	41

5.3.8.5	STOP_A Bank A STOP Command and Status register	42

5.3.8.6	DLY_A Delay flag register (Only used with Bank A channels)	42

5.3.8.7	SIGN_CSO_A Sign bit of CSO  (Only used with Bank A channels)	43

5.3.8.8	CSPF_A Bank A Current Sample Position Flag	43

5.3.8.9	CEBC_A Current Envelope Buffer Control (Only used with Bank A channels)	44

5.3.8.10	AIN_A Bank A Address Engine Interrupt Register	44

5.3.8.11	EINT_A Envelope Engine Interrupt Register (Only used with Bank A channels)	45

5.3.8.12	LFO_A & GC & CIR: Bank A Low Frequency Oscillator & Global Control & Channel Index	45

5.3.8.13	AINTEN_A Bank A Address Engine Interrupt Enable Control	48

5.3.8.14	MUSICVOL & WAVEVOL: Global Music Volume Control & Global Wave Volume Control	49

5.3.8.15	SBDELTA & DELTA_R: Sample Change Step for Legacy Sound Blaster Voice In/Out & Sample change step for recording	49

5.3.8.16	MISCINT Recording/Playback Underrun/Recording Overrun Interrupt Register	49

5.3.8.17	START_B Bank B Start Command and Status Register	51

5.3.8.18	STOP_B Bank B STOP Command and Status register	51

5.3.8.19	CSPF_B Bank B Current Sample Position Flag	52

5.3.8.20	SBBL & SBCL: SoundBlaster DMA Base Length & SoundBlaster DMA Current Length	52

5.3.8.21	SBCTRL & SBE2R & SBDD: SoundBlaster Control & SoundBlaster DMA Testing Byte Data & SoundBlaster Direct Playback Data	52

5.3.8.22	STIMER Sample Timer	53

5.3.8.23	ROM Test	54

5.3.8.24	LFO_B Bank B Low Frequency Oscillator	54

5.3.8.25	Test Mixer FIFO	54

5.3.8.26	Test Mixer Accumulator	55

5.3.8.27	AIN_B Bank B Address Engine Interrupt Register	55

5.3.8.28	AINTEN_B	55

5.3.9	DX - Indexed Channel Registers	56

5.3.9.1	CSO & ALPHA & FMS: Current Sample Offset & Sample Interpolation Coefficient & Frequency Modulation Step (Bank A & B)	56

5.3.9.2	PPTR & LBA: PSB Pointer & Loop Begin Address (Bank A & B)	56

5.3.9.3	ESO & DELTA: End Sample Offset &  Delta Sample Rate Ratio (Bank A & B)	56

5.3.9.4	FMC & RVOL & CVOL: FM Control, Reverb Volume & Chorus Volume (Bank A & B)	56

5.3.9.5	GVSEL & PAN & VOL & CTRL & Ec: Global Volume Select & Positioning Attenuation & Volume Attenuation & Control & Current Envelope (Bank A & B)	57

5.3.9.6	EBUF1 (Bank A Only)	58

5.3.9.7	EBUF2 (Bank A Only)	59

6	NX - System Overview	61

7	NX - Block Overviews	62

NX - PCI Interface	62

7.2	NX - Legacy	63

7.3	NX - Stream Buffer	63

7.4	NX - Address Engine	63

7.5	NX - Envelope Engine	63

7.6	NX - Mixer	63

7.7	NX - Capture Channel	64

7.8	NX - I2S Interface	64

7.8.1	System Environment	64

7.8.2	Software Programming Model	64

7.9	NX - AC’97 Interface	64

7.10	NX - SPDIF Interface	65

7.10.1	System Environment	65

7.10.2	Software Programming Model	65

7.11	NX - EEPROM Interface	65

8	NX - Power Management	66

8.1	References	66

8.2	Overview	66

8.3	State Definitions	66

8.3.1	States (Defined in OnNow Device Class Specification)	66

8.3.2	Caveats & Assumptions	66

Power Management State Diagram	67

8.4	Audio Power State Definitions	67

8.4.1	D0	67

8.4.2	D1	68

8.4.3	D2	68

8.4.4	D3	69

8.4.5	D4	69

9	NX - Architecture Constraints	70

9.1	Legacy	70

9.2	64 Channels	70

9.3	PCI Bus Clock	70

9.4	Programming Capture Channels	70

9.5	Using Delay Start Mode	71

9.6	Using SBDELTA/DELTA_R	71

10	NX - Internal RAM/EEPROM Configuration	72

10.1	NX - Internal ARAM & ERAM Mapping	72

NX - Internal TLB Cache Mapping	72

10.3	NX - Internal PSB Audio Sample Cache Mapping	72

10.4	NX - External EEPROM Mapping	73

11	NX - Address Map and Register Description	74

11.1	NX - PCI Configuration Space	74

11.1.1	NX - General Configuration	75

11.1.1.1	Vendor ID (Offset = 00h)	75

11.1.1.2	Device ID (Offset = 02h)	75

11.1.1.3	Command (Offset = 04h)	75

11.1.1.4	Status (Offset = 06h)	76

11.1.1.5	Revision ID (Offset = 08h)	76

11.1.1.6	Class Code (Offset = 10h)	76

11.1.1.7	Cache Line Size (Offset = 0Ch)	77

11.1.1.8	Latency Timer (Offset = 0Dh)	77

11.1.1.9	Header Type (Offset = 0Eh)	77

11.1.1.10	BIST (Offset = 0Fh)	77

11.1.1.11	I/O Base Address (Offset = 10h)	77

11.1.1.12	Memory Base Address (Offset = 14h)	77

11.1.1.13	Subsystem Vendor ID (Offset = 2Ch)	78

11.1.1.14	Subsystem ID (Offset = 2Eh)	78

11.1.1.15	Capabilities Pointer (Offset = 34h)	78

11.1.1.16	Interrupt Line (Offset = 3Ch)	78

11.1.1.17	Interrupt Pin (Offset = 3Dh)	78

11.1.1.18	Minimum Grant (Offset = 3Eh)	78

11.1.1.19	Maximum Latency (Offset = 3Fh)	78

11.1.2	NX - Legacy Configuration	79

11.1.2.1	Distributed DMA Configuration (Offset = 40h)	79

11.1.2.2	Legacy I/O Base (Offset = 44h)	79

11.1.2.3	Legacy DMA (Offset = 45h)	80

11.1.2.4	Power Management Control (Offset = 46h)	80

11.1.3	NX - Power Management Configuration	81

11.1.3.1	Capabilities ID (Offset = 48h)	81

11.1.3.2	Next Item Pointer (Offset = 49h)	81

11.1.3.3	Power Management Capabilities (Offset = 4Ah)	81

11.1.3.4	Power Management Control/Status (Offset = 4Ch)	82

11.1.3.5	Power Management Data (Offset = 4Fh)	82

11.1.4	NX - Interrupt Snooping Configuration	82

11.1.4.1	Interrupt Snooping Control (Offset = 50h)	82

11.1.4.2	Notebook Mode Control (Offset = 70h)	83

11.1.4.3	GPIO Mode Control (Offset = 72h)	83

11.2	4DWave-NX I/O or Memory Space	84

11.3	NX - Wave Register Definition	88

11.3.1	NX - DMA	88

11.3.1.1	DMAR0 (Legacy DMA Playback Buffer Base Register Port1)	88

11.3.1.2	DMAR1 (Legacy DMA Playback Buffer Base Register Port2)	88

11.3.1.3	DMAR2 (Legacy DMA Playback Buffer Base Register Port3)	88

11.3.1.4	DMAR3 (Legacy DMA Playback Buffer Base Register Port4 )	89

11.3.1.5	DMAR4 (Legacy DMA Playback Byte Count Register 1)	89

11.3.1.6	DMAR5 (Legacy DMA Playback Byte Count Register 2)	89

11.3.1.7	DMAR6 (Legacy DMA Playback Byte Count Register 3)	90

11.3.1.8	DMAR8 Legacy DMA Controller Command / Status Register	90

11.3.1.9	DMAR10 Legacy DMA Single Channel Mask Port	90

11.3.1.10	DMAR11 Legacy DMA Channel Operation Mode Register	91

11.3.1.11	DMAR12 Legacy DMA Controller First/Last Flag Clear Port	91

11.3.1.12	DMAR13 Legacy DMA Controller Master Clear Port	92

11.3.1.13	DMAR14 Legacy DMA Controller Clear Mask Port	92

11.3.1.14	DMAR15 Legacy DMA Controller Multi-Channel Mask Register	92

11.3.2	NX - Sound Blaster / Adlib	92

11.3.2.1	SBR0 (Legacy FmMusic Bank 0 Register Index / Legacy FmMusic Status)	92

11.3.2.2	SBR1/SBR3 (Legacy FmMusic Bank 0/1 Register Data Port)	93

11.3.2.3	SBR2 (Legacy FmMusic Bank 1 Register Index)	93

11.3.2.4	SBR4 (Legacy Sound Blaster Mixer Register Index)	93

11.3.2.5	SBR5 (Legacy Sound Blaster Mixer Register Data Port)	94

11.3.2.6	SBR6 (Legacy Sound Blaster ESP Reset Port)	94

11.3.2.7	SBR7 (Legacy Sound Blaster ESP Data Port)	94

11.3.2.8	SBR8 (Legacy Sound Blaster Command / Status Port)	94

11.3.2.9	SBR9 (Legacy Sound Blaster ESP Data Ready / IRQ Acknowledge Port 1)	95

11.3.2.10	SBR10 (Legacy Sound Blaster ESP Data Ready / IRQ Acknowledge Port 2 )	95

11.3.3	NX - MPU-401 UART	95

11.3.3.1	MPUR0 (Legacy MPU-401 Data Port / IRQ Acknowledge Port)	95

11.3.3.2	MPUR1 (Legacy MPU-401 Command / Status Port)	96

11.3.3.3	MPUR2 (MPU-401 Operation Control  / Status Register)	97

11.3.3.4	MPUR3 (MPU-401 MIDI-IN FIFO Access Port)	98

11.3.4	NX - SPDIF	98

11.3.4.1	SPDIF Control/CSO	98

11.3.4.2	SPDIF LBA	99

11.3.4.3	SPDIF ESO	99

11.3.5	NX - Game Port	99

11.3.5.1	GAMER0 (Gameport Control Register)	99

11.3.5.2	GAMER1 (Legacy Gameport I/O Register)	100

11.3.5.3	GAMER2 (Enhanced Gameport Position Register 1)	101

11.3.5.4	GAMER3 (Enhanced Gameport Position Register 2)	101

11.3.6	NX - I2S Registers	101

11.3.6.1	I2SR I2S Interface Control	101

11.3.7	NX - AC’97 Registers	102

11.3.7.1	ACR0 AC’97 Command/Status Register	102

11.3.7.2	ACR1 Primary & Secondary AC’97 Codec Write Register	106

11.3.7.3	ACR2 Primary AC’97 Codec Read Register	106

11.3.7.4	ACR3 Secondary AC’97 Codec Read Register	107

11.3.8	NX - General	108

11.3.8.1	ASR0 4DWave Status Register	108

11.3.8.2	ASR1  Legacy Sound Blaster Frequency Read Back Register	110

11.3.8.3	ASR2 Legacy Sound Blaster Time Constant Read Back Register	110

11.3.8.4	ASR3 4DWave Miscellaneous Legacy Control	110

11.3.8.5	ASR4 4DWave Version Control Register	111

11.3.8.6	ASR5 SB ESP Version High Byte Control Register	111

11.3.8.7	ASR6 SB ESP Version Low Byte Control Register	111

11.3.9	NX - OPL3 Channel Status Register	111

11.3.9.1	AOPLSR0 OPL3 Emulation Channel Keyon/off Trace Register	111

11.3.10	NX - SPDIF Channel Status Register	112

11.3.10.1	SPCSTATUS	112

11.3.11	NX - SERR IO Trap Register	112

11.3.11.1	SERRIO	112

11.3.12	NX - Channel Registers	113

11.3.12.1	TLBC - TLB Control Register	113

11.3.12.2	RCI 0, 1, 2, 3 Record Channel Index 0 (Mixer), 1 (Reverb), 2 (Chorus), & 3 (AC’97)	114

11.3.12.3	LCI Loopback Channel Index (I2S)	115

11.3.12.4	PSBVLD_A Bank A PCI Stream Buffer Valid Flags	115

11.3.12.5	PSBVLD_B Bank B PCI Stream Buffer Valid Flags	115

11.3.12.6	START_A Bank A Start Command and Status Register	115

11.3.12.7	STOP_A Bank A STOP Command and Status register	116

11.3.12.8	DLY_A Delay flag register (Only used with Bank A channels)	117

11.3.12.9	SIGN_CSO_A Sign bit of CSO  (Only used with Bank A channels)	117

11.3.12.10	CSPF_A Bank A Current Sample Position Flag	118

11.3.12.11	CEBC_A Current Envelope Buffer Control (Only used with Bank A channels)	118

11.3.12.12	AIN_A Bank A Address Engine Interrupt Register	119

11.3.12.13	EINT_A Envelope Engine Interrupt Register (Only used with Bank A channels)	119

11.3.12.14	LFO_A & GC & CIR: Bank A Low Frequency Oscillator & Global Control & Channel Index	120

11.3.12.15	AINTEN_A Bank A Address Engine Interrupt Enable Control	123

11.3.12.16	MUSICVOL & WAVEVOL: Global Music Volume Control & Global Wave Volume Control	123

11.3.12.17	SBDELTA & DELTA_R: Sample Change Step for Legacy Sound Blaster Voice In/Out & Sample change step for recording	124

11.3.12.18	MISCINT Recording/Playback Underrun/Recording Overrun Interrupt Register	124

11.3.12.19	START_B Bank B Start Command and Status Register	126

11.3.12.20	STOP_B Bank B STOP Command and Status register	126

11.3.12.21	CSPF_B Bank B Current Sample Position Flag	127

11.3.12.22	SBBL & SBCL: SoundBlaster DMA Base Length & SoundBlaster DMA Current Length	127

11.3.12.23	SBCTRL & SBE2R & SBDD: SoundBlaster Control & SoundBlaster DMA Testing Byte Data & SoundBlaster Direct Playback Data	128

11.3.12.24	STIMER Sample Timer	129

11.3.12.25	ROM Test	129

11.3.12.26	LFO_B Bank B Low Frequency Oscillator	129

11.3.12.27	Test Mixer FIFO	129

11.3.12.28	Test Mixer Accumulator	130

11.3.12.29	AIN_B Bank B Address Engine Interrupt Register	130

11.3.12.30	AINTEN_B	130

11.3.13	NX - Indexed Channel Registers	131

11.3.13.1	DELTA & CSO: Delta Sample Rate Ratio (LSB) & Current Sample Offset (Bank A & B)	131

11.3.13.2	PPTR & LBA: PSB Pointer & Loop Begin Address (Bank A & B)	131

11.3.13.3	DELTA & ESO: Delta Sample Rate Ratio (MSB) & End Sample Offset (Bank A & B)	131

11.3.13.4	ALPHA & FMS & FMC & RVOL & CVOL: Alpha, FM Step, FM Control, Reverb Volume & Chorus Volume (Bank A & B)	132

11.3.13.5	GVSEL & PAN & VOL & CTRL & Ec: Global Volume Select & Positioning Attenuation & Volume Attenuation & Control & Current Envelope (Bank A & B)	132

11.3.13.6	EBUF1 (Bank A Only)	133

11.3.13.7	EBUF2 (Bank A Only)	135

# Introduction

## Advanced 64-Voice PCI Accelerator for Desktop & Notebook

The 4DWAVE is an advanced 64-voice PCI audio accelerators providing full legacy compatibility, hardware wavetable synthesis, and streaming wave acceleration on a single chip for the high-performance, cost-sensitive consumer, commercial and notebook PCs. It is named 4DWAVE as it adds time as the 4th dimension to its interactive 3D positional audio wave streams. The time element includes such effects processing as adding Doppler, Chorus, and Reverb effects on top of the 3D positional audio and wavetable streams.

The 4DWAVE integrates a 64-voice wavetable engine with per voice effect processing capability.

The 4DWAVE utilizes a Digital Enhanced Game Port, when coupled with an advanced game port driver, can save up to 12% of the CPU overhead nominally required by a conventional analog game port. The 4DWAVE employs a high precision 26-bit digital mixer, providing an accurate 20-bit output and higher than 90dB signal-to-noise ratio when used with a high quality AC ’97 codec.

The 4DWAVE uses fully static design and is designed with aggressive power management in mind.  It is both ACPI-compliant and PCI Power Management (PCI-PM)-compliant.  Its hardwired-DSP approach provides the performance without the cost and power overhead of a programmable DSP engine.  With a low power 3.3V process and a space-efficient 100 LQFP package, the 4DWAVE is optimized for motherboard integration, Notebook systems and embedded applications.

The 4DWAVE-NX provides optimal support for Desktop as well as Notebook design.  For desktop, the NX supports S/PDIF digital interface to consumer electronic (CE) devices such as A/V Amplifier for high quality stereo or Digital Dolby (AC-3) audio playback.  It is AC97 Rev 1.03 and 2.0 compliant and its dual AC97 support enables 4-channel output for ultimate realism in desktop design or “digital hot docking” for notebook / docking station design.  Advanced notebook designs using more than four PCI bus masters will appreciate NX’s on-chip PCI Bus Master arbiter.  Its Dual I2S interface directly support up to two ZV-port without external glue logic, reducing cost and real estate.  Also, its flexible Dual AC’97 architecture enables resources such as speakers, microphone, CD-ROM, line input / output to be located at either the notebook or docking station, providing user friendly interface and effective resource utilization.

The 4DWAVE delivers an impressive combination of features and performance to end-users without burdening them on price. By combining PCI Bus Mastering for streaming Wave (PCM) acceleration, Hardware Wavetable synthesizer, and Digital Enhanced Game Port the 4DWAVE provides up to a 50% system level performance enhancement over an equivalent ISA audio controller. It delivers high performance, high quality audio, high-end features with efficient power management in a single-chip in a space-efficient 100 LQFP package.

It is forward pin-compatible with the future generation 4DWAVE family controller. This document will briefly describe signals in future products to enable system designers to accommodate the future 4DWAVE family products with one design, substantially simplifying future design and testing efforts.

## Feature Highlights

### Advanced PCI Bus-Mastering Accelerator

- PCI 2.1-compliant with Bus Mastering optimized for multiple streams operation

- On-chip per voice FIFO minimizes PCI bandwidth

- Up to 20X improvement over ISA DMA on PCI bus bandwidth utilization

- Hardware multi-channel 26-bit digital mixer

### High Quality Wavetable Synthesizer

- 64 voices polyphony wavetable synthesizer supports all combinations wavetable samples formats :

- Stereo/mono

- 8-/16-bits

- Signed/unsigned

- Per channel volume and envelope control, pitch shift, tremolo, and vibrato

- Per channel effect processing and effect volume control for reverb, chorus and echo

- DLS1-compliant Downloadable Samples support

### High Quality Audio and AC ’97 Support

- AC97 Rev 1.03 and Rev 2.00 compliant

- (NX) Support Dual AC97 codec for 4-channel output in desktop and digital docking for notebook design

- CD quality audio with better than 90dB SNR using an external high quality AC ’97 codec

- AC ’97 support with full duplex, independent sample rate converter for audio recording and playback

- On-chip sample rate converter ensures all internal operation at 48KHz

- High precision internal 26-bit digital mixer with 16- and 20-bit digital audio output

### Advanced Desktop, Notebook PC and Docking Features

- (NX) S/PDIF interface provides the ability to deliver digital stereo or compressed AC-3 audio stream or output of NX digital mixer to consumer electronics (CE) such as A/V amplifier for high quality playback.   Supports 32, 44.1 and 48KHz sampling rate

- (NX) Dual AC-97 supports up to 4-channel output for maximum realism

- (NX) Dual AC-97 supports Digital Hot Docking for advanced Notebook / Docking station design

- (NX) Dual I2S interfaces support advanced notebook with dual ZV-port without using external glue logic

- (NX) On-chip PCI Bus Master arbiter supports notebook design that requires additional PCI bus master such as DVD or PCI video capture device

- (NX) External EEPROM supports Subsystem ID, Subsystem Vendor ID and power management info

- (NX) Up to 3 GPIOs

### Advanced Streaming Architecture

- Streaming architecture with “Re-routable endpoint” support

- Three stereo capture channels

- AC ’97 stereo recording channel through AC-link

### Multi-Voice Support

- 64 Stereo Hardware channels

- 3D accelerator with IID, ITD, and Doppler effects on 3D positional audio buffers

- PCM accelerator for volume, pan, and pitch shift control on streaming or static buffers

### Extras

- Digital Enhanced Game port enables analog joystick to emulate digital joystick performance using enhanced game port driver. This eliminates up to 12% of CPU overhead wasted on joystick polling.

- Multimedia timer for video/audio synchronization

- Forward pin-compatible with next generation PCI audio accelerators

### Power Management

- Desktop ACPI & PCI-PM (PCI Power Management) Compatible

- Software Controls AC ’97 Codec Power States

## Reference Documents

- PCI Local Bus Specification, Revision 2.1, June 1, 1995

- ACPI - Advanced Configuration and Power Interface Specification, Revision 1.0

- PPMI - PCI Bus Power Management Interface Specification – Revision 1.0; June 30, 1997

- OnNow - Device Class Power Management Reference Specification, Audio Device Class V1.0

- AC ’97 - Audio Codec ’97 Component Specification, Revision 1.03, September 15, 1996

# DX - System Overview

The 4D Wave DX is a 64-Voice PCI Wavetable/DirectSound Audio Accelerator.  It is packaged in a 100-pin LQFP targeted at the desktop PC market and is designed to be pin compatible with Trident's next generation desktop audio controller.  As shown in Figure 1 below, the 4D Wave DX will interface to:

- PCI bus (variable frequency 25 - 33 MHz)

- A single AC'97 serial bus for communication with an AC'97 Codec

- A Digital Enhanced Game Port

- MIDI port

- 4DWave DX High Level System Diagram

There will be a single driver set for the 4D Wave devices (current and future).  The approach will allow a stable, maintainable code base.  Support of typical audio features is handled variously by hardware and driver software.  The current split of functionality is detailed below.

Hardware acceleration:

Wavetable synthesis

chorus effects

reverb effects partially accelerated in hardware

Driver software support:

FM Synthesis (VirtualFM)

MIDI (VirtualGM)

Reverb filtering

3D positional audio effects

The 3D positional audio effects will be handled by code from a third-party inserted into the 4D Wave DX driver.  The Microsoft provided HEL layer for 3D audio will not be used.

# DX - Block Overviews

- 4D Wave DX Block Diagram

## DX - PCI Interface

The PCI interface is completely PCI 2.1 compliant. By being PCI 2.1 compliant, the device is also Plug-n-Play compliant. It consists of separate master and slave controllers that operate independently.  Both master and slave support burst cycles. The slave can be programmed to accept both I/O and Memory (memory mapped I/O) cycles for 4D Wave registers. The 4D Wave register space can be mapped by configuring the PCI Config registers for I/O and Memory base addressing. The legacy register space is only accessible through I/O cycles and cannot be remapped.

4D Wave uses a single PCI interrupt. All interrupts are combined together internally to form this single interrupt signal. Internal registers must be accessed to determine the nature of the interrupt. This function is programmed differently depending if the device is in legacy emulation mode or not.

The device can be configured to use the Distributed DMA mechanism to perform DMA I/O trapping, programming and operation. This can only be used with system chipsets that support the DDMA protocol. The device conforms to the DDMA 6.0 specification.

When legacy DMA support is required and the chipset does not support DDMA, the 4D Wave device performs snooping of the DMA I/O programming cycles. It uses this to configure itself as a PCI Master and then performs the DMA cycle. The device will also capture some DMA I/O read cycles to respond with the correct status information.

## DX - Legacy

The 4D Wave supports both the SoundBlaster Pro and SoundBlaster 16 register sets. This includes the OPL3 RAM. When configured to support legacy operation, the device will respond to all I/O cycles in the legacy address regions. These are used to support both FM and General MIDI emulation in DOS.

The VirtualGM and VirtualFM technologies are based on hardware and software to provide the emulation of FM synthesis and General MIDI command interpretation in DOS or Windows. The hardware provides all the registers for reads and writes to legacy locations. Depending on the register or bit written, the hardware will engage the software to generate the correct result. This maintains complete legacy support at a low cost to the user.

The 4D Wave also support the legacy Game Port. This is the 8-bit joystick port found on most audio accelerators and many “Super I/O” chips. In addition to supporting legacy operation, the game port was enhanced to allow a dramatic reduction in both bus traffic and CPU utilization by removing the requirement of “polling”. This support can be found when using Microsoft’s DirectInput API.

The MIDI port is supported with an MPU-401 compatible UART. This port can also be used in an emulation mode (VirtualGM) to support synthesis in DOS mode games.

## DX - Stream Buffer

The stream buffer is used to buffer the data streams between the accelerator engine and the system memory. The stream buffer supports up to 64 channels with each having 4 Dwords of buffering. (Analysis was done that shows this is the “sweet spot” for cost versus performance.) The stream buffer is primarily used for playback of up to 64 streams of audio or effects. However, it can be also configured to support three (3) channels of capture data for WDM, chorus, and reverb.

## DX - Address Engine

The address engine supports 64 voice channels. Stereo, mono, 8-bit, and 16-bit formats are supported. The first 32-channels, designated Bank A, have a complete parameter set to support MIDI (GM only requires 24 voices). The second 32-channels, designated Bank B, are optimized for effect processing and DirectX/WDM audio streams.

Two LFOs are used for music synthesis. The Bank A LFO is used with MIDI for tremelo (FM) and vibrato (AM). The Bank B LFO is used for chorus or effect pitch shifting. This means that the chorus playback taps must be assigned to the Bank B voices.

Each channel is sample rate converted to 48kHz. The address engine performs all the sample address calculation including using a channel specific sample rate conversion factor and a LFO sine wave to offset the address to simulate a tremolo (FM) affect.

The LFO used by the address engine is passed on to the envelope engine to be used as a vibrato (AM) effect.

## DX - Envelope Engine

The envelope engine controls the channel volume. For the first 32 channels, it supports two global volumes, a per/channel volume, a left-right PAN, a chorus volume, and a reverb volume. The first set also includes two volume slope buffers to allow a MIDI ADSR (Attack-Decay-Sustain-Release) curve to by performed. The second 32 channels only support a per/channel volume, a left-right PAN, a chorus volume, and a reverb volume. All volume controls operate in dBs (decibels) of attenuation. This allows the attenuations to simply be summed before sending the composite to the mixer.

## DX - Mixer

The mixer supports a 26-bit accumulation on three separate stereo channels. The hi-resolution accumulation allows the 64 voices to be completely accumulated without degradations such as “clipping”. There is a separate mixer channel for main (AC’97 stereo out), reverb, and chorus.

The main mixer channel data is buffered in a FIFO before being accessed by the AC’97 interface. This allows the 4D Wave device to get several samples ahead and “averages” out the bus delay caused by other PCI or system devices.

## DX - Capture Channel

The capture channel is an AC’97 specific channel. It is used to capture record data samples from the AC’97 codec. The capture channel also supports independent down sampling and format conversion. All AC’97 data is sampled at 48kHz and can be captured in 16-bit stereo. This requires 4-bytes per sample or 192K bytes for 1 second of record data. By down sampling, the bus and memory space can be conserved. For instance, most voice functions can use 8-bit mono samples at 8kHz. This record data stream only uses 8K bytes for 1 second.

## DX - AC’97 Interface

The AC’97 interface supports the 5-pin AC’Link interface to the codec. The interface currently only supports the standard AC’97 codec configuration: stereo out and stereo in. The interface also includes a register set that allows access to the external AC’97 codec registers. Power management control is also included.

The AC’97 interface operates at a fixed 48 kHz sample rate. The 4D Wave will provide 20-bit stereo output for playback and will accept 16-bit stereo input for record.

# DX - Architecture Constraints

## Legacy

The legacy support of Sound Blaster assumes the OPL3 RAM exists in hardware. For die size & cost reasons, a single RAM is used for both the OPL3 emulation and the Bank B (Second 32 voices) playback channels. This means that when the 4D Wave is emulating the OPL3 in hardware, that the Bank B channels should all be disabled and not used. This should only occur during a DOS-Box or full DOS mode. Likewise, when there are active channels in Bank B, OPL3 emulation is not available in the hardware. This must be emulated in software through I/O trapping.

## 64 Channels

The 64 Channels of audio can cause some issues with PCI bandwidth and utilization. The 4D Wave architecture was optimized for observed scenarios of high end games and music playback. However, “psychotic” scenarios can be construed that are fundamentally legal (although they typically result is very distorted results) which could effect system performance. This is unavoidable due to the nature of audio stream when taken to the edges of its configurability and capability. For instance, “up sampling” an audio stream reduces the effectiveness of bursting by not using most of the data transferred. 4D Wave uses bursting (as do most other advanced Audio Accelerators) to reduce the overall system load. If a few channels are significantly (3 octaves) up-sampled, there are no issues. However, if all 64 channels are taken to this extreme, then the PCI bus is at its limit to try and transfer the data to the accelerator. Care must be taken to understand and possibly off-load the synthesis of these streams, in some cases, from the accelerator to the CPU.

## PCI Bus Clock

The 4D Wave algorithm engines operate off of the PCI clock. This allows lower power and reduces the cost by not requiring a PLL or another clock. The downside to this is that the 4D Wave is dependent on this clock for performance. A 10% drop in the PCI clock (30MHz), results in a 10% drop in the 4D Wave performance. The 4D Wave is designed with over 25% overhead. However, if the 4D Wave was put in a 25MHz (~30% reduction) PCI system, it may not be able to process all 64 channels. Likewise, the PCI Bus would potentially not have the bandwidth to transfer the data.

## Programming Capture Channels

When programming a 4D Wave channel to act as a capture channel there are several special settings that must be followed.

- LBA (the start address of the capture buffer) must be 4 dword aligned.

- When in non-loop mode ESO = # of samples to capture.

- When in loop mode ESO = (# of samples) – 1

- DELTA must always be programmed to 0x1000. 4D Wave only allows capturing at a 48KHz rate.

- CSO (current sample offset) must be 0

- ALPHA must be 0

- FMS must be 0 (no frequency modulation can be applied to the capture buffer).

- The data type must always be set to 16-bit signed stereo. 4D Wave does not support capturing in any other format.

- VOL (channel volume) must be 0xFF (mute).

- RVOL (reverb volume) must be 0x7F (mute).

- CVOL (chorus volume) must be 0x7F (mute).

- When programming RVOL and CVOL (AudioBase + 0xEC) only word writes are allowed. If a dword access is done to this address the EC value for the previous and current channel may be corrupted.

## Using Delay Start Mode

When programming a channel to use the Delay Start Mode feature (EBUF mode) the programmer must first set the channel’s delay bit in the DLY_A register (0x88) before setting the channel’s start bit.

## Using SBDELTA/DELTA_R

When setting up 4D Wave to handle legacy playback or record the SBDELTA (reg 0xAC) must be used. During legacy playback SBDELTA is (playback frequency)/48KHz in a 4.12 format. When programming legacy record SBDELTA is the inverse of its playback counter part: 48KHz/(record frequency) in a 4.12 format.

# DX - Address Map and Register Description

The 4D Wave DX has addressable locations in three address spaces: PCI Configuration Space, I/O Space, and Memory Space. These spaces are initialized through Plug-n-Play system software routines by configuring the registers in the 256-byte PCI Configuration Register space.

## DX - PCI Configuration Space

The 4D Wave DX is a single function device and, therefore, only supports a single PCI configuration space. The device is both PPMI and DDMA compatible, which requires additional registers for set-up. The device does not support an external Serial EEPROM, therefore, Subsystem Vendor ID, Subsystem ID and PPMI “Data” values (Power Consumption and Dissipation) are not supported.

- 4D Wave DX PCI Configuration Register Space

| Offset (Hex) | Byte 3 | Byte 2 | Byte 1 | Byte 0 |
| --- | --- | --- | --- | --- |
| 00h | Device ID (Read Only = 2000h) |  | Vendor ID (Read Only = 1023h) |  |
| 04h | Status |  | Command |  |
| 08h | Class Code (Read Only = 040100h) |  |  | Revision ID (Read Only = 00h) |
| 0Ch | BIST (Read Only = 0000h) | Header Type (Read Only = 00h) | Latency Timer | Cache Line Size (Read Only = 0000h) |
| 10h | Audio IO Base Address Register |  |  |  |
| 14h | Audio Memory Base Address Register |  |  |  |
| 18-28h | RSVD (Read Only = 00000000h) |  |  |  |
| 2Ch | Subsystem ID (Read Only = 2000h) |  | Subsystem Vendor ID (Read Only = 1023h) |  |
| 30h | RSVD (Read Only = 00000000h) |  |  |  |
| 34h | RSVD (Read Only = 000000h) |  |  | Cap_Ptr (Read Only = 48h) |
| 38h | RSVD (Read Only = 00000000h) |  |  |  |
| 3Ch | MAX_LAT (Read Only = 05h) | MIN_GNT (Read Only = 02h) | Interrupt Pin (Read Only = 01h) | Interrupt Line |
| 40h | DDMA_CFG |  |  |  |
| 44h | RSVD (Read Only = 00h) | LEGACY_CTRL | LEGACY_DMA | LEGACY_IOBASE |
| 48h | Power Management Capabilities (Read Only = 0601h |  | Next_Ptr (Read Only = 00h) | Cap_ID (Read Only = 01h) |
| 4Ch | Power Value Data (Read Only = 00h) | PMCSR_BSE (Read Only – 00h) | Power Management Control/Status |  |
| 50h | RSVD (Read Only = 0000h) |  | Interrupt Snooping Control |  |

### DX - General Configuration

#### Vendor ID (Offset = 00h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [15:0] | 1023h | R | Trident Microsystems’s PCI Vendor ID |

#### Device ID (Offset = 02h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [15:0] | 2000h | R | 4D Wave DX Device ID |

#### Command (Offset = 04h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [15:10] | 00000b | R | Reserved |
| [9] | 0 | R | Fast Back-to-Back enable for master transactions. The 4D Wave does not support this feature. This bit is hard wired to a ‘0’. |
| [8] | 0 | R/W | SERR# enable. 0 = Disables the SERR# Driver 1 = Enables the SERR# Driver |
| [7] | 0 | R | Address / Data stepping or Wait cycle control. The 4D Wave does not support this feature. This bit is hard wired to a ‘0’. |
| [6] | 0 | R/W | Parity Enable. 0 = Ignores parity errors. 1 = Report parity errors. |
| [5] | 0 | R | VGA palette snoop. The 4D Wave does not support this feature. This bit is hard wired to a ‘0’. |
| [4] | 0 | R | Enable the “Memory Write and Invalidate” command. The 4D Wave does not support this feature. This bit is hard wired to a ‘0’. |
| [3] | 0 | R | Enable the device to monitor Special Cycle commands. The 4D Wave does not support this feature. This bit is hard wired to a ‘0’. |
| [2] | 0 | R/W | Bus Master Enable 0 = Disables the Bus Master Operation 1 = Enables the Bus Master Operation |
| [1] | 0 | R/W | Memory Space Enable 0 = Disables the device to respond to Memory Space cycles 1 = Enables the device to respond to Memory Space cycles |
| [0] | 0 | R/W | I/O Space Enable 0 = Disables the device to respond to I/O Space cycles 1 = Enables the device to respond to I/O Space cycles |

#### Status (Offset = 06h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [15] | 0 | R/Write 1 to Clear | Parity Error Detected. This bit is set by the device, whenever it detects a parity error, even when the Command bit [6] is disabled. |
| [14] | 0 | R/Write 1 to Clear | SERR# status. Set whenever this device asserts SERR#. |
| [13] | 0 | R/Write 1 to Clear | Received Master-Abort status. Set whenever a transaction to this device is terminated with a Master-Abort. |
| [12] | 0 | R/Write 1 to Clear | Received Target-Abort status. Set whenever a transaction by this device is terminated with a Target-Abort. |
| [11] | 0 | R | Signalled Target-Abort status. Set whenever a transaction to this device is terminated with a Target-Abort. This will never happen on 4D Wave |
| [10:9] | 01b | R | DEVSEL Timing. The device supports “Medium” DEVSEL timing. |
| [8] | 0 | R/Write 1 to Clear | Data Parity Error Detected. This bit is set by the Master device when 1) it detects a data parity error, 2) it is the current Master, & 3) when the Command bit [6] is enabled to report parity errors. |
| [7] | 0 | R | Capable of Fast Back-to-Back cycles. The 4D Wave does not support this feature. This bit is hard wired to a ‘0’. |
| [6] | 0 | R | User Definable Features. The 4D Wave does not support this feature. This bit is hard wired to a ‘0’. |
| [5] | 0 | R | 66 Mhz Capable. The 4D Wave does not support this feature. This bit is hard wired to a ‘0’. |
| [4] | 1 | R | Capabilities List. The 4D Wave supports “New Capabilities” structure. |
| [3:0] | 00000b | R | Reserved |

#### Revision ID (Offset = 08h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:0] | 00h | R | 4D Wave DX Revision. First revision = 00h. |

#### Class Code (Offset = 10h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [23:16] | 04h | R | Base Class. Multimedia = 04h |
| [15:8] | 01h | R | Sub Class. Audio = 01h |
| [7:0] | 00h | R | Interface Class. None Defined = 00h |

#### Cache Line Size (Offset = 0Ch)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:0] | 00h | R | Cache Line Size. The 4D Wave does not support this feature. This bit is hard wired to a ‘0’. |

#### Latency Timer (Offset = 0Dh)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:3] | 00000b | R/W | Latency Timer in 8-PCI Clock increments. |
| [2:0] | 000b | R | Latency Timer LS 3-bits. Always 000b. This forces the latency to be incremented by 8-PCI Clocks at a time. |

#### Header Type (Offset = 0Eh)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7] | 0 | R | Multi-function device enable. The 4D Wave DX is not a multi-function device. |
| [6:0] | 0000000b | R | Header Layout. The 4D Wave DX uses a “standard” configuration layout. |

#### BIST (Offset = 0Fh)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:0] | 00h | R | The 4D Wave does not support this feature. These bits are hard wired to a ‘00h’. |

#### I/O Base Address (Offset = 10h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [31:8] | 000000h | R/W | I/O Base Address [31:8]. Specifies the MS 24-bits of the Audio I/O base address. |
| [7:2] | 000000b | R | I/O Base Address [7:2]. Forces alignment to a 256-byte block. |
| [1] | 0 | R | Reserved. |
| [0] | 1 | R | I/O Base identifier. |

#### Memory Base Address (Offset = 14h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [31:12] | 00000h | R/W | Memory Base Address [31:12]. Specifies the MS 20-bits of the Audio Memory base address. |
| [11:4] | 00h | R | Memory Base Address [11:4]. Forces alignment to a 4K-byte block. |
| [3] | 0 | R | Prefetchable. The 4D Wave uses this space for Memory mapped I/Os. This is not a prefetchable or mergeable address space. |
| [2:1] | 00b | R | Type. This can be located anywhere in 32-bit address space. |
| [0] | 0 | R | Memory Base identifier. |

#### Subsystem Vendor ID (Offset = 2Ch)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [15:0] | 1023h | R/(W) | Trident Microsystems’s PCI Vendor ID is used as the default. This register can be written if bit [1] in PCI Config [46h] is enabled. |

#### Subsystem ID (Offset = 2Eh)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [15:0] | 2000h | R | The 4D Wave does not support this feature. These bits are hard wired to a 2000h. |

#### Capabilities Pointer (Offset = 34h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:0] | 48h | R | Capabilities Pointer. Absolute offset to the start of the extended capabilities configuration space. |

#### Interrupt Line (Offset = 3Ch)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:0] | 00h | R/W | Interrupt Line. This is used by the driver and Plug-n-Play setup code to identify the interrupt used by this device. |

#### Interrupt Pin (Offset = 3Dh)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:0] | 01h | R | Interrupt Pin. This is used to tell what pin the device uses. The 4D Wave DX uses the INTA# pin. |

#### Minimum Grant (Offset = 3Eh)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:0] | 02h | R | Minimum Latency. The minimum time to complete a burst is 500 ns. (2 x .25us increment) This is assuming that the chipset typically has a 7 clock lead off and the burst is for 4 Dwords. |

#### Maximum Latency (Offset = 3Fh)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:0] | 05h | R | Maximum Latency. The maximum time between request cycles is set at 1.25us. A 48kHz sample period is 20.8 microseconds. This is assuming an average of 16 data requests per sample period. |

### DX - Legacy Configuration

#### Distributed DMA Configuration (Offset = 40h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [31:4] | 0000000h | R/W | DDMA Base Address [31:4] |
| [3] | 0 | R/W | Non Legacy Extended Addressing Control (Fully 32 bit Addressing) 0 = disabled 1 = enabled |
| [2:1] | 00b | R | Legacy DMA Transfer Size Control 00 8 bit transfer, legacy |
| [0] | 0 | R/W | DDMA Slave Channel Access Enable Control 0 = disabled 1 = enabled |

#### Legacy I/O Base (Offset = 44h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7] | 0 | R/W | MPU-401 Legacy Address Space Enable 0 = MPU401Base disable 1 = MPU401Base enable |
| [6] | 0 | R/W | MPU-401 Legacy Address Space Select 0 = MPU401Base 0330h-0333h 1 = MPU401Base 0300h-0303h |
| [5] | 0 | R/W | Game Port Legacy Address Space Enable 0 = GAMEBase disable 1 = GAMEBase enable |
| [4] | 0 | R/W | Game Port Legacy Address Space Select 0 = GAMEBase 0200h-0207h 1 = GAMEBase 0208h-020Fh |
| [3] | 0 | R/W | Adlib Legacy Address Space Enable 0 = ADLIBBase disable 1 = ADLIBBase enable |
| [2] | 0 | R/W | Adlib Legacy Address Space Select 0 = ADLIBBase 0388h-038Bh 1 = ADLIBBase 038Ch-038Fh |
| [1] | 0 | R/W | Sound Blaster Legacy Address Space Enable 0 = SBBase disable 1 = SBBase enable |
| [0] | 0 | R/W | Sound Blaster Legacy Address Space Select 0 = SBBase 0220h-022Fh 1 = SBBase 0240h-024Fh |

#### Legacy DMA (Offset = 45h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:4] | 0h | R | Reserved |
| [3] | 0 | R | Reserved |
| [2] | 0 | R/W | Enable SERR on DMA read accesses to 0/1 or 2/3 depending on DMA snooping channel select (bit[0]). 0 = SERR generation disabled on DMA IO reads 1 = SERR generation enabled on DMA IO reads |
| [1] | 0 | R/W | Enable Legacy DMA Snooping/Trapping 0 = DMA trapping disable 1 = DMA trapping enable |
| [0] | 0 | R/W | DMA Snooping Channel Select 0 = DMA channel 1 trapping 1 = DMA channel 0 trapping |

#### Legacy Control (Offset = 46h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:3] | 00000b | R/W | Reserved |
| [2] | 0 | R/W | Audio Engine Reset 0 = Normal 1 = Reset Audio Registers & Wave Engine State Machines When this bit is ‘1’, the audio block (including wavetable and legacy audio) will be reset. It must be set to ‘0’, to exit reset. |
| [1] | 0 | R/W | Sub-System Vendor ID Write Enable 0 = Sub-System Vendor ID is Read Only 1 = Sub-System Vendor ID is Read/Write |
| [0] | 0 | R | Reserved |

### DX - Power Management Configuration

#### Capabilities ID (Offset = 48h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:0] | 01h | R | Identifies the capability as being the PCI Power Management Registers. |

#### Next Item Pointer (Offset = 49h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:0] | 00h | R | By being 00h, indicates the end of the linked-list of extended capabilities. |

#### Power Management Capabilities (Offset = 4Ah)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [15:11] | 00000b | R | PME Support – The 4D Wave DX does not support PME generation. |
| [10] | 1 | R | D2 Support – The 4D Wave does support the D2 power state. |
| [9] | 1 | R | D1 Support – The 4D Wave does support the D1 power state. |
| [8:6] | 000b | R | Reserved |
| [5] | 0 | R | Device Specific Initialization – The 4D Wave does not require any device specific initialization. |
| [4] | 0 | R | Auxiliary Power Source - The 4D Wave DX does not support separate internal power support. (Or PME generation.) |
| [3] | 0 | R | PME Clock - The 4D Wave DX does not support PME generation and, therefore, does not need the PCI clock to generate a PME. |
| [2:0] | 001b | R | Version – The 4D Wave DX support Revision 1.0 of the PCI Power Management Interface specification. |

#### Power Management Control/Status (Offset = 4Ch)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [15] | 0 | R | PME Status – The 4D Wave DX does not support PME generation. |
| [14:13] | 00b | R | Data Scale – The 4D Wave DX does not support Power value reporting through the “Data” register. |
| [12:9] | 0000b | R | Data Select - – The 4D Wave DX does not support Power value reporting through the “Data” register. |
| [8] | 0 | R | PME Enable – The 4D Wave DX does not support PME generation. |
| [7:2] | 000000b | R | Reserved |
| [1:0] | 00b | R/W | Power State – This is used to determine the current power state. Software updates this register when changing power states 00b – D0 01b – D1 10b – D2 11b – D3hot |

### DX - Interrupt Snooping Configuration

#### Interrupt Snooping Control (Offset = 50h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [15:8] | 00h | R/W | Interrupt Vector: Compared to vector returned on AD[7:0] during a PCI interrupt acknowledge cycle. If it matches, then disable INTA# until all internal interrupts have been cleared. (The only interrupts enabled should be legacy SoundBlaster related.) |
| [7:1] | 00h | R | Reserved |
| [0] | 0 | R/W | Interrupt Snoop Enable 0 = Disable Interrupt Snooping 1 = Enable Interrupt Snooping (During Interrupt Acknowledge) |

## 4DWave-DX I/O or Memory Space

The 4D Wave can be configured to respond to the separate legacy register addresses, as well as, the combined Wave register space. The Wave register space can be accessed through traditional I/O cycle or memory mapped I/O cycles.

- 4D Wave Register Space

The Legacy registers are mapped into the bottom 64 bytes of the Wave register address space.

- Legacy Mapping into 4DWave-DX Register Space

The table below details the address map of the 4D Wave DX internal register set.  These Wave registers are addressable using either the Audio I/O Base Address or the Audio Memory Base Address. The legacy portion of these registers is available through their respective legacy addresses as well. By providing both an I/O and a memory aperture to these registers, the 4D Wave can be tuned for both compatibility and performance. Both I/O and Memory apertures can be enabled simultaneously.

The Wave registers consume a 256-byte aligned address space. The I/O mapping only allows the 256-byte space to be accessed. The memory mapping consumes 4K-bytes, however, only the bottom 256-bytes actually addresses the Wave registers.

All register contents reside on-chip, however, some registers must be accessed through a base indexed manner. This is true for voice channel specific registers and OPL3 RAM locations. All I/O locations which are not defined will return H'00000000 when read.  There is no affect when writing to non-defined registers.

The Wave registers indexed by 00h through DFh are global in function and are not specific to a particular voice channel.

The registers E0h through EFh are voice channel specific for each of the 64 voice channels (0-63) while the registers F0h through FFh are implemented only for channels the first 32 voice channels (0-31).

The OPL3 RAM is shared between Legacy FM Synthesis emulation and the second set of 32 voice channels (32-63).  During legacy emulation, the second 32 voice-channel specific registers (32-63) are not available for use as detailed below. Likewise, when in DirectX 64 voice channel mode, the OPL3 RAM (accessed through the Sound Blaster/Adlib registers) is not available for transfers.

- 4DWave DX Internal Registers

| Audio Base Offset (Hex) | Byte 3 | Byte 2 | Byte 1 | Byte 0 |
| --- | --- | --- | --- | --- |
| 00 | DMAR3 | DMAR2 | DMAR1 | DMAR0 |
| 04 | RSVD | DMAR6 | DMAR5 | DMAR4 |
| 08 | DMAR11 | DMAR10 | RSVD | DMAR8 |
| 0C | DMAR15 | DMAR14 | DMAR13 | DMAR12 |
| 10 | SBR3/SBR1 | SBR2 | SBR1/SBR3 | SBR0 |
| 14 | SBR6 | SBR6 | SBR5 | SBR4 |
| 18 | SBR7 | SBR7 | RSVD | RSVD |
| 1C | SBR10 | SBR9 | SBR8 | SBR8 |
| 20 | MPUR3 | MPUR2 | MPUR1 | MPUR0 |
| 24-2C | RSVD (Read Only = h'00000000) |  |  |  |
| 30 | RSVD | RSVD | GAMER1 | GAMER0 |
| 34 | GAMER2 |  |  |  |
| 38 | GAMER3 |  |  |  |
| 3C | RSVD (Read Only = h'00000000) |  |  |  |
| 40 | ACR0 |  |  |  |
| 44 | ACR1 |  |  |  |
| 48 | ACR2 |  |  |  |
| 4C | RSVD (Read Only = h'00000000) |  |  |  |
| 50 | ASR0 |  |  |  |
| 54 | RSVD | ASR2 | ASR1 |  |
| 58 | ASR3 |  |  |  |
| 5C | ASR6 | ASR5 | RSVD | ASR4 |
| 60 | AOPLSR0 |  |  |  |
| 64-6C | RSVD (Read Only = h’00000000) |  |  |  |
| 70 | RSVD | RCI2 | RCI1 | RCI0 |
| 74 | RSVD (Read Only = h'00000000) |  |  |  |
| 78 | PSBVLD_A Channels 0-31 |  |  |  |
| 7C | PSBVLD_B Channels 32-63 |  |  |  |
| 80 | START_A Channels 0-31 |  |  |  |
| 84 | STOP_A Channels 0-31 |  |  |  |
| 88 | DLY_A |  |  |  |
| 8C | SIGN_CSO_A |  |  |  |
| 90 | CSPF_A Channels 0-31 |  |  |  |
| 94 | CEBC_A |  |  |  |
| 98 | AIN_A Channels 0-31 |  |  |  |
| 9C | EINT_A |  |  |  |
| A0 | GC(12:8) +LFOCTRL_A | LFOCOUNT_A | GC(7:0) | CIR |
| A4 | AINTEN_A Channels 0-31 |  |  |  |
| A8 | MUSICVOL |  | WAVEVOL |  |
| AC | SBDELTA |  | DELTA_R |  |
| B0 | MISCINT |  |  |  |
| B4 | START_B Channels 32-63 |  |  |  |
| B8 | STOP_B Channels 32-63 |  |  |  |
| BC | CSPF _B Channels 32-63 |  |  |  |
| C0 | SBBL |  | SBCL |  |
| C4 | SBE2R | RSVD | SBDD | SBCTRL |
| C8 | RSVD | STIMER |  |  |
| CC | LFOCTRL_B | LFOCOUNT_B | ROM_TEST |  |
| D0 | T_FIFO [FIFO(39:24)] |  | T_FIFO [FIFO(19:4)] |  |
| D4 | T_DIGIMIXER [ADL(19:4)] |  | T_DIGIMIXER [ADR(19:4)] |  |
| D8 | AIN_B Channels 32-63 |  |  |  |
| DC | AINTEN_B Channels 32-63 |  |  |  |
| ARAM0 Channels 0-31 |  |  |  |  |
| E0 | CSO |  | ALPHA(11:4) | ALPHA(3:0) + FMS |
| E4 | PSBPTR[1:0] + LBA[29:0] |  |  |  |
| E8 | ESO |  | DELTA |  |
| EC | RSVD | RSVD | FMC + RVOL(6:1) | RVOL(0) + CVOL |
| ARAM1 Channel 32-63 |  |  |  |  |
| E0 | CSO |  | ALPHA(11:4) | ALPHA(3:0) + FMS |
| E4 | PSBPTR[1:0] + LBA[29:0] |  |  |  |
| E8 | ESO |  | DELTA |  |
| EC | RSVD | RSVD | FMC + RVOL(6:1) | RVOL(0) + CVOL |
| ERAM Channels 0-63 |  |  |  |  |
| F0 | GVSEL + PAN | VOL | CTRL + Ec(11:8) | Ec(7:0) |
| ERAM Channels 0-31 |  |  |  |  |
| F4 | EBUF1 |  |  |  |
| F8 | EBUF2 |  |  |  |
| FC | RSVD (Read Only = h'00000000) |  |  |  |

- NOTE: If SoundBlaster emulation is enabled, only the Bank A (0 – 31) channels can be used. If SoundBlaster emulation is NOT enabled, then all 64 channels in Bank A & B can be used.

## DX - Wave Register Definition

### DX - DMA

#### DMAR0 (Legacy DMA Playback Buffer Base Register Port1)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 00h; DDMABase + 0h, 0000h / 0002h | 8-bits | Read/Write | 00h |
| Bit | Description |  |  |
| 7:0 | Read: Legacy DMA Playback Buffer Current Address [7:0] Write: (Simultaneously writes both) Legacy DMA Playback Buffer Base Address [7:0] Legacy DMA Playback Buffer Current Address [7:0] |  |  |

#### DMAR1 (Legacy DMA Playback Buffer Base Register Port2)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 01h; DDMABase + 1h, 0000h / 0002h | 8-bits | Read/Write | 00h |
| Bit | Description |  |  |
| 7:0 | Read: Legacy DMA Playback Buffer Current Address [15:8] Write: (Simultaneously writes both) Legacy DMA Playback Buffer Base Address [15:8] Legacy DMA Playback Buffer Current Address [15:8] |  |  |

#### DMAR2 (Legacy DMA Playback Buffer Base Register Port3)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 02h; DDMABase + 2h, 0087h / 0083h | 8-bits | Read/Write | 00h |
| Bit | Description |  |  |
| 7:0 | Read: Legacy DMA Playback Buffer Current Address [23:16] Write: (Simultaneously writes both) Legacy DMA Playback Buffer Base Address [23:16] Legacy DMA Playback Buffer Current Address [23:16] |  |  |

#### DMAR3 (Legacy DMA Playback Buffer Base Register Port4 )

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 03h; DDMABase + 3h | 8-bits | Read/Write | 00h |
| Bit | Description |  |  |
| 7:0 | Read: Legacy DMA Playback Buffer Current Address [31:24] Write: (Simultaneously writes both) Legacy DMA Playback Buffer Base Address [31:24] Legacy DMA Playback Buffer Current Address [31:24] |  |  |

#### DMAR4 (Legacy DMA Playback Byte Count Register 1)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 04h; DDMABase + 4h, 0001h/0003h | 8-bits | Read/Write | 00h |
| Bit | Description |  |  |
| 7:0 | Read: Legacy DMA Playback Buffer Current Count [7:0] Write: (Simultaneously writes both) Legacy DMA Playback Buffer Base Count [7:0] Legacy DMA Playback Buffer Current Count [7:0] |  |  |

#### DMAR5 (Legacy DMA Playback Byte Count Register 2)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 05h; DDMABase + 5h, 0001h/0003h | 8-bits | Read/Write | 00h |
| Bit | Description |  |  |
| 7:0 | Read: Legacy DMA Playback Buffer Current Count [15:8] Write: (Simultaneously writes both) Legacy DMA Playback Buffer Base Count [15:8] Legacy DMA Playback Buffer Current Count [15:8] |  |  |

#### DMAR6 (Legacy DMA Playback Byte Count Register 3)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 06h; DDMABase + 6h | 8-bits | Read/Write | 00h |
| Bit | Description |  |  |
| 7:0 | Read: Legacy DMA Playback Buffer Current Count [23:16] Write: (Simultaneously writes both) Legacy DMA Playback Buffer Base Count [23:16] Legacy DMA Playback Buffer Current Count [23:16] |  |  |

#### DMAR8 Legacy DMA Controller Command / Status Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 08h; DDMABase + 8h, 0008h | 8-bits | Read | 00h |
| Bit | Description |  |  |
| 7:0 | Status register for implemented legacy 8237-A DMA channel |  |  |

#### DMAR10 Legacy DMA Single Channel Mask Port

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 0Ah; 000Ah | 0-bits | Write | 00h |
| Bit | Description |  |  |
| x | Channel mask register for implemented legacy 8237-A DMA channel |  |  |

#### DMAR11 Legacy DMA Channel Operation Mode Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 0Bh; DDMABase + Bh, 000Bh | 8-bits | Write | 00h |
| Bit | Description |  |  |
| 7:6 | Legacy DMA transfer request signal timing control 00 = demand mode select 01 = single mode select 10 = block mode select 11 = cascade mode select Not used and reserved for 4Dwave |  |  |
| 5 | Legacy DMA transfer address control 0 = address increment select 1 = address decrement select (not supported by 4DWave) |  |  |
| 4 | Legacy DMA transfer T/C mode control 0 = autoinitialization disable 1 = autoinitialization enable if (Bit 4 == 0) when DMATCReached goes from low to high set DMAR15.0 to 1 else if (Bit 4 == 1) when DMATCReached goes from low to high, set DMA current transfer address 31:0 = DMA base address 31:0 set DMA current count 23:0 = DMA base count 23:0 |  |  |
| 3:2 | Legacy DMA transfer I/O type control 00 = verify transfer (not supported by 4DWave) 01 = write transfer (recording by 4DWave) 10 = read transfer (playback by 4DWave) 11 = illegal & reserved |  |  |
| 1:0 | Channel Select: Not used/reserved |  |  |

#### DMAR12 Legacy DMA Controller First/Last Flag Clear Port

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 0Ch; 000Ch | 0-bits | Write | 00h |
| Bit | Description |  |  |
| x | Clear First/Last flag (First/Last = 0) for implemented legacy 8237-A DMA channel. |  |  |

#### DMAR13 Legacy DMA Controller Master Clear Port

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 0Dh; DDMABase + Dh, 000Dh | 0-bits | Write | 00h |
| Bit | Description |  |  |
| x | Master Clear for implemented legacy 8237-A DMA channel. |  |  |

#### DMAR14 Legacy DMA Controller Clear Mask Port

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 0Eh; 000Eh | 0-bits | Write | 00h |
| Bit | Description |  |  |
| x | Multi-channel mask clear for the implemented legacy 8237-A DMA channel |  |  |

#### DMAR15 Legacy DMA Controller Multi-Channel Mask Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 0Fh, DDMABase + 0Fh, 000Fh | 1-bits | Write | 0b |
| Bit | Description |  |  |
| x | Multi-channel mask register for implemented legacy 8237-A DMA channel |  |  |

### DX - Sound Blaster / Adlib

#### SBR0 (Legacy FmMusic Bank 0 Register Index / Legacy FmMusic Status)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 10h; SBBase + 0h; SBBase + 8h; AdlibBase + 0h | 8-bits | Read | 00h |
| Bit | Description |  |  |
| 7 | FM Timer Interrupt Flag (Equal to Bit 6 + Bit 5) |  |  |
| 6 | FM Timer 1 Overflowed Flag |  |  |
| 5 | FM Timer 2 Overflowed Flag |  |  |
| 4:0 | Reserved |  |  |
| I/O Port | Size | Access | Default |
| AudioBase + 10h; SBBase + 0h; SBBase + 8h; AdlibBase + 0h | 8-bits | Write | 00h |
| Bit | Description |  |  |
| 7:0 | OPL3 Bank0 Index |  |  |

#### SBR1/SBR3 (Legacy FmMusic Bank 0/1 Register Data Port)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 11h; AudioBase + 13h; SBBase + 1h; SBBase + 3h; SBBase + 9h; AdlibBase + 1h; AdlibBase +3h; | 8-bits | Read/Write | 00h |
| Bit | Description |  |  |
| 7:0 | OPL3 Data register |  |  |

#### SBR2 (Legacy FmMusic Bank 1 Register Index)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 12h; SBBase + 2h; AdlibBase + 2h | 8-bits | Read/Write | 00h |
| Bit | Description |  |  |
| 7:0 | OPL3 Bank1 Index |  |  |

#### SBR4 (Legacy Sound Blaster Mixer Register Index)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 14h; SBBase + 4h | 8-bits | Read/Write | 00h |
| Bit | Description |  |  |
| 7:0 | SoundBlaster 16/Pro Mixer Index Register |  |  |

#### SBR5 (Legacy Sound Blaster Mixer Register Data Port)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 15h; SBBase + 5h | 8-bits | Read/Write | xxh |
| Bit | Description |  |  |
| 7:0 | SoundBlaster 16/Pro Mixer Data register (Index by SBR4) |  |  |

#### SBR6 (Legacy Sound Blaster ESP Reset Port)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 16h, 17h; SBBase + 6h, 7h | 8-bits | Write Only (Read = FFh) | FFh |
| Bit | Description |  |  |
| 7:1 | Reserved (0’s) |  |  |
| 0 | 0 = Escape From SB16 / SBPRO ESP Reset State 1 = Enter Legacy SB16 / SBPRO ESP Reset State |  |  |

#### SBR7 (Legacy Sound Blaster ESP Data Port)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 1Ah, 1Bh; SBBase + Ah, Bh | 8-bits | Read | AAh |
| Bit | Description |  |  |
| 7:0 | Data returned by Legacy SB16 / SBPRO ESP Read Operation |  |  |

#### SBR8 (Legacy Sound Blaster Command / Status Port)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 1Ch, 1Dh; SBBase + Ch, Dh | 8-bits | Read/Write | 00h |
| Bit | Description |  |  |
| 7 | Write: SB Command or Data Bit [7] of Command (Operator) or Data (Operand ) Written to Legacy SB ESP Read: ESP Busy 0 = Legacy SB ESP is Available For Next Command / Data 1 = Legacy SB ESP is Busy. |  |  |
| 6:0 | Write: SB Command or Data Bit [6:0] of Command (Operator) or Data (Operand ) Written to Legacy SB ESP Read: Reserved Bit [6:0] = Reserved |  |  |

#### SBR9 (Legacy Sound Blaster ESP Data Ready / IRQ Acknowledge Port 1)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 1Eh; SBBase + Eh | 1-bits | Read | 2Ah |
| Bit | Description |  |  |
| 7 | Non-BX Command Data Acknowledge 0 = Data is not available on SBR7 (SBBase + Ah, Bh). 1 = Data is available on SBR7 (SBBase + Ah, Bh).. |  |  |
| 6:0 | Reserved |  |  |

#### SBR10 (Legacy Sound Blaster ESP Data Ready / IRQ Acknowledge Port 2 )

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 1Fh; SBBase + Fh | 1-bits | Read | 2Ah |
| Bit | Description |  |  |
| 7 | BX Command Data Acknowledge 0 = Data is not available on SBR7 (SBBase + Ah, Bh). 1 = Data is available on SBR7 (SBBase + Ah, Bh).. |  |  |
| 6:0 | Reserved |  |  |

### DX - MPU-401 UART

#### MPUR0 (Legacy MPU-401 Data Port / IRQ Acknowledge Port)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 20h; MIDIBase + 0h | 8-bits | Read/Write | (Reset as xxh) (MIDI-IN FIFO Data) |
| Bit | Description |  |  |
| 7:0 | Read: MPU-401 Acknowledge Byte or External MIDI Input Data in MIDI-IN FIFO; Write: MIDI Output Data |  |  |

#### MPUR1 (Legacy MPU-401 Command / Status Port)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 21h; MIDIBase + 1h | 8-bits | Read/Write | 80h |
| Bit | Description |  |  |
| 7 | Read: MPU-401 Status 0 = Ack. Byte is available or External MIDI Input Data is Available in MIDI-IN FIFO; 1 = NO Acknowledge Byte or External MIDI Input Data; Write: MPU-401 Command MIDI Command Data [7] |  |  |
| 6 | Read: MPU-401 Status 0 = Ready for MIDI Data Output or New MIDI Command 1 = MIDI-OUT FIFO is Full Write: MPU-401 Command MIDI Command Data [6] |  |  |
| 5 | Read: MPU-401 Status 0 = MIDI-IN FIFO is not Full 1 = MIDI-IN FIFO is Full Write: MPU-401 Command MIDI Command Data [5] |  |  |
| 4 | Read: MPU-401 Status 0 = MPU401 engine is at PASS-THRU mode 1 = MPU401 engine is at UART mode Write: MPU-401 Command MIDI Command Data [4] |  |  |
| 3:0 | Read: MPU-401 Status Reserved Write: MPU-401 Command MIDI Command Data [3:0] |  |  |

#### MPUR2 (MPU-401 Operation Control  / Status Register)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 22h; MIDIBase + 2h | 8-bits | [7:2] Read/Write [1:0] Read | 10h |
| Bit | Description |  |  |
| 7 | MIDI-IN FIFO Source – Internal Loopback 0 = MIDI-IN FIFO Source From External MIDI-IN Pad; 1 = MIDI-IN FIFO Source From MIDI-OUT FIFO (MIDI-OUT Pad is also disabled, always high); |  |  |
| 6 | MIDI-OUT Pad Source – External Loopback 0 = External MIDI-OUT Pad Source From MIDI-OUT FIFO; 1 = External MIDI-OUT Pad Source From External MIDI-In Pad; |  |  |
| 5 | MIDI Clock Control 0 = Regular MIDI Clock is being used ; 1 = Fast MIDI Clock (12.288MHz) is being used; |  |  |
| 4 | Pass-Thru Mode UART Enable 0 = MPUR0 Disconnect From MIDI-OUT FIFO; 1 = Connect MPUR0 to MIDI-OUT FIFO; |  |  |
| 3 | MIDI-IN Interrupt Enable 0 = Interrupt will be generated When MIDI-IN FIFO is not Empty; 1 = Interrupt will not be generated When MIDI-IN FIFO is not Empty; |  |  |
| 2 | MIDI-OUT Fifo Full Mask 0 = Normal MIDI-OUT Fifo full operation 1 = Mask MIDI-OUT Fifo full flag if MIDI-IN and MIDI-OUT Fifos are full and internal loopback is enabled |  |  |
| 1 | Read: MIDI-OUT FIFO status 0 = MIDI-OUT FIFO is Empty; 1 = MIDI-OUT FIFO is Not Empty; Write: Reserved |  |  |
| 0 | Read: MIDI-IN FIFO status 0 = MIDI-IN FIFO is Empty; 1 = MIDI-IN FIFO is not Empty; Write: Reserved |  |  |

#### MPUR3 (MPU-401 MIDI-IN FIFO Access Port)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| MIDIBase + 3h, AudioBase + 23h | 8-bits | Read | xxh |
| Bit | Description |  |  |
| 7:0 | MIDI Data Serialized In MIDI-IN FIFO |  |  |

### DX - Game Port

#### GAMER0 (Gameport Control Register)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 30h | 8-bits | Read/Write | 00h |
| Bit | Description |  |  |
| 7 | Enhanced Gameport Enable 0 = Disable Enhanced Digital Gameport; 1 = Enable Enhanced Digital Gameport; |  |  |
| 6 | Enhanced Gameport Test Mode 0 = Disable Testmode for Enhanced Digital Gameport; 1 = Enable Testmode for Enhanced Digital Gameport; |  |  |
| 5:0 | Reserved |  |  |

When Bit [6] is set, the game counter will overflow after every 1024 AC’97 BITCLKs. Bit [6] is only useful when Bit [7] is set.

Bit [7] will be automatically cleared if there are any I/O operation to GAMER1.

#### GAMER1 (Legacy Gameport I/O Register)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 31h; GameBase + 0h – 7h | 8-bits | Read/Write | F0h |
| Bit | Description |  |  |
| 7 | Write: Nothing Read: 0 = Joystick B Button1 Pressed (Input pad at low level) 1 = Joystick B Button1 Released (Input pad at high level) |  |  |
| 6 | Write: Nothing Read: 0 = Joystick B Button0 Pressed (Input pad at low level) 1 = Joystick B Button0 Released (Input pad at high level) |  |  |
| 5 | Write: Nothing Read: 0 = Joystick A Button1 Pressed (Input pad at low level) 1 = Joystick A Button1 Released (Input pad at high level) |  |  |
| 4 | Write: Nothing Read: 0 = Joystick A Button0 Pressed (Input pad at low level) 1 = Joystick A Button0 Released (Input pad at high level) |  |  |
| 3 | Write: Charge the R-C network Read: 0 = Joystick B Y-Axis input pad at low level 1 = Joystick B Y-Axis input pad at high level |  |  |
| 2 | Write: Charge the R-C network Read: 0 = Joystick B X-Axis input pad at low level 1 = Joystick B X-Axis input pad at high level |  |  |
| 1 | Write: Charge the R-C network Read: 0 = Joystick A Y-Axis input pad at low level 1 = Joystick A Y-Axis input pad at high level |  |  |
| 0 | Write: Charge the R-C network Read: 0 = Joystick A X-Axis input pad at low level 1 = Joystick A X-Axis input pad at high level |  |  |

#### GAMER2 (Enhanced Gameport Position Register 1)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 34h | 32-bits | Read/Write | FFFFFFFFh |
| Bit | Description |  |  |
| 31:16 | Joystick A Y-Axis Position Latched Value (16-bit unsigned) |  |  |
| 15:0 | Joystick A X-Axis Position Latched Value (16-bit unsigned) |  |  |

#### GAMER3 (Enhanced Gameport Position Register 2)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 38h | 32-bits | Read/Write | FFFFFFFFh |
| Bit | Description |  |  |
| 31:16 | Joystick B Y-Axis Position Latched Value (16-bit unsigned) |  |  |
| 15:0 | Joystick B X-Axis Position Latched Value (16-bit unsigned) |  |  |

### DX - AC’97 Registers

#### ACR0	AC’97 Codec Write Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 40h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:16 | Write Data for the AC’97 Codec |  |  |
| 15 | AC’97 Write Command/Status Bit Read: 0 = Write has completed; Ready to write the AC’97 Codec register 1 = Busy writing AC’97 Codec (indexed by bits [6:0]) Write: 0 = Do Nothing 1 = Write AC’97 Codec register (indexed by bits [6:0]) with bits [31:16] |  |  |
| 14:7 | Reserved |  |  |
| 6:0 | Index of the AC’97 Codec register to be written |  |  |

#### ACR1	AC’97 Codec Read Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 44h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:16 | Read: Read Data from the AC’97 Codec Write: Reserved |  |  |
| 15 | AC’97 Read Command/Status Bit Read: 0 = Ready to read the AC’97 Codec Register; Last Read cycle completed 1 = Busy reading AC’97 Codec (indexed by bits [6:0]) Write: 0 = Do Nothing 1 = Read AC’97 Codec register (indexed by bits [6:0]) with bits [31:16] |  |  |
| 14:7 | Reserved |  |  |
| 6:0 | Index of the AC’97 Codec register to be read |  |  |

#### ACR2	AC’97 Command/Status Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 48h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:7 | Reserved – Always write 0’s |  |  |
| 6 | SoundBlaster Control 0 = Generate Interrupt when recording block length expired 1 = Don’t generate Interrupt when recording block length expired |  |  |
| 5 | 0 = 48kHz signal will always be passed to Wave Engine 1 = 48kHz signal will only be passed with there is an active channel (out or in) |  |  |
| 4 | Read Only 0 = AC’97 is not ready for operation 1 = AC’97 is not ready for operation |  |  |
| 3 | Read Only 0 = Digital Audio Data from the AC’97 ADC is not valid (Record off) 1 = Digital Audio Data from the AC’97 ADC is valid (Record on) |  |  |
| 2 | Reserved – Always write 0’s |  |  |
| 1 | 0 = Digital Audio Data to the AC’97 DAC is not valid (Playback off) 1 = Digital Audio Data to the AC’97 DAC is valid (Playback on) |  |  |
| 0 | 0 = No “warm” reset 1 = Warm reset AC’97 Codec |  |  |

### DX - General

#### ASR0	4D Wave Status Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 50h | 32-bits | Read | 00000000h |
| Bit | Description |  |  |
| 31:28 | Reserved |  |  |
| 27 | SB ESP is at special DMA mode |  |  |
| 26:25 | 00 = SB ESP is at get operator state 01 = SB ESP is at get first operand state 11 = SB ESP is at get second operand state 10 = SB ESP is at get third operand state |  |  |
| 24 | SB Mixer Soft-Reset (If a ‘1’) |  |  |
| 23 | SB PRO Command Captured Most Recently (Non-Bx or Cx Type Command Captured) (If a ‘1’) |  |  |
| 22 | Sb16 Command Captured Most Recently (Bx or Cx Type Command Captured) (If a ‘1’) |  |  |
| 21 | SB Engine Sample Rate Set By Frequency Most Recently (If a ‘1’) |  |  |
| 20 | SB Engine Sample Rate Set By Time Constant Most Recently (If a ‘1’) |  |  |
| 19 | SB16 Mixer Register Update (If a ‘1’) |  |  |
| 18 | SB PRO Mixer Register Update (If a ‘1’) |  |  |
| 17 | OPL3 Bank1 Key On/Off (If a ‘1’) |  |  |
| 16 | OPL3 Bank0 Key On/Off (If a ‘1’) |  |  |
| 15 | 0 = AC-97 codec is not ready 1 = AC-97 codec is ready |  |  |
| 14 | 0 = SB Mixer Register MX0E.1 is 0 1 = SB Mixer Register MX0E.1 is 1 |  |  |
| 13 | 0 = SB ESP is not at Direct Recording Mode 1 = SB ESP is at Direct Recording Mode |  |  |
| 12 | 0 = SB ESP has no ack byte 1 = SB ESP has ack byte that needs to be read out |  |  |
| 11 | 0 = SB ESP DMA Command is not valid 1 = SB ESP DMA command is valid |  |  |
| 10 | 0 = SB ESP Engine at Digital Audio Off State 1 = SB ESP Engine at Digital Audio On State |  |  |
| 9:8 | 00 = SB ESP Engine Command Port Not Busy 01 = SB ESP Engine Command Port Busy 10 = SB ESP DMA Test Busy 11 = SB ESP Command Buffer Full |  |  |
| 7 | 0 = 8 bit data format 1 = 16 bit data format |  |  |
| 6 | 0 = mono 1 = stereo |  |  |
| 5 | 0 = unsigned data format 1 = signed data format |  |  |
| 4 | 0 = playback 1 = recording |  |  |
| 3 | 0 = SB DMA loop disable 1 = SB DMA loop enable |  |  |
| 2:0 | Legacy Command 000 = stop: No operation. No contribution to Digital Mixer 001 = run: Normal operation. 010 = silent_DMA: SBCL will count; CA, CBC won’t count. No data fetching. No interpolation. No contribution to Digital Mixer 011 = reserved 100 = silent_SB: SBCL, CA & CBC will count as the same as run mode.No data fetching. No interpolation. No contribution to Digital Mixer 101 = pause: SBCL, CA & CBC don’t change; let SBALPHA unchanged, CACHE_HIT=1; drive current LD (or LD_L, LD_R) to Digital Mixer 110 = reserved 111 = direct play: SBCL, CA & CBC don’t change. drive SBDD to Digital Mixer |  |  |

Bits [24:16] will be cleared to a ‘0’ after this register is read.

Only one bit of Bit 21 and Bit 20 can be set 1 by implemented SB ESP Engine at any time.

Only one bit of Bit 23 and Bit 22 can be set 1 by implemented SB ESP Engine at any time.

#### ASR1  Legacy Sound Blaster Frequency Read Back Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 54h | 16-bits | Read | AC44h |
| Bit | Description |  |  |
| 15:0 | Sample Frequency Set by SB Command 41h or 42h |  |  |

#### ASR2 Legacy Sound Blaster Time Constant Read Back Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 56h | 8-bits | Read | F5h |
| Bit | Description |  |  |
| 7:0 | Time Constant Value Set by SB Command 40h |  |  |

#### ASR3 4D Wave Scratch Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 58h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:0 | General Register: Used for SW Driver to save flags |  |  |

#### ASR4 4D Wave Version Control Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 5Ch | 8-bits | Read | 01h |
| Bit | Description |  |  |
| 7:0 | 4DWave Revision: Changed at each tape out of the chip |  |  |

#### ASR5 SB ESP Version High Byte Control Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 5Eh | 8-bits | Read/Write | 04h |
| Bit | Description |  |  |
| 7:4 | Reserved (0’s) |  |  |
| 3:0 | SoundBlaster Revision – High Byte |  |  |

#### ASR6 SB ESP Version Low Byte Control Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 5Fh | 8-bits | Read/Write | 02h |
| Bit | Description |  |  |
| 7:4 | Reserved (0’s) |  |  |
| 3:0 | SoundBlaster Revision – Low Byte |  |  |

### OPL3 Channel Status Register

#### AOPLSR0 OPL3 Emulation Channel Keyon/off Trace Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 60h | 32-bits | Read | 00h |
| Bit | Description |  |  |
| 31:25 | Reserved |  |  |
| 24:16 | Bank1 channel 8-0 key on/off event captured (B0h-B8h) |  |  |
| 15:14 | Reserved |  |  |
| 13:9 | Bank0 rhythm channel 4-0 key on/off event captured (BDh) |  |  |
| 8:0 | Bank0 channel 8-0 key on/off event captured (B0h-B8h) |  |  |

### DX - Channel Registers

There are two banks of 32 channels: A & B. The 32 voice channels in Bank A are optimized for MIDI. The 32 voice channels in Bank B are optimized for DirectX Buffers, WDM Streaming Audio, and WAV files. Both Banks support most of the same operations, the chief difference is that Bank A includes the Envelope Slope control that is needed for MIDI. Bank B does not include any Amplitude Slope control. Bank A is available at all times. Bank B is only available when the SoundBlaster register space is not enabled for DOS compatibility, which is the typical case while in Windows.

- Bank A = Channels 0 – 31

- Bank B = Channels 32 – 63

#### CI0, RCI1, RCI2 Record Channel Index 0 (Mixer), 1 (Reverb), & 2 (Chorus)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 70h | 32-bits | Read | 00000000h |
| Bit | Description |  |  |
| 31:24 | Reserved |  |  |
| 23 | RCI2 Enable 0 = Disabled; Record channel inactive 1 = Enabled; The channel select by |  |  |
| 22 | Reserved |  |  |
| 21:16 | RCI2 Index – These 6-bits select one of the 64 channels & it’s respective stream buffer as the record channel for this stream. |  |  |
| 15 | RCI1 Enable 0 = Disabled; Record channel inactive 1 = Enabled; The channel select by |  |  |
| 14 | Reserved |  |  |
| 13:8 | RCI1 Index – These 6-bits select one of the 64 channels & it’s respective stream buffer as the record channel for this stream. |  |  |
| 7 | RCI0 Enable 0 = Disabled; Record channel inactive 1 = Enabled; The channel select by |  |  |
| 6 | Reserved |  |  |
| 5:0 | RCI0 Index – These 6-bits select one of the 64 channels & it’s respective stream buffer as the record channel for this stream. |  |  |

There are three record channel index registers for Mixer, Reverb & Chorus. These registers specify what “channel”, if enabled, is selected as a record channel. (Note. The respective channel’s START bit must also be set.)

#### PSBVLD_A Bank A PCI Stream Buffer Valid Flags

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 78h | 32-bits | Read/Write (For testing only) | 00000000h |
| Bit | Description |  |  |
| 31:0 | PCI Stream Buffer (PSB) Valid Status; Bit [n] = Channel [n] 0 = PSB Channel Data Invalid 1 = PSB Channel Data Valid |  |  |

#### PSBVLD_B Bank A PCI Stream Buffer Valid Flags

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 7Ch | 32-bits | Read/Write (For testing only) | 00000000h |
| Bit | Description |  |  |
| 31:0 | PCI Stream Buffer (PSB) Valid Status; Bit [n] = Channel [n] 0 = PSB Channel Data Invalid 1 = PSB Channel Data Valid |  |  |

#### START_A Bank A Start Command and Status Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 80h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:0 | Bit [n] = Channel [n] Read: Channel Status 0 = Stopped 1 = Running Write: Channel Start 0 = Do Nothing; Doesn’t effect start / running operation 1 = Start Channel (Also sets the Channel status bit) |  |  |

This register and the STOP_A register are used as Bank A channel start/stop command registers when they are written, and used as channel running/stopped status register when they are read. Bit [n] is for channel [n].

Stopped

When bit [n] is read as ‘0’, it means that any operation of channel [n], including address generation, sample data fetching, interpolation, and envelope calculation is stopped. And this channel has no contribution to the digital mixer.

This bit will be reset from ‘1’ to ‘0’ in four cases.

1) When a ‘1’ is written to the corresponding bit in register STOP.

2) When out of data, i.e. when sample loop disabled and CSO (Current Sample Offset) >= ESO (End Sample Offset).

3) When Ec (current envelope) drops down to -63.984375 dB.

4) When current envelope buffer is in delay-stop mode, and EDLY count down to ‘0’.

Running

When bit [n] is read as ‘1’, it means channel [n] is working.

This bit will be set from ‘0’ to ‘1’ only when a ‘1’ is written to the corresponding bit in register START_A.

#### STOP_A Bank A STOP Command and Status register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 84h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:0 | Bit [n] = Channel [n] Read: Channel Status (Same as START_A) 0 = Stopped 1 = Running Write: Channel Stop 0 = Do Nothing; Doesn’t effect start / running operation 1 = Stop Channel (Also clears the Channel status bit) |  |  |

#### DLY_A Delay flag register (Only used with Bank A channels)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 88h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:0 | Bit [n] = Channel [n] Read: Delay Status 0 = Normal 1 = Channel is currently in delay mode This bit will toggle from ‘0’ to ‘1’ only when the envelope engine begins to deal with a delay mode buffer. Write: 0 = Do Nothing; Doesn’t change 1 = Set Channel to Delay Mode |  |  |

#### SIGN_CSO_A Sign bit of CSO  (Only used with Bank A channels)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 8Ch | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:0 | Channel Sign Bit for CSO; Bit [n] = Channel [n] Read: 0 = Current Sample Offset (CSO) is greater than or equal to the Loop Begin Address (LBA) 1 = Current Sample Offset (CSO) is less than the Loop Begin Address (LBA) Write: 0 = Do Nothing; Doesn’t change 1 = Set Channel CSO bit to ‘1’ When channel [n] is stopped (STOP_A), bit [n] will be reset to ‘0’. |  |  |

#### CSPF_A Bank A Current Sample Position Flag

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 90h | 32-bits | Read | 00000000h |
| Bit | Description |  |  |
| 31:0 | Channel Current Sample Position; Bit [n] = Channel [n] 0 = Current Sample Offset (CSO) is less than the End Sample Offset/2 (ESO/2) {CSO < ESO/2} 1 = Current Sample Offset (CSO) is greater than or equal to End Sample Offset/2 (ESO/2) {ESO/2 <= CSO} When channel [n] is stopped (STOP_A), bit [n] will be reset to ‘0’. |  |  |

#### CEBC_A Current Envelope Buffer Control (Only used with Bank A channels)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 94h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:0 | Current Envelope Buffer; Bit [n] = Channel [n] Read: 0 = Buffer 1 (EBUF1) 1 = Buffer 2 (EBUF2) Write: 0 = Do Nothing; Doesn’t change 1 = Toggle to other Buffer |  |  |

#### AIN_A Bank A Address Engine Interrupt Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 98h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:0 | Address Engine Interrupts; Bit [n] = Channel [n]; Any bit [n] that changes from a ‘0’ to a ‘1’ (rising edge) with result in and IRQ, if enabled. Read: Address Engine Channel Interrupt Status 0 = No Interrupt 1 = Interrupt This bit will be set in 2 cases: When CSO (current sample offset) >= ESO (end sample offset), and ENDLP_IE (end of loop INT enable bit in Global Control register) =1 and AINTEN_A bit n is set 1 for channel n. When CSO (current sample offset) >= ESO/2 (middle of ESO), and MIDLP_IE (middle of loop INT enable bit in Global Control register) =1 and AINTEN_A bit n is set 1 for channel n. Write: (Write ‘1’ to clear) 0 = Do Nothing; Doesn’t change 1 = Reset bit to ‘0’ |  |  |

#### EINT_A Envelope Engine Interrupt Register (Only used with Bank A channels)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 9Ch | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:0 | Envelope Engine Interrupts; Bit [n] = Channel [n]; Any bit [n] that changes from a ‘0’ to a ‘1’ (rising edge) with result in and IRQ, if enabled. Read: Address Engine Channel Interrupt Status 0 = No Interrupt 1 = Interrupt This bit will be set in 2 cases: When envelope buffer toggled, and ETOG_IE (envelope toggle INT enable bit in Global Control register) =1. When Ec (current envelope) <= FFFh (-63.984375 dB), and EDROP_IE (envelope dropping to -63.984375dB INT enable bit in Global Control register) =1. Write: (Write ‘1’ to clear) 0 = Do Nothing; Doesn’t change 1 = Reset bit to ‘0’ |  |  |

#### LFO_A & GC & CIR: Bank A Low Frequency Oscillator & Global Control & Channel Index

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + A0h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:30 | Operation Control when recording to mono sample. 00 = left 01 = right 10 = (left+right+1)/2 11= reserved. |  |  |
| 29:28 | DMA Status Reg (I/O 0008h) read handling control bits. 00 = Never assert StatusRDY 01 = StatusRDY = DMATCReached 10 = StatusRDY = DMATCReached ¦ LegacyDRQ 11 = In this case, handshaking with StatusWR and manipulation of return byte should been done. StatusRDY keep ‘0’ when initialization. If(StatusWR ==1) { StatusRDY = 1; if(DMAChannel==0) { ReturnByte[7:0] = {InputByte[7:5], DMAR8[4], InputByte[3:1], DMAR8[0]}; } else { ReturnByte[7:0] = {InputByte[7:6], DMAR8[5], InputByte[4], InputByte[3:2], DMAR8[1], InputByte[0]}; } } if(DMASNOOPCS_==0 & AD[7:0] = 8 & Data_rdy_ == 0 & StatusRDY==1) StatusRDY = 0; |  |  |
| 27 | Test Loopback: This bit is used for wave engine loopback testing. 0 = Normal 1 = Force recording engine get new data from playback FIFO instead of AC’Link. |  |  |
| 26 | Bank A LFO enable bit. 0 = disabled 1 = enabled |  |  |
| 25:24 | Clock rate select of Bank A LFO counter. 00 = LFO counter clock rate is 48kHz 01 = LFO counter clock rate is 48kHz/4 10 = LFO counter clock rate is 48kHz/16 11 = LFO counter clock rate is 48kHz/64 |  |  |
| 23:16 | Initial value of the Bank A LFO counter which will count down to 0 then reload |  |  |
| 15 | EDROP_IE is the INT enable bit for current envelope dropping to -63.984375dB 0 = disable 1 = enable |  |  |
| 14 | ETOG_IE is the INT enable bit for envelope buffer toggling. (Only Bank A) 0 = disable 1 = enable |  |  |
| 13 | MIDLP_IE is the INT enable bit for middle of loop. 0 = disable 1 = enable |  |  |
| 12 | ENDLP_IE is the INT enable bit for end of loop. 0 = disable 1 = enable |  |  |
| 11 | UNDERUN_IE is the INT enable bit for playback underrun 0 = disable 1 = enable When playback FIFO is empty, if this bit is set as ‘1’, an IRQ will be issued. |  |  |
| 10 | OVERUN_IE is the INT enable bit for recording overrun. 0 = disable 1 = enable When recording FIFO is full, if this bit is set as ‘1’, an IRQ will be issued. |  |  |
| 9 | PAUSE is pause command/status bit. Reading from this bit will return the paused/normal status. 0 = Normal 1 = Paused This bit will toggle from ‘1’ to ‘0’ when a ‘0’ is written in; and will toggle from ‘0’ to ‘1’ when a ‘1’ is written in and current bus master cycle has finished. Writing to this bit is the pause/resume command. 0 = Resume 1 = Pause Address engine and envelope engine will be paused after current bus master cycle complete. |  |  |
| 8 | RST_Stimer is used to reset playback sample timer counter. Read : Always return 0; Write: 0 = Do Nothing 1 = Reset Stimer |  |  |
| 7 | PSB RAM Test Enable 0 = Normal – test mode disabled; E0-ECh address their defined space 1 = Test Mode; E0-ECh address the 4 Dwords/Channel of PCI Stream Buffering; CIR is still used as the channel index |  |  |
| 6 | PSB Valid Test Enable 0 = Normal – test mode disabled; Channel Buffer Valid Flags are unaffected 1 = Test Mode; All channel valid flags are always seen as invalid. They are never seen as valid. This forces all bus cycles onto the PCI Bus for debug purposes. |  |  |
| 5:0 | Channel Index: This is used to select a channel for access. For Bank A, 00h selects channel 0 and 1Fh selects channel 31. For Bank B, 20h selects channel 32 and 3F selects channel 63. |  |  |

#### AINTEN_A Bank A Address Engine Interrupt Enable Control

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + A4h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:0 | Address Engine Channel Interrupt Enable; Bit [n] = Channel [n] 0 = Disable address engine interrupt for channel [n] 1 = Enable address engine interrupt for channel [n] |  |  |

#### MUSICVOL & WAVEVOL: Global Music Volume Control & Global Wave Volume Control

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + A8h | 32-bits | Read/Write | 00008080h |
| Bit | Description |  |  |
| 31:24 | Music Right Volume (format of 6.2) 0 0dB (no attenuation: default) 80h -32dB FFh -64dB |  |  |
| 23:16 | Music Left Volume (format of 6.2) 0 0dB (no attenuation: default) 80h -32dB FFh -64dB |  |  |
| 15:8 | Wave Right Volume (format of 6.2) 0 0dB(no attenuation) 80h -32dB (default) FFh -64dB |  |  |
| 7:0 | Wave Left Volume (format of 6.2) 0 0dB(no attenuation) 80h -32dB (default) FFh -64dB |  |  |

#### SBDELTA & DELTA_R: Sample Change Step for Legacy Sound Blaster Voice In/Out & Sample change step for recording

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + ACh | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:16 | Reserved |  |  |
| 15:0 | Delta Change Step for Legacy Playback/Record (format of 4.12) |  |  |

#### MISCINT Recording/Playback Underrun/Recording Overrun Interrupt Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + B0h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:18 | Reserved (0’s) |  |  |
| 17 | OPLTIMER_IE is the OPL3 timer interrupt enable bit. 0 = disable 1 = enable |  |  |
| 16 | PB_24K_MODE is playback 48k/24k mode control bit. 0 = Wave engine drives sample to CODEC at 48Khz (Default) 1 = Wave engine drives sample to CODEC at 24Khz (In this mode, Delta should be programmed twice as that in 48Khz mode). |  |  |
| 15:12 | Reserved |  |  |
| 11 | Mixer_overflow_flag is a flag which indicates the result of mixer accumulator exceeds 7FFFFh. This bit will be set to ‘1’ once accumulator overflows. Write ‘1’ will clear this bit. |  |  |
| 10 | Mixer_underflow_flag is a flag which indicates that the result of mixer accumulator is less than 80000h. This bit will be set to ‘1’ once accumulator underflows. Write ‘1’ will clear this bit. |  |  |
| 9 | REC_OVERUN is recording overrun status bit. Active high. This bit will be set to ‘1’ if the recording is running & rec_req_ is active & data_rdy is not active. |  |  |
| 8 | PB_UNDERUN is the playback FIFO underrun status bit. Active high. This bit will be set to ‘1’ if playback is running & the FIFO is empty & the 48kHz clock is coming. |  |  |
| 6 | ENVELOPE_IRQ is the Wave-table Envelope Engine IRQ bit. Active high. Read Only. Bit[6] = ¦ EINT[31:0] |  |  |
| 5 | ADDRESS_IRQ is the Wave-table Address Engine IRQ bit. Active high. Read Only. Bit[5] = (¦ AINT_A[31:0] ) ¦ (¦AINT_B[31:0]) |  |  |
| 4 | OPL3_IRQ is the legacy OPL3 IRQ bit. Active high. Read Only. Bit[4] = timerirq (signal from Legacy Audio block) |  |  |
| 3 | MPU401_IRQ is the MPU401 IRQ bit. Active high. Read Only. Bit[3] = mpu401irq (signal from Legacy Audio block) |  |  |
| 2 | SB_IRQ is the SoundBlaster IRQ bit. Active high. Read Only. Bit[2] = sbirq (signal from Legacy Audio block) |  |  |
| 1 | REC_OVERUN_IRQ) is recording overrun IRQ bit. Active high. Read Only. Bit[1] = OVERUN_IE & Bit[9] |  |  |
| 0 | PB_UNDERUN_IRQ is playback FIFO underrun IRQ bit. Active high. Read Only. Bit[0] = UNDERUN_IE & Bit[8] |  |  |

Bits [6:0] (read only) are the interrupt request bits. All of these six bits form signal AUDIO_INT.

AUDIO_INT = 	PB_UNDERUN_IRQ | REC_OVERUN_IRQ | SB_IRQ | MPU401_IRQ |

OPL3_IRQ | ADDRESS_IRQ | ENVELOPE_IRQ

#### START_B Bank B Start Command and Status Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + B4h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:0 | Bit [n] = Channel [n] Read: Channel Status 0 = Stopped 1 = Running Write: Channel Start 0 = Do Nothing; Doesn’t effect start / running operation 1 = Start Channel (Also sets the Channel status bit) |  |  |

This register and the STOP_B register are used as Bank B channel start/stop command registers when they are written, and used as channel running/stopped status register when they are read. Bit [n] is for channel [n].

Stopped

When bit [n] is read as ‘0’, it means that any operation of channel [n], including address generation, sample data fetching, interpolation, and envelope calculation is stopped. And this channel has no contribution to the digital mixer.

This bit will be reset from ‘1’ to ‘0’ in four cases.

1) When a ‘1’ is written to the corresponding bit in register STOP.

2) When out of data, i.e. when sample loop disabled and CSO (Current Sample Offset) >= ESO (End Sample Offset).

3) When Ec (current envelope) drops down to -63.984375 dB.

4) When current envelope buffer is in delay-stop mode, and EDLY count down to ‘0’.

Running

When bit [n] is read as ‘1’, it means channel [n] is working.

This bit will be set from ‘0’ to ‘1’ only when a ‘1’ is written to the corresponding bit in register START_B.

#### STOP_B Bank B STOP Command and Status register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + B8h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:0 | Bit [n] = Channel [n] Read: Channel Status (Same as START_B) 0 = Stopped 1 = Running Write: Channel Stop 0 = Do Nothing; Doesn’t effect start / running operation 1 = Stop Channel (Also clears the Channel status bit) |  |  |

#### CSPF_B Bank B Current Sample Position Flag

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + BCh | 32-bits | Read | 00000000h |
| Bit | Description |  |  |
| 31:0 | Channel Current Sample Position; Bit [n] = Channel [n] 0 = Current Sample Offset (CSO) is less than the End Sample Offset/2 (ESO/2) {CSO < ESO/2} 1 = Current Sample Offset (CSO) is greater than or equal to End Sample Offset/2 (ESO/2) {ESO/2 <= CSO} When channel [n] is stopped (STOP_B), bit [n] will be reset to ‘0’. |  |  |

#### SBBL & SBCL: SoundBlaster DMA Base Length & SoundBlaster DMA Current Length

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + C0h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:16 | SBBL: SoundBlaster DMA block length |  |  |
| 15:0 | SBCL: Current value of the SoundBlaster DMA block length counter |  |  |

If SoundBlaster DMA loop is enabled (SBCTRL[3]=1), every time when SBCL changed from 0 to FFFFh, a INT will be issued, the contents of SBCL is reloaded from SBBL, and DMA operation continues.

If sound blaster DMA loop is not enabled (SBCTRL[3]=0), every time when SBCL changed from 0 to FFFFh, a INT will be issued, the contents of SBCL is reloaded from SBBL, and set LegacyCMD to 101(pause).

SBCTRL bit 7 is used to determine the counter operation mode (byte count or word count). The counter is a count down counter.

#### SBCTRL & SBE2R & SBDD: SoundBlaster Control & SoundBlaster DMA Testing Byte Data & SoundBlaster Direct Playback Data

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + C4h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:24 | SBE2R is the SoundBlaster DMA testing byte command data port (write only). Any time after bits [31:24] have been written, E2Status (source from wave engine) will be set high. E2Status will be cleared after the testing byte has been sent to the system location. |  |  |
| 23:16 | Reserved |  |  |
| 15:8 | SBDD is the SoundBlaster direct mode playback data port. |  |  |
| 7 | 0 = 8 bit data format 1 = 16 bit data format |  |  |
| 6 | 0 = mono 1 = stereo |  |  |
| 5 | 0 = unsigned data format 1 = signed data format |  |  |
| 4 | 0 = playback 1 = recording |  |  |
| 3 | SoundBlaster DMA loop enable control 0 = loop disabled. 1 = loop enabled |  |  |
| 2:0 | LegacyCMD 000 = stop: No any operation. No contribution to Digital Mixer 001 = run: Normal operation. 010 = silent_DMA: SBCL will count; CA, CBC won’t count. No data fetching. No interpolation. No contribution to Digital Mixer 011 = reserved 100 = silent_SB: SBCL, CA & CBC will count as the same as run mode. No data fetching. No interpolation. No contribution to Digital Mixer 101 = pause: SBCL, CA & CBC don’t change. Let SBALPHA unchanged, CACHE_HIT=1; drive current LD (or LD_L, LD_R) to Digital Mixer 110 = reserved 111 = Direct_playback: SBCL, CA & CBC don’t change. Drive SBDD to Digital Mixer |  |  |

#### STIMER Sample Timer

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + C8h | 32-bits | Read | 00000000h |
| Bit | Description |  |  |
| 31:24 | Reserved |  |  |
| 23:0 | STIMER will show current state of the sample timer counter. The STIMER will count up every 48kHz clock and will be reset when RST_Stimer bit is being written. |  |  |

#### ROM Test

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + CCh | 16-bits | Read | xxxxh |
| Bit | Description |  |  |
| 15:12 | Reserved (0’s) |  |  |
| 11:0 | Internal ROM Data [11:0] |  |  |

#### LFO_B Bank B Low Frequency Oscillator

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + CEh | 16-bits | Read/Write | 0000h |
| Bit | Description |  |  |
| 15:11 | Reserved (0’s) |  |  |
| 10 | Bank B LFO enable bit. 0 = disabled 1 = enabled |  |  |
| 9:8 | Clock rate select of Bank B LFO counter. 00 = LFO counter clock rate is 48kHz 01 = LFO counter clock rate is 48kHz/4 10 = LFO counter clock rate is 48kHz/16 11 = LFO counter clock rate is 48kHz/64 |  |  |
| 7:0 | Initial value of the Bank B LFO counter which will count down to 0 then reload |  |  |

#### Test Mixer FIFO

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + D0h | 32-bits | Read | xxxxxxxxh |
| Bit | Description |  |  |
| 31:16 | Mixer FIFO Audio Left Sample [19:4] |  |  |
| 15:0 | Mixer FIFO Audio Right Sample [19:4] |  |  |

#### Test Mixer Accumulator

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + D4h | 32-bits | Read | xxxxxxxxh |
| Bit | Description |  |  |
| 31:16 | Mixer Accumulator Audio Left Sample [19:4] |  |  |
| 15:0 | Mixer Accumulator Audio Right Sample [19:4] |  |  |

#### AIN_B Bank B Address Engine Interrupt Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + D8h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:0 | Address Engine Interrupts; Bit [n] = Channel [n]; Any bit [n] that changes from a ‘0’ to a ‘1’ (rising edge) with result in and IRQ, if enabled. Read: Address Engine Channel Interrupt Status 0 = No Interrupt 1 = Interrupt This bit will be set in 2 cases: When CSO (current sample offset) >= ESO (end sample offset), and ENDLP_IE (end of loop INT enable bit in Global Control register) =1 and AINTEN_B bit n is set 1 for channel n. When CSO (current sample offset) >= ESO/2 (middle of ESO), and MIDLP_IE (middle of loop INT enable bit in Global Control register) =1 and AINTEN_B bit n is set 1 for channel n. Write: (Write ‘1’ to clear) 0 = Do Nothing; Doesn’t change 1 = Reset bit to ‘0’ |  |  |

#### AINTEN_B

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + DCh | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:0 | Address Engine Channel Interrupt Enable; Bit [n] = Channel [n] 0 = Disable address engine interrupt for channel [n] 1 = Enable address engine interrupt for channel [n] |  |  |

### DX - Indexed Channel Registers

There are 64 independent voice channels. The first 32 voice channels are designated Bank A. The second 32 voice channels are designated Bank B. Every voice channel has it’s own parameter register set. Bank A and Bank B parameters differ. These 64 register sets will share the same I/O space. The global register CIR is used to select the current accessible channel.

#### CSO & ALPHA & FMS: Current Sample Offset & Sample Interpolation Coefficient & Frequency Modulation Step (Bank A & B)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + E0h | 32-bits | Read/Write | xxxxxxxxh |
| Bit | Description |  |  |
| 31:16 | CSO is the offset of current sample relative to loop begin sample |  |  |
| 16:4 | ALPHA is sample interpolation coefficient, which stands for the linear interpolation ratio between current sample and the next one. |  |  |
| 3:0 | FMS is Frequency Modulation Step. |  |  |

#### PPTR & LBA: PSB Pointer & Loop Begin Address (Bank A & B)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + E4h | 32-bits | Read/Write | xxxxxxxxh |
| Bit | Description |  |  |
| 31:30 | PCI Stream Buffer address pointer; This is used by the hardware to store the current pointer location into each channels 4-Dword stream buffer. It can be ignored by software. |  |  |
| 29:0 | Linear address of the loop begin sample. Bit 0 will always be zero in 16-bit sample data mode. The MSB 2-bits [31:30] of the 32-bit LBA address are always 00b. This limits the addressable range to the lower 1GB of physical address space. |  |  |

#### ESO & DELTA: End Sample Offset &  Delta Sample Rate Ratio (Bank A & B)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + E8h | 32-bits | Read/Write | xxxxxxxxh |
| Bit | Description |  |  |
| 31:16 | ESO is the offset of loop end sample relative to loop begin sample. |  |  |
| 15:0 | DELTA is sample change step in format 4.12 (four bits integer, 12 bits fraction), which stands for the frequency ratio: Fs/48KHz, while Fs is the sum of sample rate and pitch shifting rate. |  |  |

#### FMC & RVOL & CVOL: FM Control, Reverb Volume & Chorus Volume (Bank A & B)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + ECh | 32-bits | Read/Write | xxxxxxxxh |
| Bit | Description |  |  |
| 31:16 | Reserved. |  |  |
| 15:14 | Reserved – Always write “11”. (Used to be FMC) |  |  |
| 13:7 | RVOL -- Reverb send volume format: 5.2, (0- -32dB) 7Fh as mute |  |  |
| 6:0 | CVOL -- Chorus send volume format: 5.2, (0- -32dB) 7Fh as mute |  |  |

#### GVSEL & PAN & VOL & CTRL & Ec: Global Volume Select & Positioning Attenuation & Volume Attenuation & Control & Current Envelope (Bank A & B)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + F0h | 32-bits | Read/Write | xxxxxxxxh |
| Bit | Description |  |  |
| 31 | GVSEL is global volume select bit. 0 = select MUSICVOL 1 = select WAVEVOL |  |  |
| 30 | PAN Control - Positioning attenuation control selects attenuated channel. 0 = left 1 = right |  |  |
| 29:24 | PAN Volume - Positioning attenuation value in format of 4.2. Special value is 3Fh, which means mute. |  |  |
| 23:16 | VOL is channel volume attenuation in format of 5.3. 00h stands for 0 dB attenuation, FEh stands for -31.875 dB attenuation. FFh stands for Mute. |  |  |
| 15 | Selects 8/16 bit sample data 0 = 8-bit data 1 = 16-bit data |  |  |
| 14 | Selects mono/stereo sample data 0 = mono 1 = stereo |  |  |
| 13 | Selects unsigned/signed sample data 0 = unsigned 1 = signed |  |  |
| 12 | Loop mode enable bit. 0 = disable 1 = enable |  |  |
| 11:0 | Ec is current envelope in format of 6.6 (Six bits integer and six bits fraction). 000h stands for 0dB, FFFh stands for -63.984375 dB. Note: Ec for Bank A is controlled by EBUF1 or EBUF2. Ec for Bank B is static (does not inc/dec per sample period) and is used for better volume control. |  |  |

#### EBUF1 (Bank A Only)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + F4h | 32-bits | Read/Write | xxxxxxxxh |
| Bit | Description |  |  |
| 31:30 | AMS_H is Amplitude Modulation Step High part. AMS[3:2] |  |  |
| 29:28 | EMOD (Envelope Mode) defines the mode of operation 00 = DEC Mode (ramp from 0dB to –64dB) 01 = INC Mode (ramp from –64dB to 0dB) 10 = DELAY Mode (See Sub-Mode [27:26]) 11 = STILL Mode (Ec is static, buffer never toggles) |  |  |
| 27:26 | DEC Mode = EMAT[11:10] INC Mode = EMAT[11:10] DELAY Mode = Sub-Mode 00 = Delay_hold 01 = Delay_start 10 = Delay_stop 11 = Reserved STILL Mode = Reserved |  |  |
| 25:20 | DEC Mode = EAMT[9:4] INC Mode = EMAT[9:4] Delay Mode = Reserved STILL Mode = Reserved |  |  |
| 19:16 | DEC Mode = EMAT[3:0] INC Mode = EMAT[3:0] Delay Mode = EDLY[19:16] STILL Mode = Reserved |  |  |
| 15:8 | DEC Mode = EINIT[7:0] Provides the initial value of the 8-bit counter ECNT INC Mode = EINIT[7:0] Provides the initial value of the 8-bit counter ECNT Delay Mode = EDLY[15:8] Delay counter STILL Mode = Reserved |  |  |
| 7:0 | DEC Mode = ECNT[5:0] 8-bit Counter INC Mode = ECNT[5:0] Delay Mode = EDLY[7:0] STILL Mode = Reserved |  |  |

EAMT[11:0] = Absolute ramping amount with a range from 0dB to 63.984375dB.

Every time EAMT = 000h, the Envelope Engine will toggle the CEBC flag (Select the other buffer)

ECNT[7:0] = 8-bit Counter; Decrements every 48kHz clock; Whenever it reaches 00h:

ECNT reloads with EINT

EAMT decreases by 1

Ec decreases by 1

EINIT[7:0] = Initial value of ECNT[7:0]

EDLY[19:0] = 20-bit Delay Counter; Decrements every 48kHz clock; Whenever it reaches 00000h:

If Sub-Mode = Delay_hold: Engine will toggle the CEBC flag

If Sub-Mode = Delay_start: Engine with reset the DLY flag

If Sub-Mode = Delay_stop: Engine will reset the START/STOP flag (to stop)

#### EBUF2 (Bank A Only)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + F8h | 32-bits | Read/Write | xxxxxxxxh |
| Bit | Description |  |  |
| 31:30 | AMS_L is Amplitude Modulation Step Low part. AMS[1:0] |  |  |
| 29:28 | EMOD (Envelope Mode) defines the mode of operation 00 = DEC Mode (ramp from 0dB to –64dB) 01 = INC Mode (ramp from –64dB to 0dB) 10 = DELAY Mode (See Sub-Mode [27:26]) 11 = STILL Mode (Ec is static, buffer never toggles) |  |  |
| 27:26 | DEC Mode = EMAT[11:10] INC Mode = EMAT[11:10] DELAY Mode = Sub-Mode 00 = Delay_hold 01 = Delay_start 10 = Delay_stop 11 = Reserved STILL Mode = Reserved |  |  |
| 25:20 | DEC Mode = EAMT[9:4] INC Mode = EMAT[9:4] Delay Mode = Reserved STILL Mode = Reserved |  |  |
| 19:16 | DEC Mode = EMAT[3:0] INC Mode = EMAT[3:0] Delay Mode = EDLY[19:16] STILL Mode = Reserved |  |  |
| 15:8 | DEC Mode = EINIT[7:0] Provides the initial value of the 8-bit counter ECNT INC Mode = EINIT[7:0] Provides the initial value of the 8-bit counter ECNT Delay Mode = EDLY[15:8] Delay counter STILL Mode = Reserved |  |  |
| 7:0 | DEC Mode = ECNT[5:0] 8-bit Counter INC Mode = ECNT[5:0] Delay Mode = EDLY[7:0] STILL Mode = Reserved |  |  |

EBUF2 is totally as same as EBUF1except bits 31-30 are AMS_L (Amplitude Modulation Step Low part).

EAMT[11:0] = Absolute ramping amount with a range from 0dB to 63.984375dB.

Every time EAMT = 000h, the Envelope Engine will toggle the CEBC flag (Select the other buffer)

ECNT[7:0] = 8-bit Counter; Decrements every 48kHz clock; Whenever it reaches 00h:

ECNT reloads with EINT

EAMT decreases by 1

Ec decreases by 1

EINIT[7:0] = Initial value of ECNT[7:0]

EDLY[19:0] = 20-bit Delay Counter; Decrements every 48kHz clock; Whenever it reaches 00000h:

If Sub-Mode = Delay_hold: Engine will toggle the CEBC flag

If Sub-Mode = Delay_start: Engine with reset the DLY flag

If Sub-Mode = Delay_stop: Engine will reset the START/STOP flag (to stop)

# NX - System Overview

The 4DWave-NX is a 64-Voice PCI Wavetable/DirectSound Audio Accelerator.  It is packaged in a 100-pin LQFP targeted at the notetop PC market and is designed to be pin compatible with Trident's next generation desktop audio controller.  As shown in Figure 1 below, the 4DWave-NX will interface to:

- PCI bus (variable frequency 25 - 33 MHz)

- A dual AC'97 serial bus for communication with an AC'97 Codec

- A Digital Enhanced Game Port

- MIDI port

- I2S Interface

- 4DWave-NX High Level System Diagram

There will be a single driver set for the 4DWave devices (current and future).  The approach will allow a stable, maintainable code base.  Support of typical audio features is handled variously by hardware and driver software.  The current split of functionality is detailed below.

Hardware acceleration:

Wavetable synthesis

chorus effects

reverb effects partially accelerated in hardware

Driver software support:

FM Synthesis (VirtualFM)

MIDI (VirtualGM)

Reverb filtering

3D positional audio effects

The 3D positional audio effects will be handled by code from a third-party inserted into the 4DWave-NX driver.  The Microsoft provided HEL layer for 3D audio will not be used.

# NX - Block Overviews

- 4DWave-NX Block Diagram

## NX - PCI Interface

The PCI interface is completely PCI 2.1 compliant. By being PCI 2.1 compliant, the device is also Plug-n-Play compliant. It consists of separate master and slave controllers that operate independently.  The master supports burst cycles. The slave can be programmed to accept both I/O and Memory (memory mapped I/O) cycles for 4DWave registers. The 4DWave register space can be mapped by configuring the PCI Config registers for I/O and Memory base addressing. The legacy register space is only accessible through I/O cycles and cannot be remapped.

4DWave uses a single PCI interrupt. All interrupts are combined together internally to form this single interrupt signal. Internal registers must be accessed to determine the nature of the interrupt. This function is programmed differently depending if the device is in legacy emulation mode or not.

The PCI Interface also provides “scatter-gather” memory access when operating under Windows 9x or NT. This allows the accelerator to view the external address space as a contiguous 16 MB memory space. TLB look-up with a 64 entry cache is used to translate between the virtual and physical address spaces. This allows support for larger memory buffers that are not contiguous in memory. The scatter-gather mechanism is based on 4KB pages.

The device can be configured to use the Distributed DMA mechanism to perform DMA I/O trapping, programming and operation. This can only be used with system chipsets that support the DDMA protocol. The device conforms to the DDMA 6.0 specification.

When legacy DMA support is required and the chipset does not support DDMA, the 4DWave device performs snooping of the DMA I/O programming cycles. It uses this to configure itself as a PCI Master and then performs the DMA cycle. The device will also capture some DMA I/O read cycles to respond with the correct status information.

If neither DDMA nor legacy DMA support can be used, NX can be configured to perform legacy DMA on the PCI bus through Intel’s PC/PCI protocol. The PC/PCI protocol is a 2 pin interface which sends requests directly to the 8237 DMA controller (located in the PCI to ISA bridge). The 8237 responds by granting NX and then puts the DMA cycle on the PCI bus. The NX will then respond to the IO portion of the DMA cycle.

## NX - Legacy

The 4DWave supports both the SoundBlaster Pro and SoundBlaster 16 register sets. This includes the OPL3 RAM. When configured to support legacy operation, the device will respond to all I/O cycles in the legacy address regions. These are used to support both FM and General MIDI emulation in DOS.

The VirtualGM and VirtualFM technologies are based on hardware and software to provide the emulation of FM synthesis and General MIDI command interpretation in DOS or Windows. The hardware provides all the registers for reads and writes to legacy locations. Depending on the register or bit written, the hardware will engage the software to generate the correct result. This maintains complete legacy support at a low cost to the user.

The 4DWave also supports the legacy Game Port. This is the 8-bit joystick port found on most audio accelerators and many “Super I/O” chips. In addition to supporting legacy operation, the game port was enhanced to allow a dramatic reduction in both bus traffic and CPU utilization by removing the requirement of “polling”. This support can be found when using Microsoft’s DirectInput API.

The MIDI port is supported with an MPU-401 compatible UART. This port can also be used in an emulation mode (VirtualGM) to support synthesis in DOS mode games.

## NX - Stream Buffer

The stream buffer is used to buffer the data streams between the accelerator engine and the system memory. The stream buffer supports up to 64 channels with each having 4 Dwords of buffering. (Analysis was done that shows this is the “sweet spot” for cost versus performance.) The stream buffer is primarily used for playback of up to 64 streams of audio or effects. However, it can be also configured to support three (3) channels of capture data for WDM, chorus, and reverb.

## NX - Address Engine

The address engine supports 64 voice channels. Stereo, mono, 8-bit, and 16-bit formats are supported. The first 32-channels, designated Bank A, have a complete parameter set to support MIDI (GM only requires 24 voices). The second 32-channels, designated Bank B, are optimized for effect processing and DirectX/WDM audio streams.

Two LFOs are used for music synthesis. The Bank A LFO is used with MIDI for tremelo (FM) and vibrato (AM). The Bank B LFO is used for chorus or effect pitch shifting. This means that the chorus playback taps must be assigned to the Bank B voices.

Each channel is sample rate converted to 48kHz. The address engine performs all the sample address calculation including using a channel specific sample rate conversion factor and a LFO sine wave to offset the address to simulate a tremolo (FM) affect.

The LFO used by the address engine is passed on to the envelope engine to be used as a vibrato (AM) effect.

## NX - Envelope Engine

The envelope engine controls the channel volume. For the first 32 channels, it supports two global volumes, a per/channel volume, a left-right PAN, a chorus volume, and a reverb volume. The first set also includes two volume slope buffers to allow a MIDI ADSR (Attack-Decay-Sustain-Release) curve to by performed. The second 32 channels only support a per/channel volume, a left-right PAN, a chorus volume, and a reverb volume. All volume controls operate in dBs (decibels) of attenuation. This allows the attenuations to simply be summed before sending the composite to the mixer.

## NX - Mixer

The mixer supports a 26-bit accumulation on three separate stereo channels. The hi-resolution accumulation allows the 64 voices to be completely accumulated without degradations such as “clipping”. There is a separate mixer channel for main (AC’97 stereo out), reverb, and chorus.

The main mixer channel data is buffered in a FIFO before being accessed by the AC’97 interface. This allows the 4DWave device to get several samples ahead and “averages” out the bus delay caused by other PCI or system devices.

The reverb mixer channel also is buffered in a FIFO for 4-channel operation with a second AC’97 codec.

## NX - Capture Channel

The capture channel is an AC’97 specific channel. It is used to capture record data samples from the AC’97 codec. The capture channel also supports independent down sampling and format conversion. All AC’97 data is sampled at 48kHz and can be captured in 16-bit stereo. This requires 4-bytes per sample or 192K bytes for 1 second of record data. By down sampling, the bus and memory space can be conserved. For instance, most voice functions can use 8-bit mono samples at 8kHz. This record data stream only uses 8K bytes for 1 second.

## NX - I2S Interface

The I2S interface supports input stereo data at variable input sample rates. The interface is intended to be used with a PCMCIA “zoom video” device or a MPEG2 decoder chip-set. The data is piped to the address engine where it is sample rate converted and then mixed with the output audio stream. This provides a low latency playback mechanism, which is assumed by these devices.

### System Environment

The only mode of operation is the low latency digital loopback or mixing mode. In this mode, the data is captured over the interface and buffered. The PSB and respective Address Engine voice is set up for playback. Whenever the PSB needs more data, it is pulled from the I2S interface rather then over the PCI bus. The I2S interface is responsible for dealing with any synchronization issues.

### Software Programming Model

The Driver software will be required to work with the Application & API in setting up the interface. The following steps are required to initialize the hardware.

Steps 1 & 2 are only needed if the API does not inform us of the input sample rate. DirectMusic should pass us this parameter when the channel is requested.

- Turn on the external I2S interface, but do NOT turn on the internal I2S interface or the Address Engine voice.

- Read the clock count register to determine the input SCLK frequency. Use this value to setup the SRC values for the Address Engine.

- Program the allocated Address Engine voice in a continuous playback looping mode with the determined SRC factors. The loop should be 16 samples long. The data format will be 16-bit signed stereo. Assign the PSB voice to the I2S interface.

- Turn on the internal I2S interface and then the Address Engine voice channel.

TBD: How does the I2S volume get controlled? Do we (Can we) add it to the volume control panel? Or is this a DirectMusic / WDM controlled option.

## NX - AC’97 Interface

The AC’97 interface supports the 5-pin AC’Link interface to the codec. An additional input is used to support a second AC’97 codec. The interface currently supports both the standard AC’97 codec configuration and the Rev 2.0 Addendum. The interface supports stereo out and stereo in from both codecs simultaneously. The interface also includes a register set that allows access to the external AC’97 codec registers. Power management control is also included.

The AC’97 interface operates at a fixed 48 kHz sample rate. The 4DWave will provide 20-bit stereo output for playback and will accept 16-bit stereo input for record.

## NX - SPDIF Interface

### System Environment

The SPDIF interface is a single output pin that provides serial data in the SPDIF format. The origin of the data can either be from external memory (AC-3 data) or directly from the NX mixer output. Data retrieved from external memory can be supplied at 3 frequencies 32KHz, 44.1KHz and 48KHz. The SPDIF output pin is shared by the I2S interface data input which means that the user can either have I2S or SPDIF but not both.

### Software Programming Model

The Driver software will be required to work with the Application & API in setting up the interface. The following steps are required to initialize the hardware. There are 2 data sources for SPDIF and each has a separate programming model.

SPDIF DATA SOURCE FROM INTERNAL NX AUDIO MIXER (48KHZ Only)

- Program the SPDIF Channel Status register.

- Program SPDIF mode (00)

- Remove SPDIF reset

- Enable SPDIF output

SPDIF DATA SOURCE FROM EXTERNAL MEMORY (32, 44.1 and 48KHZ)

- Program SPDIF Channel Status, ESO, CSO and LBA registers (All of these should be 4 Dword aligned).

- Program SPDIF mode

- Remove SPDIF reset

- Enable SPDIF bus mastering

- Poll SPDIF CSO until CSO has incremented (shows valid data has entered SPDIF fifo)

- Enable SPDIF output

When disabling and re-enabling SPDIF output when the data source is external memory the programmer must disable SPDIF bus mastering and resume programming from step #4 above. This will flush the SPDIF fifo of any old data.

## NX - EEPROM Interface

The EEPROM interface is a 3 pin interface (ROM_CLK, ROM_CS and ROM_DATA) which reads data out of an external EEPROM. Immediately after PCI reset is de-asserted the EEPROM interface reads locations E, F and 0 of the 16x16 EEPROM and loads these values into the PCI subsystem vendor ID, subsystem ID and power data registers respectively. EEPROM locations 0-D are used for power management data which is loaded into the power management data register in the PCI configuration space (0x4F) when the appropriate value is written in the data select field. The EEPROM must be programmed before the NX is used in system operation. NX only supports the Microchip 93C06/46 Serial EEPROM or similar devices (16x16, 3-pin I/F, 1us access time).

# NX - Power Management

## References

The Audio Power Management system is influenced, encouraged, and specified by the following documents.

- ACPI - Advanced Configuration and Power Interface Specification – Revision 1.0

- PPMI - PCI Bus Power Management Interface Specification – Revision 1.0; June 30, 1997

- OnNow - Device Class Power Management Reference Specification – Audio Device Class V1.0

- AC’97 - Audio Codec ’97 Component Specification – Revision 1.03; September 15, 1996

## Overview

Over the past two years there have been multiple industry power management initiatives for the PC platform. Recently, these initiatives have begun to converge on a consistent definition and direction that encompasses the system, the OS, the application, and the IC device.

Our perspective is from that of the audio IC device vendor. From this viewpoint we have a number of goals, both technical and marketing.

- Support all required features to allow “logo” compliance

- Required states

- Required state transitions & events

- Required pins/signals

- Required state transition latency

- Support advanced features as required to support customer needs

- Power dissipation requirements

- Additional pins/signals

- Separate power wells

- Most efficient implementation

- Correct software - hardware architecture

## State Definitions

There are four defined power states of a device: D0, D1, D2,  & D3. The stated D3 is sometimes referred to as two sub-states: D3hot & D3cold. The 4DWave-NX has no difference between D3hot (power still on) and D3cold (power off) power states. Therefore, we will use the general definition of D3.

### States (Defined in OnNow Device Class Specification)

D0 = Full power & functionality state.

D1 = Low-power state with no context or data loss. This is referred as a “pause” state. Fast transition to the D0 state.

D2 = Low-power state with some context and data loss. Transition to the D0 state <200us.

D3 = Off state. Device context is lost. Full initialization and context restoration required. There is no time limit for state transitions to D0.

### Caveats & Assumptions

States D0, D2, & D3 are required to be supported.

All lost device context (if any was lost) must be restored by the device driver when returning the device to the D0 state.

The software (OS) has a better understanding of the system / application needs. For systems with WDM drivers, the Class Driver will control all state transitions. (No need for hardware timers or internal event generator for state transitions.)

The software will not trigger a state transition unless the activity or inactivity determines that one is prudent.

The AC’97 Codec will have channels in the “ready” state in less then 200 us after a “warm” reset.

NX supports a special D4 power state which allows the driver to power-up the device w/o changing the power state register.

### Power Management State Diagram

## Audio Power State Definitions

The audio device hardware actually only needs to support two (2) “on” states: D0-Full On & D1-Idle. 4DWave-NX also supports the lower power D2 state and a special pseudo D0 state (D4).

### D0

The power is on and the device in operating. The device is either Playing, Recording, or Full Duplex.

- Playing

- File/buffer handle(s) are open.

- Device is playing sample data from file/buffer. (MIDI, WAVE, etc.)

- Recording

- File/buffer handle(s) are open.

- Device is capturing sample data to file/buffer.

- Foreground Recording

- Normal application is recording.

- Recording is considered foreground unless specifically designated low priority.

- Background Recording

- Low priority recording application. (Voice response systems, security, etc.)

- Full Duplex

- Device is both Playing and Recording (foreground)

| POWER STATE | Bus | 4DWave-NX | AC’97 Codec |
| --- | --- | --- | --- |
| D0 State | 33 MHz operation | Complete functionality | All channels are enabled. All ADC/DAC are turned on. Clocks are running. |

### D1

The power is on but the consumption is less then the D0 state. Device context is completely coherent and static. The device is not recording (background or foreground) or playing. File/buffer handles are all closed.

The PPMI specification allows some operation in the D1 state. The 4DWave-NX is also dependent on the PCI clock for operation in the D1 state.

| POWER STATE | Bus | 4DWave-NX | AC’97 Codec |
| --- | --- | --- | --- |
| D1 State | 33MHz operation | NX PCI Bus Mastering is disabled. Any channels currently running will eventually stop. NX no longer responds to I/O or memory cycles. | All channels are enabled. All ADC/DAC are turned on. Clocks are running |

### D2

The power is on but the consumption is less then the D0 state. Device context is completely coherent and static. The device is not recording (background or foreground) or playing. File/buffer handles are all closed.

The difference between D1 and D2 power states vary between the Device Class & PPMI specification. The PPMI specification was interpreted as more limiting and therefore chosen to define the D1 & D2 states.

| POWER STATE | Bus | 4DWave-NX | AC’97 Codec |
| --- | --- | --- | --- |
| D2 State | 0 – 33MHz operation | NX PCI Bus Mastering is disabled. Any channels currently running will eventually be stopped. NX no longer responds to I/O or memory cycles. AC97 Codec powered down Game, MIDI, I2S and EEPROM interfaces are clamped (EEPROM is not clamped if in Notebook mode) | AC97 is powered down according to power management policies set in ACR0. |

### D3

The power can be on or off. The device will generate an internal PCI_RST# when exiting the D3 state (if power remains on) and software will need to do a complete context restoration.

| POWER STATE | Bus | 4DWave-NX | AC’97 Codec |
| --- | --- | --- | --- |
| D3 State | <33 MHz operation | NX PCI Bus Mastering is disabled. Any channels currently running will eventually be stopped. NX no longer responds to I/O or memory cycles. AC97 Codec powered down Game, MIDI, I2S and EEPROM interfaces are clamped (EEPROM is not clamped if in Notebook mode) | AC97 is powered down according to power management policies set in ACR0. |

### D4

By setting bit[2] of the Power Management Control register the software driver can bring the NX into a “pseudo DO” state regardless of the current power management state.  Once this bit is cleared NX will power down to the current state held in the power management state bits.

| POWER STATE | Bus | 4DWave-NX | AC’97 Codec |
| --- | --- | --- | --- |
| D4 State | 33 MHz operation | Complete functionality | All channels are enabled. All ADC/DAC are turned on. Clocks are running. |

# NX - Architecture Constraints

## Legacy

The legacy support of Sound Blaster assumes the OPL3 RAM exists in hardware. For die size & cost reasons, a single RAM is used for both the OPL3 emulation and the Bank B (Second 32 voices) playback channels. This means that when the 4DWave is emulating the OPL3 in hardware, that the Bank B channels should all be disabled and not used. This should only occur during a DOS-Box or full DOS mode. Likewise, when there are active channels in Bank B, OPL3 emulation is not available in the hardware. This must be emulated in software through I/O trapping.

## 64 Channels

The 64 Channels of audio can cause some issues with PCI bandwidth and utilization. The 4DWave architecture was optimized for observed scenarios of high end games and music playback. However, “psychotic” scenarios can be construed that are fundamentally legal (although they typically result is very distorted results) which could effect system performance. This is unavoidable due to the nature of audio stream when taken to the edges of its configurability and capability. For instance, “up sampling” an audio stream reduces the effectiveness of bursting by not using most of the data transferred. 4DWave uses bursting (as do most other advanced Audio Accelerators) to reduce the overall system load. If a few channels are significantly (3 octaves) up-sampled, there are no issues. However, if all 64 channels are taken to this extreme, then the PCI bus is at its limit to try and transfer the data to the accelerator. Care must be taken to understand and possibly off-load the synthesis of these streams, in some cases, from the accelerator to the CPU.

## PCI Bus Clock

The 4DWave algorithm engines operate off of the PCI clock. This allows lower power and reduces the cost by not requiring a PLL or another clock. The downside to this is that the 4DWave is dependent on this clock for performance. A 10% drop in the PCI clock (30MHz), results in a 10% drop in the 4DWave performance. The 4DWave is designed with over 25% overhead. However, if the 4DWave was put in a 25MHz (~30% reduction) PCI system, it may not be able to process all 64 channels. Likewise, the PCI Bus would potentially not have the bandwidth to transfer the data.

## Programming Capture Channels

When programming a 4D Wave channel to act as a capture channel there are several special settings that must be followed.

- LBA (the start address of the capture buffer) must be 4 dword aligned.

- When in non-loop mode ESO = # of samples to capture (must be a 4 Dword multiple).

- When in loop mode ESO = (# of samples) – 1 (must be a 4 Dword multiple – 1).

- DELTA must always be programmed to 0x1000. 4D Wave only allows capturing at a 48KHz rate.

- CSO (current sample offset) must be 0.

- ALPHA must be 0.

- FMS must be 0 (no frequency modulation can be applied to the capture buffer).

- The data type must always be set to 16-bit stereo. 4D Wave does not support capturing in any other format.

- VOL (channel volume) must be 0xFF (mute).

- RVOL (reverb volume) must be 0x7F (mute).

- CVOL (chorus volume) must be 0x7F (mute).

- When programming RVOL and CVOL (Audiobase + 0xEC) only word writes are allowed. If a dword access is done to this address the EC value for the previous and current channel may be corrupted.

## Using Delay Start Mode

When programming a channel to use the Delay Start Mode feature (EBUF mode) the programmer must first set the channel’s delay bit in the DLY_A register (0x88) before setting the channel’s start bit.

## Using SBDELTA/DELTA_R

When setting up the 4D Wave to handle legacy playback or record the SBDELTA (register 0xAC) must be used. During legacy playback SBDELTA is (playback frequency)/48KHz in a 4.12 format. When programming legacy record SBDELTA is the inverse of its playback counter part: 48KHz/(record frequency) in a 4.12 format.

# NX - Internal RAM/EEPROM Configuration

The 4DWave-NX uses two types of internal RAMs: 128x32 and 64x32. The 64x32 is used for the TLB Cache. All the other RAMs are comprised of 1 or 2 of the 128x32’s.

## NX - Internal ARAM & ERAM Mapping

The shaded area is used for OPL3 and Mixer CT1745 emulation. In this mode, only 32 channels are available.

## NX - Internal TLB Cache Mapping

## NX - Internal PSB Audio Sample Cache Mapping

## NX - External EEPROM Mapping

# NX - Address Map and Register Description

The 4DWave-NX has addressable locations in three address spaces: PCI Configuration Space, I/O Space, and Memory Space. These spaces are initialized through Plug-n-Play system software routines by configuring the registers in the 256-byte PCI Configuration Register space.

## NX - PCI Configuration Space

The 4DWave-NX is a single function device and, therefore, only supports a single PCI configuration space. The device is both PPMI and DDMA compatible, which requires additional registers for set-up. The device does support an external Serial EEPROM, therefore, Subsystem Vendor ID, Subsystem ID and PPMI “Data” values (Power Consumption and Dissipation) are supported.

- 4DWave-NX PCI Configuration Register Space

| Offset (Hex) | Byte 3 | Byte 2 | Byte 1 | Byte 0 |
| --- | --- | --- | --- | --- |
| 00h | Device ID (Read Only = 2001h) |  | Vendor ID (Read Only = 1023h) |  |
| 04h | Status |  | Command |  |
| 08h | Class Code (Read Only = 040100h) |  |  | Revision ID (Read Only = 00h) |
| 0Ch | BIST (Read Only = 0000h) | Header Type (Read Only = 00h) | Latency Timer | Cache Line Size (Read Only = 0000h) |
| 10h | Audio IO Base Address Register |  |  |  |
| 14h | Audio Memory Base Address Register |  |  |  |
| 18-28h | RSVD (Read Only = 00000000h) |  |  |  |
| 2Ch | Subsystem ID (Read Only = ROM) |  | Subsystem Vendor ID (Read Only = ROM) |  |
| 30h | RSVD (Read Only = 00000000h) |  |  |  |
| 34h | RSVD (Read Only = 000000h) |  |  | Cap_Ptr (Read Only = 48h) |
| 38h | RSVD (Read Only = 00000000h) |  |  |  |
| 3Ch | MAX_LAT (Read Only = 05h) | MIN_GNT (Read Only = 02h) | Interrupt Pin (Read Only = 01h) | Interrupt Line |
| 40h | DDMA_CFG |  |  |  |
| 44h | RSVD (Read Only = 00h) | Power Management Control | LEGACY_DMA | LEGACY_IOBASE |
| 48h | Power Management Capabilities (Read Only = 0601h |  | Next_Ptr (Read Only = 00h) | Cap_ID (Read Only = 01h) |
| 4Ch | Power Data (Read Only = ROM) | PMCSR_BSE (Read Only – 00h) | Power Management Control/Status |  |
| 50h | RSVD (Read Only = 0000h) |  | Interrupt Snooping Control |  |
| 70h | GPIO_CTRL |  | RSVD(Read Only = 00h) | NB_CTRL |

### NX - General Configuration

#### Vendor ID (Offset = 00h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [15:0] | 1023h | R | Trident Microsystems’s PCI Vendor ID |

#### Device ID (Offset = 02h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [15:0] | 2001h | R | 4DWave-NX Device ID |

#### Command (Offset = 04h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [15:10] | 00000b | R | Reserved |
| [9] | 0 | R | Fast Back-to-Back enable for master transactions. The 4DWave does not support this feature. This bit is hard wired to a ‘0’. |
| [8] | 0 | R/W | SERR# enable. 0 = Disables the SERR# Driver 1 = Enables the SERR# Driver |
| [7] | 0 | R | Address / Data stepping or Wait cycle control. The 4DWave does not support this feature. This bit is hard wired to a ‘0’. |
| [6] | 0 | R/W | Parity Enable. 0 = Ignores parity errors. 1 = Report parity errors. |
| [5] | 0 | R | VGA palette snoop. The 4DWave does not support this feature. This bit is hard wired to a ‘0’. |
| [4] | 0 | R | Enable the “Memory Write and Invalidate” command. The 4DWave does not support this feature. This bit is hard wired to a ‘0’. |
| [3] | 0 | R | Enable the device to monitor Special Cycle commands. The 4DWave does not support this feature. This bit is hard wired to a ‘0’. |
| [2] | 0 | R/W | Bus Master Enable 0 = Disables the Bus Master Operation 1 = Enables the Bus Master Operation |
| [1] | 0 | R/W | Memory Space Enable 0 = Disables the device to respond to Memory Space cycles 1 = Enables the device to respond to Memory Space cycles |
| [0] | 0 | R/W | I/O Space Enable 0 = Disables the device to respond to I/O Space cycles 1 = Enables the device to respond to I/O Space cycles |

#### Status (Offset = 06h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [15] | 0 | R/Write 1 to Clear | Parity Error Detected. This bit is set by the device, whenever it detects a parity error, even when the Command bit [6] is disabled. |
| [14] | 0 | R/Write 1 to Clear | SERR# status. Set whenever this device asserts SERR#. |
| [13] | 0 | R/Write 1 to Clear | Received Master-Abort status. Set whenever a transaction to this device is terminated with a Master-Abort. |
| [12] | 0 | R/Write 1 to Clear | Received Target-Abort status. Set whenever a transaction by this device is terminated with a Target-Abort. |
| [11] | 0 | R | Signalled Target-Abort status. Set whenever a transaction to this device is terminated with a Target-Abort. This will never happen on 4DWave |
| [10:9] | 01b | R | DEVSEL Timing. The device supports “Medium” DEVSEL timing. |
| [8] | 0 | R/Write 1 to Clear | Data Parity Error Detected. This bit is set by the Master device when 1) it detects a data parity error, 2) it is the current Master, & 3) when the Command bit [6] is enabled to report parity errors. |
| [7] | 0 | R | Capable of Fast Back-to-Back cycles. The 4DWave does not support this feature. This bit is hard wired to a ‘0’. |
| [6] | 0 | R | User Definable Features. The 4DWave does not support this feature. This bit is hard wired to a ‘0’. |
| [5] | 0 | R | 66 Mhz Capable. The 4DWave does not support this feature. This bit is hard wired to a ‘0’. |
| [4] | 1 | R | Capabilities List. The 4DWave supports “New Capabilities” structure. |
| [3:0] | 0000b | R | Reserved |

#### Revision ID (Offset = 08h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:0] | 00h | R | 4DWave-NX Revision. First revision = 00h. |

#### Class Code (Offset = 10h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [23:16] | 04h | R | Base Class. Multimedia = 04h |
| [15:8] | 01h | R | Sub Class. Audio = 01h |
| [7:0] | 00h | R | Interface Class. None Defined = 00h |

#### Cache Line Size (Offset = 0Ch)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:0] | 00h | R | Cache Line Size. The 4DWave does not support this feature. This bit is hard wired to a ‘0’. |

#### Latency Timer (Offset = 0Dh)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:3] | 00000b | R/W | Latency Timer in 8-PCI Clock increments. |
| [2:0] | 000b | R | Latency Timer LS 3-bits. Always 000b. This forces the latency to be incremented by 8-PCI Clocks at a time. |

#### Header Type (Offset = 0Eh)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7] | 0 | R | Multi-function device enable. The 4DWave-NX is not a multi-function device. |
| [6:0] | 0000000b | R | Header Layout. The 4DWave-NX uses a “standard” configuration layout. |

#### BIST (Offset = 0Fh)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:0] | 00h | R | The 4DWave does not support this feature. These bits are hard wired to a ‘00h’. |

#### I/O Base Address (Offset = 10h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [31:8] | 000000h | R/W | I/O Base Address [31:8]. Specifies the MS 24-bits of the Audio I/O base address. |
| [7:2] | 000000b | R | I/O Base Address [7:2]. Forces alignment to a 256-byte block. |
| [1] | 0 | R | Reserved. |
| [0] | 1 | R | I/O Base identifier. |

#### Memory Base Address (Offset = 14h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [31:12] | 00000h | R/W | Memory Base Address [31:12]. Specifies the MS 20-bits of the Audio Memory base address. |
| [11:4] | 00h | R | Memory Base Address [11:4]. Forces alignment to a 4K-byte block. |
| [3] | 0 | R | Prefetchable. The 4DWave uses this space for Memory mapped I/Os. This is not a prefetchable or mergeable address space. |
| [2:1] | 00b | R | Type. This can be located anywhere in 32-bit address space. |
| [0] | 0 | R | Memory Base identifier. |

#### Subsystem Vendor ID (Offset = 2Ch)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [15:0] | Rom[E] | R | Subsystem Vendor ID is loaded from ROM Word [E] when Desktop mode is enabled. If Notebook mode is enabled this register will return 0 unless written to using the SID/SVID write enable in PCI config register 45h. |

#### Subsystem ID (Offset = 2Eh)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [15:0] | Rom[F] | R | Subsystem Vendor ID is loaded from ROM Word [F] when Desktop mode is enabled. If Notebook mode is enabled this register will return 0 unless written to using the SID/SVID write enable in PCI config register 45h. |

#### Capabilities Pointer (Offset = 34h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:0] | 48h | R | Capabilities Pointer. Absolute offset to the start of the extended capabilities configuration space. |

#### Interrupt Line (Offset = 3Ch)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:0] | 00h | R/W | Interrupt Line. This is used by the driver and Plug-n-Play setup code to identify the interrupt used by this device. |

#### Interrupt Pin (Offset = 3Dh)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:0] | 01h | R | Interrupt Pin. This is used to tell what pin the device uses. The 4DWave-NX uses the INTA# pin. |

#### Minimum Grant (Offset = 3Eh)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:0] | 02h | R | Minimum Latency. The minimum time to complete a burst is 500 ns. (2 x .25us increment) This is assuming that the chipset typically has a 7 clock lead off and the burst is for 4 Dwords. |

#### Maximum Latency (Offset = 3Fh)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:0] | 05h | R | Maximum Latency. The maximum time between request cycles is set at 1.25us. A 48kHz sample period is 20.8 microseconds. This is assuming an average of 16 data requests per sample period. |

### NX - Legacy Configuration

#### Distributed DMA Configuration (Offset = 40h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [31:4] | 0000000h | R/W | DDMA Base Address [31:4] |
| [3] | 0 | R/W | Non Legacy Extended Addressing Control (Fully 32 bit Addressing) 0 = disabled 1 = enabled |
| [2:1] | 00b | R | Legacy DMA Transfer Size Control 00 = 8 bit transfer, legacy |
| [0] | 0 | R/W | DDMA Slave Channel Access Enable Control 0 = disabled 1 = enabled |

#### Legacy I/O Base (Offset = 44h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7] | 0 | R/W | MPU-401 Legacy Address Space Enable 0 = MPU401Base disable 1 = MPU401Base enable |
| [6] | 0 | R/W | MPU-401 Legacy Address Space Select 0 = MPU401Base 0330h-0333h 1 = MPU401Base 0300h-0303h |
| [5] | 0 | R/W | Game Port Legacy Address Space Enable 0 = GAMEBase disable 1 = GAMEBase enable |
| [4] | 0 | R/W | Game Port Legacy Address Space Select 0 = GAMEBase 0200h-0207h 1 = GAMEBase 0208h-020Fh |
| [3] | 0 | R/W | Adlib Legacy Address Space Enable 0 = ADLIBBase disable 1 = ADLIBBase enable |
| [2] | 0 | R/W | Adlib Legacy Address Space Select 0 = ADLIBBase 0388h-038Bh 1 = ADLIBBase 038Ch-038Fh |
| [1] | 0 | R/W | Sound Blaster Legacy Address Space Enable 0 = SBBase disable 1 = SBBase enable |
| [0] | 0 | R/W | Sound Blaster Legacy Address Space Select 0 = SBBase 0220h-022Fh 1 = SBBase 0240h-024Fh |

#### Legacy DMA (Offset = 45h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:6] | 0h | R | Reserved |
| [5] | 0 | R/W | Subsystem ID / Subsystem VID Write Enable 0 = Disable PCI configuration writes to Subsystem ID and Subsystem VID registers. 1 = Enable PCI configuration writes to Subsystem ID and Subsystem VID registers. This feature needs to be used to program the SID and SVID registers when the NX is in notebook mode. |
| [4] | 0 | R/W | MIDI IN/OUT FIFO x8 Mode 0 = The MIDI input fifo is loaded serially either from MIDI_IN or the MIDI output fifo. 1 = The MIDI input fifo is connected to the MIDI output fifo using an 8-bit data bus. |
| [3] | 0 | R/W | PC/PCI Enable 0 = PC/PCI disabled 1 = PC/PCI enabled |
| [2] | 0 | R/W | Global IO SERR Enable 0 = All SERR generation from IO accesses is disabled 1 = SERR generation from selected IO addresses is enabled |
| [1] | 0 | R/W | Enable Legacy DMA Snooping/Trapping 0 = DMA trapping disable 1 = DMA trapping enable |
| [0] | 0 | R/W | DMA Snooping Channel Select 0 = DMA channel 1 trapping 1 = DMA channel 0 trapping |

#### Power Management Control (Offset = 46h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:5] | 000b | R | Reserved |
| [4] | 0 | R/W | IO Clamp Disable 0 = EEPROM, GAME, MIDI & I2S interfaces are clamped when in power state D2 or D3 1 = No interfaces are clamped in any of the power states |
| [3] | 0 | R/W | DMA Read Trap Disable 0 = Respond to DMA read cycles if enabled (TDMA). 1 = Do not respond to DMA reads of DMA CH0 or CH1. |
| [2] | 0 | R/W | Power Management State Over-ride 0 = Stay in current power state 1 = Over-ride power state setting and go to D4 power state (“pseudo D0”) |
| [1] | 0 | R/W | Power Management Disable 0 = Power management states are enabled 1 = Power management states are disabled |
| [0] | 0 | R/W | Audio Engine Reset 0 = Normal 1 = Reset Audio Registers & Wave Engine State Machines When this bit is ‘1’, the audio block (including wavetable, legacy audio, & AC’97 codec) will be reset. It must be set to ‘0’, to exit reset. This is intended for D3 power states. |

### NX - Power Management Configuration

#### Capabilities ID (Offset = 48h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:0] | 01h | R | Identifies the capability as being the PCI Power Management Registers. |

#### Next Item Pointer (Offset = 49h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:0] | 00h | R | By being 00h, indicates the end of the linked-list of extended capabilities. |

#### Power Management Capabilities (Offset = 4Ah)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [15:11] | 00000b | R | PME Support – The 4DWave-NX does not support PME generation. |
| [10] | 1 | R | D2 Support – The 4DWave does support the D2 power state. |
| [9] | 1 | R | D1 Support – The 4DWave does support the D1 power state. |
| [8:6] | 000b | R | Reserved |
| [5] | 0 | R | Device Specific Initialization – The 4DWave does not require any device specific initialization. |
| [4] | 0 | R | Auxiliary Power Source - The 4DWave-NX does not support separate internal power support. (Or PME generation.) |
| [3] | 0 | R | PME Clock - The 4DWave-NX does not support PME generation and, therefore, does not need the PCI clock to generate a PME. |
| [2:0] | 001b | R | Version – The 4DWave-NX supports Revision 1.0 of the PCI Power Management Interface specification. |

#### Power Management Control/Status (Offset = 4Ch)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [15] | 0 | R | PME Status – The 4DWave-NX does not support PME generation. |
| [14:13] | Rom[0] data[15:14] | R | Data Scale – The 4DWave-NX will load this from the serial ROM. This will be in bit[15:14] of the ROM Word [x] where Data_Select = x. |
| [12:9] | 0000b | R/W | Data Select - The Power Value select. It is used to address the ROM Word [x] where Data_Select = x. |
| [8] | 0 | R | PME Enable – The 4DWave-NX does not support PME generation. |
| [7:2] | 000000b | R | Reserved |
| [1:0] | 00b | R/W | Power State – This is used to determine the current power state. Software updates this register when changing power states 00b – D0 01b – D1 10b – D2 11b – D3hot |

#### Power Management Data (Offset = 4Fh)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:0] | Rom[0] Data[7:0] | R | Power Management Data – This data is loaded from the serial ROM whenever PCI Config Register [4Ch] is written. The data = bits [7:0] of the ROM Word [x] where Data_Select = x. |

### NX - Interrupt Snooping Configuration

#### Interrupt Snooping Control (Offset = 50h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [15:8] | 00h | R/W | Interrupt Vector: Compared to vector returned on AD[7:0] during a PCI interrupt acknowledge cycle. If it matches, then disable INTA# until all internal interrupts have been cleared. (The only interrupts enabled should be legacy SoundBlaster related.) |
| [7:1] | 00h | R | Reserved |
| [0] | 0 | R/W | Interrupt Snoop Enable 0 = Disable Interrupt Snooping 1 = Enable Interrupt Snooping (During Interrupt Acknowledge) |

#### Notebook Mode Control (Offset = 70h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [7:3] | 0 | R | Reserved |
| [2] | 0 | R/W | Dual I2S mux select (this bit is only operational in Notebook mode). 0 = Select I2S port 1 = Select I2S2 port |
| [1] | 0 – DT 1 - NB | R/W | PC/PCI / XARB mode select. When powering up in Desktop mode this defaults to 0. When powering up in Notebook mode this defaults to 1. 0 = pins are used for PC/PCI functionality 1 = pins are used for XARB functionality |
| [0] | 0 – DT 1 - NB | R/W | MIDI / I2S mode select. When powering up in Desktop mode this defaults to 0. When powering up in Notebook mode this defaults to 1. 0 = pins are used for MIDI functionality 1 = pins are used for dual I2S functionality |

#### GPIO Mode Control (Offset = 72h)

| Bits | POR | Read/Write | Description |
| --- | --- | --- | --- |
| [15:9] | 0 | R | Reserved |
| [8] | 0 | R | GPIO[2] Input status. This bit will reflect the data value on the GPIO[2] pin. |
| [7] | 0 | R/W | GPIO[2] Data value. When GPIO[2] is configured as an output this is the value that will be driven on the GPIO[2] pin. |
| [6] | 1 | R/W | GPIO[2] Input/Output Select. 1 = GPIO[2] is an Output Only pin. 0 = GPIO[2] is an Input Only pin. |
| [5] | 0 | R | GPIO[1] Input status. This bit will reflect the data value on the GPIO[1] pin. |
| [4] | 0 | R/W | GPIO[1] Data value. When GPIO[1] is configured as an output this is the value that will be driven on the GPIO[1] pin. |
| [3] | 1 | R/W | GPIO[1] Input/Output Select (this bit is only valid when the NX Notebook mode is enabled at power-up). 1 = GPIO[1] is an Output Only pin. 0 = GPIO[1] is an Input Only pin. |
| [2] | 0 | R | GPIO[0] Input status. This bit will reflect the data value on the GPIO[0] pin. |
| [1] | 0 | R/W | GPIO[0] Data value. When GPIO[0] is configured as an output this is the value that will be driven on the GPIO[0] pin. |
| [0] | 1 | R/W | GPIO[0] Input/Output Select (this bit is only valid when the NX Notebook mode is enabled at power-up). 1 = GPIO[0] is an Output Only pin. 0 = GPIO[0] is an Input Only pin. |

## 4DWave-NX I/O or Memory Space

The 4DWave can be configured to respond to the separate legacy register addresses, as well as, the combined Wave register space. The Wave register space can be accessed through traditional I/O cycle or memory mapped I/O cycles.

The Legacy registers are mapped into the bottom 64 bytes of the Wave register address space.

- 4DWave-NX Register Space

- Legacy Mapping into 4DWave-NX Register Space

The table below details the address map of the 4DWave-NX internal register set.  These Wave registers are addressable using either the Audio I/O Base Address or the Audio Memory Base Address. The legacy portion of these registers is available through their respective legacy addresses as well. By providing both an I/O and a memory aperture to these registers, the 4DWave can be tuned for both compatibility and performance. Both I/O and Memory apertures can be enabled simultaneously.

The Wave registers consume a 256-byte aligned address space. The I/O mapping only allows the 256-byte space to be accessed. The memory mapping consumes 4K-bytes, however, only the bottom 256-bytes actually addresses the Wave registers.

All register contents reside on-chip, however, some registers must be accessed through a base indexed manner. This is true for voice channel specific registers and OPL3 RAM locations. All I/O locations which are not defined will return H'00000000 when read.  There is no affect when writing to non-defined registers.

The Wave registers indexed by 00h through DFh are global in function and are not specific to a particular voice channel.

The registers E0h through EFh are voice channel specific for each of the 64 voice channels (0-63) while the registers F0h is implement for all 64 channels, however, F4h through FFh are implemented only for channels the first 32 voice channels (0-31).  The first 32-channels  (Bank A) are optimized for MIDI and the second 32-channels (Bank B) are optimized for DirectSound and WAV playback.

The OPL3 RAM is shared between Legacy FM Synthesis emulation and the second set (Bank B) of 32 voice channels (32-63).  During legacy emulation, the second 32 voice-channel specific registers (32-63) are not available for use as detailed below. Likewise, when in DirectX 64 voice channel mode, the OPL3 RAM (accessed through the Sound Blaster/Adlib registers) is not available for transfers.

- 4DWave-NX Internal Registers

| Audio Base Offset (Hex) | Byte 3 |  | Byte 2 | Byte 1 | Byte 0 |
| --- | --- | --- | --- | --- | --- |
| 00 | DMAR3 |  | DMAR2 | DMAR1 | DMAR0 |
| 04 | RSVD |  | DMAR6 | DMAR5 | DMAR4 |
| 08 | DMAR11 |  | DMAR10 | RSVD | DMAR8 |
| 0C | DMAR15 |  | DMAR14 | DMAR13 | DMAR12 |
| 10 | SBR3/SBR1 |  | SBR2 | SBR1/SBR3 | SBR0 |
| 14 | SBR6 |  | SBR6 | SBR5 | SBR4 |
| 18 | SBR7 |  | SBR7 | RSVD | RSVD |
| 1C | SBR10 |  | SBR9 | SBR8 | SBR8 |
| 20 | MPUR3 |  | MPUR2 | MPUR1 | MPUR0 |
| 24 | SPCTRL | SPCSO |  |  |  |
| 28 | SPLBA |  |  |  |  |
| 2C | RSVD | SPESO |  |  |  |
| 30 | RSVD |  | RSVD | GAMER1 | GAMER0 |
| 34 | GAMER2 |  |  |  |  |
| 38 | GAMER3 |  |  |  |  |
| 3C | I2SR |  |  |  |  |
| 40 | ACR0 |  |  |  |  |
| 44 | ACR1 |  |  |  |  |
| 48 | ACR2 |  |  |  |  |
| 4C | ACR3 |  |  |  |  |
| 50 | ASR0 |  |  |  |  |
| 54 | RSVD |  | ASR2 | ASR1 |  |
| 58 | ASR3 |  |  |  |  |
| 5C | ASR6 |  | ASR5 | RSVD | ASR4 |
| 60 | AOPLSR0 |  |  |  |  |
| 64 | SPCSTATUS |  |  |  |  |
| 68 | RSVD (Read Only = h'00000000) |  |  | SERRIO |  |
| 6C | TLBC |  |  |  |  |
| 70 | RCI3 |  | RCI2 | RCI1 | RCI0 |
| 74 | RSVD (Read Only = h'000000) |  |  |  | RCI4 |
| 78 | PSBVLD_A Channels 0-31 |  |  |  |  |
| 7C | PSBVLD_B Channels 32-63 |  |  |  |  |
| 80 | START_A Channels 0-31 |  |  |  |  |
| 84 | STOP_A Channels 0-31 |  |  |  |  |
| 88 | DLY_A |  |  |  |  |
| 8C | SIGN_CSO_A |  |  |  |  |
| 90 | CSPF_A Channels 0-31 |  |  |  |  |
| 94 | CEBC_A |  |  |  |  |
| 98 | AIN_A Channels 0-31 |  |  |  |  |
| 9C | EINT_A |  |  |  |  |
| A0 | GC(12:8) +LFOCTRL_A |  | LFOCOUNT_A | GC(7:0) | CIR |
| A4 | AINTEN_A Channels 0-31 |  |  |  |  |
| A8 | MUSICVOL |  |  | WAVEVOL |  |
| AC | RSVD |  |  | DELTA_R |  |
| B0 | MISCINT |  |  |  |  |
| B4 | START_B Channels 32-63 |  |  |  |  |
| B8 | STOP_B Channels 32-63 |  |  |  |  |
| BC | CSPF _B Channels 32-63 |  |  |  |  |
| C0 | SBBL |  |  | SBCL |  |
| C4 | SBE2R |  | RSVD | SBDD | SBCTRL |
| C8 | RSVD |  | STIMER |  |  |
| CC | LFOCTRL_B |  | LFOCOUNT_B | ROM_TEST |  |
| D0 | T_FIFO [FIFO(39:24)] |  |  | T_FIFO [FIFO(19:4)] |  |
| D4 | T_DIGIMIXER [ADL(19:4)] |  |  | T_DIGIMIXER [ADR(19:4)] |  |
| D8 | AIN_B Channels 32-63 |  |  |  |  |
| DC | AINTEN_B Channels 32-63 |  |  |  |  |
| ARAM Channels 0-63 |  |  |  |  |  |
| E0 | DELTA[7:0] |  | CSO[23:0] |  |  |
| E4 | PPTR[31:30] & LBA[29:0] |  |  |  |  |
| E8 | DELTA[15:8] |  | ESO[23:0] |  |  |
| EC | ALPHA(11:4) |  | ALPHA(3:0) + FMS | FMC + RVOL(6:1) | RVOL(0) + CVOL |
| ERAM Channels 0-63 |  |  |  |  |  |
| F0 | GVSEL + PAN |  | VOL | CTRL + Ec(11:8) | Ec(7:0) |
| ERAM Channels 0-31 |  |  |  |  |  |
| F4 | EBUF1 |  |  |  |  |
| F8 | EBUF2 |  |  |  |  |
| FC | RSVD (Read Only = h'00000000) |  |  |  |  |

- NOTE: If SoundBlaster emulation is enabled, only the Bank A (0 – 31) channels can be used. If SoundBlaster emulation is NOT enabled, then all 64 channels in Bank A & B can be used.

## NX - Wave Register Definition

### NX - DMA

#### DMAR0 (Legacy DMA Playback Buffer Base Register Port1)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 00h; DDMABase + 0h, 0000h / 0002h | 8-bits | Read/Write | 00h |
| Bit | Description |  |  |
| 7:0 | Read: Legacy DMA Playback Buffer Current Address [7:0] Write: (Simultaneously writes both) Legacy DMA Playback Buffer Base Address [7:0] Legacy DMA Playback Buffer Current Address [7:0] |  |  |

#### DMAR1 (Legacy DMA Playback Buffer Base Register Port2)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 01h; DDMABase + 1h, 0000h / 0002h | 8-bits | Read/Write | 00h |
| Bit | Description |  |  |
| 7:0 | Read: Legacy DMA Playback Buffer Current Address [15:8] Write: (Simultaneously writes both) Legacy DMA Playback Buffer Base Address [15:8] Legacy DMA Playback Buffer Current Address [15:8] |  |  |

#### DMAR2 (Legacy DMA Playback Buffer Base Register Port3)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 02h; DDMABase + 2h, 0087h / 0083h | 8-bits | Read/Write | 00h |
| Bit | Description |  |  |
| 7:0 | Read: Legacy DMA Playback Buffer Current Address [23:16] Write: (Simultaneously writes both) Legacy DMA Playback Buffer Base Address [23:16] Legacy DMA Playback Buffer Current Address [23:16] |  |  |

#### DMAR3 (Legacy DMA Playback Buffer Base Register Port4 )

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 03h; DDMABase + 3h | 8-bits | Read/Write | 00h |
| Bit | Description |  |  |
| 7:0 | Read: Legacy DMA Playback Buffer Current Address [31:24] Write: (Simultaneously writes both) Legacy DMA Playback Buffer Base Address [31:24] Legacy DMA Playback Buffer Current Address [31:24] |  |  |

#### DMAR4 (Legacy DMA Playback Byte Count Register 1)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 04h; DDMABase + 4h, 0001h/0003h | 8-bits | Read/Write | 00h |
| Bit | Description |  |  |
| 7:0 | Read: Legacy DMA Playback Buffer Current Count [7:0] Write: (Simultaneously writes both) Legacy DMA Playback Buffer Base Count [7:0] Legacy DMA Playback Buffer Current Count [7:0] |  |  |

#### DMAR5 (Legacy DMA Playback Byte Count Register 2)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 05h; DDMABase + 5h, 0001h/0003h | 8-bits | Read/Write | 00h |
| Bit | Description |  |  |
| 7:0 | Read: Legacy DMA Playback Buffer Current Count [15:8] Write: (Simultaneously writes both) Legacy DMA Playback Buffer Base Count [15:8] Legacy DMA Playback Buffer Current Count [15:8] |  |  |

#### DMAR6 (Legacy DMA Playback Byte Count Register 3)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 06h; DDMABase + 6h | 8-bits | Read/Write | 00h |
| Bit | Description |  |  |
| 7:0 | Read: Legacy DMA Playback Buffer Current Count [23:16] Write: (Simultaneously writes both) Legacy DMA Playback Buffer Base Count [23:16] Legacy DMA Playback Buffer Current Count [23:16] |  |  |

#### DMAR8 Legacy DMA Controller Command / Status Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 08h; DDMABase + 8h, 0008h | 8-bits | Read | 00h |
| Bit | Description |  |  |
| 7:0 | Status register for implemented legacy 8237-A DMA channel |  |  |

#### DMAR10 Legacy DMA Single Channel Mask Port

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 0Ah; 000Ah | 0-bits | Write | 00h |
| Bit | Description |  |  |
| x | Channel mask register for implemented legacy 8237-A DMA channel |  |  |

#### DMAR11 Legacy DMA Channel Operation Mode Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 0Bh; DDMABase + Bh, 000Bh | 8-bits | Write | 00h |
| Bit | Description |  |  |
| 7:6 | Legacy DMA transfer request signal timing control 00 = demand mode select 01 = single mode select 10 = block mode select 11 = cascade mode select Not used and reserved for 4Dwave |  |  |
| 5 | Legacy DMA transfer address control 0 = address increment select 1 = address decrement select (not supported by 4DWave) |  |  |
| 4 | Legacy DMA transfer T/C mode control 0 = autoinitialization disable 1 = autoinitialization enable if (Bit 4 == 0) when DMATCReached goes from low to high set DMAR15.0 to 1 else if (Bit 4 == 1) when DMATCReached goes from low to high, set DMA current transfer address 31:0 = DMA base address 31:0 set DMA current count 23:0 = DMA base count 23:0 |  |  |
| 3:2 | Legacy DMA transfer I/O type control 00 = verify transfer (not supported by 4DWave) 01 = write transfer (recording by 4DWave) 10 = read transfer (playback by 4DWave) 11 = illegal & reserved |  |  |
| 1:0 | Channel Select: Not used/reserved |  |  |

#### DMAR12 Legacy DMA Controller First/Last Flag Clear Port

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 0Ch; 000Ch | 0-bits | Write | 00h |
| Bit | Description |  |  |
| x | Clear First/Last flag (First/Last = 0) for implemented legacy 8237-A DMA channel. |  |  |

#### DMAR13 Legacy DMA Controller Master Clear Port

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 0Dh; DDMABase + Dh, 000Dh | 0-bits | Write | 00h |
| Bit | Description |  |  |
| x | Master Clear for implemented legacy 8237-A DMA channel. |  |  |

#### DMAR14 Legacy DMA Controller Clear Mask Port

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 0Eh; 000Eh | 0-bits | Write | 00h |
| Bit | Description |  |  |
| x | Multi-channel mask clear for the implemented legacy 8237-A DMA channel |  |  |

#### DMAR15 Legacy DMA Controller Multi-Channel Mask Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| DDMABase + 0Fh, AudioBase + 0Fh, 000Fh | 1-bits | Write | 0b |
| Bit | Description |  |  |
| x | Multi-channel mask register for implemented legacy 8237-A DMA channel |  |  |

### NX - Sound Blaster / Adlib

#### SBR0 (Legacy FmMusic Bank 0 Register Index / Legacy FmMusic Status)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 10h; SBBase + 0h; SBBase + 8h; AdlibBase + 0h | 8-bits | Read | 00h |
| Bit | Description |  |  |
| 7 | FM Timer Interrupt Flag (Equal to Bit 6 + Bit 5) |  |  |
| 6 | FM Timer 1 Overflowed Flag |  |  |
| 5 | FM Timer 2 Overflowed Flag |  |  |
| 4:0 | Reserved |  |  |
| I/O Port | Size | Access | Default |
| AudioBase + 10h; SBBase + 0h; SBBase + 8h; AdlibBase + 0h | 8-bits | Write | 00h |
| Bit | Description |  |  |
| 7:0 | OPL3 Bank0 Index |  |  |

#### SBR1/SBR3 (Legacy FmMusic Bank 0/1 Register Data Port)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 11h; AudioBase + 13h; SBBase + 1h; SBBase + 3h; SBBase + 9h; AdlibBase + 1h; AdlibBase +3h; | 8-bits | Read/Write | 00h |
| Bit | Description |  |  |
| 7:0 | OPL3 Data register |  |  |

#### SBR2 (Legacy FmMusic Bank 1 Register Index)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 12h; SBBase + 2h; AdlibBase + 2h | 8-bits | Read/Write | 00h |
| Bit | Description |  |  |
| 7:0 | OPL3 Bank1 Index |  |  |

#### SBR4 (Legacy Sound Blaster Mixer Register Index)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 14h; SBBase + 4h | 8-bits | Read/Write | 00h |
| Bit | Description |  |  |
| 7:0 | SoundBlaster 16/Pro Mixer Index Register |  |  |

#### SBR5 (Legacy Sound Blaster Mixer Register Data Port)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 15h; SBBase + 5h | 8-bits | Read/Write | xxh |
| Bit | Description |  |  |
| 7:0 | SoundBlaster 16/Pro Mixer Data register (Index by SBR4) |  |  |

#### SBR6 (Legacy Sound Blaster ESP Reset Port)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 16h, 17h; SBBase + 6h, 7h | 8-bits | Write Only (Read = FFh) | FFh |
| Bit | Description |  |  |
| 7:1 | Reserved |  |  |
| 0 | 0 = Escape From SB16 / SBPRO ESP Reset State 1 = Enter Legacy SB16 / SBPRO ESP Reset State |  |  |

#### SBR7 (Legacy Sound Blaster ESP Data Port)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 1Ah, 1Bh; SBBase + Ah, Bh | 8-bits | Read | AAh |
| Bit | Description |  |  |
| 7:0 | Data returned by Legacy SB16 / SBPRO ESP Read Operation |  |  |

#### SBR8 (Legacy Sound Blaster Command / Status Port)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 1Ch, 1Dh; SBBase + Ch, Dh | 8-bits | Read/Write | 00h |
| Bit | Description |  |  |
| 7 | Write: SB Command or Data Bit [7] of Command (Operator) or Data (Operand ) Written to Legacy SB ESP Read: ESP Busy 0 = Legacy SB ESP is Available For Next Command / Data 1 = Legacy SB ESP is Busy. |  |  |
| 6:0 | Write: SB Command or Data Bit [6:0] of Command (Operator) or Data (Operand ) Written to Legacy SB ESP Read: Reserved Bit [6:0] = Reserved |  |  |

#### SBR9 (Legacy Sound Blaster ESP Data Ready / IRQ Acknowledge Port 1)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 1Eh; SBBase + Eh | 1-bits | Read | 2Ah |
| Bit | Description |  |  |
| 7 | Non-BX Command Data Acknowledge 0 = Data is not available on SBR7 (SBBase + Ah, Bh). 1 = Data is available on SBR7 (SBBase + Ah, Bh).. |  |  |
| 6:0 | Reserved |  |  |

#### SBR10 (Legacy Sound Blaster ESP Data Ready / IRQ Acknowledge Port 2 )

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 1Fh; SBBase + Fh | 1-bits | Read | 2Ah |
| Bit | Description |  |  |
| 7 | BX Command Data Acknowledge 0 = Data is not available on SBR7 (SBBase + Ah, Bh). 1 = Data is available on SBR7 (SBBase + Ah, Bh).. |  |  |
| 6:0 | Reserved |  |  |

### NX - MPU-401 UART

#### MPUR0 (Legacy MPU-401 Data Port / IRQ Acknowledge Port)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 20h; MIDIBase + 0h | 8-bits | Read/Write | xxh (Reset as xxh) |
| Bit | Description |  |  |
| 7:0 | Read: MPU-401 Acknowledge Byte or External MIDI Input Data in MIDI-IN FIFO; Write: MIDI Output Data |  |  |

#### MPUR1 (Legacy MPU-401 Command / Status Port)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 21h; MIDIBase + 1h | 8-bits | Read/Write | 80h |
| Bit | Description |  |  |
| 7 | Read: MPU-401 Status 0 = Ack. Byte is available or External MIDI Input Data is Available in MIDI-IN FIFO; 1 = NO Acknowledge Byte or External MIDI Input Data; Write: MPU-401 Command MIDI Command Data [7] |  |  |
| 6 | Read: MPU-401 Status 0 = Ready for MIDI Data Output or New MIDI Command 1 = MIDI-OUT FIFO is Full Write: MPU-401 Command MIDI Command Data [6] |  |  |
| 5 | Read: MPU-401 Status 0 = MIDI-IN FIFO is not Full 1 = MIDI-IN FIFO is Full Write: MPU-401 Command MIDI Command Data [5] |  |  |
| 4 | Read: MPU-401 Status 0 = MPU401 engine is at PASS-THRU mode 1 = MPU401 engine is at UART mode Write: MPU-401 Command MIDI Command Data [4] |  |  |
| 3:0 | Read: MPU-401 Status Reserved Write: MPU-401 Command MIDI Command Data [3:0] |  |  |

#### MPUR2 (MPU-401 Operation Control  / Status Register)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 22h; MIDIBase + 2h | 8-bits | [7:2] Read/Write [1:0] Read | 10h |
| Bit | Description |  |  |
| 7 | MIDI-IN FIFO Source – Internal Loopback 0 = MIDI-IN FIFO Source From External MIDI-IN Pad 1 = MIDI-IN FIFO Source From MIDI-OUT FIFO (MIDI-OUT pad is also disabled, always high) |  |  |
| 6 | MIDI-OUT Pad Source – External Loopback 0 = External MIDI-OUT Pad Source From MIDI-OUT FIFO; 1 = External MIDI-OUT Pad Source From External MIDI-In Pad; |  |  |
| 5 | MIDI Clock Control 0 = Regular MIDI Clock is being used ; 1 = Fast MIDI Clock (12.288MHz) is being used; |  |  |
| 4 | Pass-Thru Mode UART Enable 0 = MPUR0 Disconnect From MIDI-OUT FIFO; 1 = Connect MPUR0 to MIDI-OUT FIFO; |  |  |
| 3 | MIDI-IN Interrupt Enable 0 = Interrupt will be generated When MIDI-IN FIFO is not Empty; 1 = Interrupt will not be generated When MIDI-IN FIFO is not Empty; |  |  |
| 2 | MIDI-OUT Fifo Full Mask 0 = Normal MIDI-OUT fifo full operation 1 = Mask MIDI-OUT fifo full flag if MIDI-IN and MIDI-OUT fifos are full and internal loopback is enabled |  |  |
| 1 | Read: MIDI-OUT FIFO status 0 = MIDI-OUT FIFO is Empty; 1 = MIDI-OUT FIFO is Not Empty; Write: Reserved |  |  |
| 0 | Read: MIDI-IN FIFO status 0 = MIDI-IN FIFO is Empty; 1 = MIDI-IN FIFO is not Empty; Write: Reserved |  |  |

#### MPUR3 (MPU-401 MIDI-IN FIFO Access Port)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| MIDIBase + 3h AudioBase + 23h | 8-bits | Read | xxh |
| Bit | Description |  |  |
| 7:0 | MIDI Data Serialized In MIDI-IN FIFO |  |  |

### NX - SPDIF

#### SPDIF Control/CSO

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 24h | 32-bits | Read/Write | 00xxxxxxh |
| Bit | Description |  |  |
| 31 | RSVD |  |  |
| 30 | SPDIF Test Mode 0 = SPDIF Test Mode disabled 1= SPDIF Test Mode enabled |  |  |
| 29 | SP Enable Data 0 = SPDIF Data Output Disabled 1 = SPDIF Data Output Enabled |  |  |
| 28 | SP Bus Request 0 = SPDIF Will not request NX PCI master for data 1 = SPDIF Will request NX PCI master for data when needed |  |  |
| 27 | SP Reset 0 = SPDIF Reset active, all SPDIF logic will remain in reset state 1 = SPDIF Reset inactive |  |  |
| 26:24 | SP Mode 000 = 48KHz mode - data source is from the AC97 fifo, no bus mastering 100 = 48KHz mode - data source is from system memory 101 = 32KHz mode - data source is from system memory 110 = 44.1KHz mode - data source is from system memory and external crystal frequency to generate 44.1KHz is 11.2896MHz 111 = 44.1KHz mode - data source is from system memory and external crystal frequency to generate 44.1KHz is 16.9344MHz |  |  |
| 23:0 | CSO is the offset of current sample relative to loop begin sample |  |  |

#### SPDIF LBA

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 28h | 32-bits | Read/Write | xxxxxxxxh |
| Bit | Description |  |  |
| 31:2 | LBA is the linear address of the loop begin sample. |  |  |
| 1:0 | RSVD |  |  |

#### SPDIF ESO

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 2Ch | 32-bits | Read/Write | 00xxxxxxh |
| Bit | Description |  |  |
| 31:24 | RSVD |  |  |
| 23:0 | ESO is the offset of loop end sample relative to loop begin sample (# of Dword samples: 1 sample = 16-bit Left & 16-bit Right). |  |  |

### NX - Game Port

#### GAMER0 (Gameport Control Register)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 30h | 8-bits | Read/Write | 00h |
| Bit | Description |  |  |
| 7 | Enhanced Gameport Enable 0 = Disable Enhanced Digital Gameport; 1 = Enable Enhanced Digital Gameport; |  |  |
| 6 | Enhanced Gameport Test Mode 0 = Disable Testmode for Enhanced Digital Gameport; 1 = Enable Testmode for Enhanced Digital Gameport; |  |  |
| 5:0 | Reserved |  |  |

When Bit [6] is set, the game counter will overflow after every 1024 AC’97 BITCLKs. Bit [6] is only useful when Bit [7] is set.

Bit [7] will be automatically cleared if there are any I/O operation to GAMER1.

#### GAMER1 (Legacy Gameport I/O Register)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 31h; GameBase + 0h – 7h | 8-bits | Read/Write | F0h |
| Bit | Description |  |  |
| 7 | Write: Nothing Read: 0 = Joystick B Button1 Pressed (Input pad at low level) 1 = Joystick B Button1 Released (Input pad at high level) |  |  |
| 6 | Write: Nothing Read: 0 = Joystick B Button0 Pressed (Input pad at low level) 1 = Joystick B Button0 Released (Input pad at high level) |  |  |
| 5 | Write: Nothing Read: 0 = Joystick A Button1 Pressed (Input pad at low level) 1 = Joystick A Button1 Released (Input pad at high level) |  |  |
| 4 | Write: Nothing Read: 0 = Joystick A Button0 Pressed (Input pad at low level) 1 = Joystick A Button0 Released (Input pad at high level) |  |  |
| 3 | Write: Charge the R-C network Read: 0 = Joystick B Y-Axis input pad at low level 1 = Joystick B Y-Axis input pad at high level |  |  |
| 2 | Write: Charge the R-C network Read: 0 = Joystick B X-Axis input pad at low level 1 = Joystick B X-Axis input pad at high level |  |  |
| 1 | Write: Charge the R-C network Read: 0 = Joystick A Y-Axis input pad at low level 1 = Joystick A Y-Axis input pad at high level |  |  |
| 0 | Write: Charge the R-C network Read: 0 = Joystick A X-Axis input pad at low level 1 = Joystick A X-Axis input pad at high level |  |  |

#### GAMER2 (Enhanced Gameport Position Register 1)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 34h | 32-bits | Read/Write | FFFFFFFFh |
| Bit | Description |  |  |
| 31:16 | Joystick A Y-Axis Position Latched Value (16-bit unsigned) |  |  |
| 15:0 | Joystick A X-Axis Position Latched Value (16-bit unsigned) |  |  |

#### GAMER3 (Enhanced Gameport Position Register 2)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 38h | 32-bits | Read/Write | FFFFFFFFh |
| Bit | Description |  |  |
| 31:16 | Joystick B Y-Axis Position Latched Value (16-bit unsigned) |  |  |
| 15:0 | Joystick B X-Axis Position Latched Value (16-bit unsigned) |  |  |

### NX - I2S Registers

#### I2SR	I2S Interface Control

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 3Ch | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:14 | Reserved: Always write a “0” |  |  |
| 13:8 | SCLK Rate: This register contains the number of PCI Clocks between rising edges of SCLK. It is used by software to determine the sample rate conversion factor used by the wave engine. |  |  |
| 7:6 | Reserved: Always write a “0” |  |  |
| 5 | I2S Fifo Full Flag 0 = I2S Fifo not full 1 = I2S fifo full |  |  |
| 4 | I2S L/R sample flag: This is a read-only bit that reflects the state of the “lrclk” input. Software can use this to determine which part of the sample is being received. 0 = Left 1 = Right |  |  |
| 3 | I2S Active: This bit is used to determine if the I2S interface is operating. The I2S specification allows the clock to be stopped. 0 = I2S clock operational 1 = I2S clock stopped |  |  |
| 2:1 | Reserved: Always write a “0” |  |  |
| 0 | I2S Enable: This bit is used to enable the interface to operate and accept data. This bit should be set only after the address engine and the rest of the registers have been configured. 0 = Idle – No Samples accepted 1 = Operational |  |  |

### NX - AC’97 Registers

#### ACR0	AC’97 Command/Status Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 40h | 32-bits | Read/Write | 1b1b0000h |
| Bit | Description |  |  |
| 31 | Secondary AC’97 Codec Power On Control Read: 0 = Power is Off; Will be cleared by “power control” state machine when AC’97 codec is fully powered off. 1 = Power is On; Will be set by “power control” state machine when AC’97 codec is fully powered on. Write: 0 = Do nothing 1 = Start “power control” state machine to turn power codec ON |  |  |
| 30 | Secondary AC’97 Codec Power Off Control Read: 0 = Power is Off; Will be cleared by “power control” state machine when AC’97 codec is fully powered off. 1 = Power is On; Will be set by “power control” state machine when AC’97 codec is fully powered on. Write: 0 = Do nothing 1 = Start “power control” state machine to turn power codec OFF |  |  |
| 29 | RSVD |  |  |
| 28 | Secondary Codec Digital Link (bitclk) Auto Power Down Policy 0 = Do not power off this subsection of the codec when a power down is requested 1 = Power off this subsection of the codec when a power down is requested (default) |  |  |
| 27 | Secondary Codec Analog Mixer w/ Vref Off Auto Power Down Policy 0 = Do not power off this subsection of the codec when a power down is requested 1 = Power off this subsection of the codec when a power down is requested (default) |  |  |
| 26 | Secondary Codec Analog Mixer w/ Vref On Auto Power Down Policy 0 = Do not power off this subsection of the codec when a power down is requested (default) 1 = Power off this subsection of the codec when a power down is requested |  |  |
| 25 | Secondary Codec DAC Auto Power Down Policy 0 = Do not power off this subsection of the codec when a power down is requested 1 = Power off this subsection of the codec when a power down is requested (default) |  |  |
| 24 | Secondary Codec ADC Auto Power Down Policy 0 = Do not power off this subsection of the codec when a power down is requested 1 = Power off this subsection of the codec when a power down is requested (default) |  |  |
| 23 | Primary AC’97 Codec Power On Control Read: 0 = Power is Off; Will be cleared by “power control” state machine when AC’97 codec is fully powered off. 1 = Power is On; Will be set by “power control” state machine when AC’97 codec is fully powered on. Write: 0 = Do nothing 1 = Start “power control” state machine to turn power codec ON |  |  |
| 22 | Primary AC’97 Codec Power Off Control Read: 0 = Power is Off; Will be cleared by “power control” state machine when AC’97 codec is fully powered off. 1 = Power is On; Will be set by “power control” state machine when AC’97 codec is fully powered on. Write: 0 = Do nothing 1 = Start “power control” state machine to turn power codec OFF |  |  |
| 21 | RSVD |  |  |
| 20 | Primary Codec Digital Link (bitclk) Auto Power Down Policy 0 = Do not power off this subsection of the codec when a power down is requested 1 = Power off this subsection of the codec when a power down is requested (default) |  |  |
| 19 | Primary Codec Analog Mixer w/ Vref Off Auto Power Down Policy 0 = Do not power off this subsection of the codec when a power down is requested 1 = Power off this subsection of the codec when a power down is requested (default) |  |  |
| 18 | Primary Codec Analog Mixer w/ Vref On Auto Power Down Policy 0 = Do not power off this subsection of the codec when a power down is requested (default) 1 = Power off this subsection of the codec when a power down is requested |  |  |
| 17 | Primary Codec DAC Auto Power Down Policy 0 = Do not power off this subsection of the codec when a power down is requested 1 = Power off this subsection of the codec when a power down is requested (default) |  |  |
| 16 | Primary Codec ADC Auto Power Down Policy 0 = Do not power off this subsection of the codec when a power down is requested 1 = Power off this subsection of the codec when a power down is requested (default) |  |  |
| 15:10 | RSVD |  |  |
| 9 | Input Slot 3&4 Data Destination 0 = Primary Codec goes to Legacy Record Engine; Secondary Codec goes to PSB Capture (RCI3) Channel 1 = Secondary Codec goes to Legacy Record Engine; Primary Codec goes to PSB Capture (RCI3) Channel |  |  |
| 8 | Reserved: Always write a “0”. |  |  |
| 7 | Reserved: Always write a “0”. |  |  |
| 6 | Secondary Codec Status: Read Only 0 = Secondary AC’97 is not ready for operation 1 = Secondary AC’97 is ready for operation |  |  |
| 5 | Secondary ADC Status: Read Only 0 = Digital Audio Data from the Secondary AC’97 ADC is not valid (Record off) 1 = Digital Audio Data from the Secondary AC’97 ADC is valid (Record on) |  |  |
| 4 | Output Slot 7&8 Enable 0 = Digital Audio Data to the Slot 7&8 AC’97 DAC is not valid (Playback off) 1 = Digital Audio Data to the Slot 7&8 AC’97 DAC is valid (Playback on) If ‘1’ (Enabled), Reverb data from mixer goes into this slot. |  |  |
| 3 | Primary Codec Status: Read Only 0 = Primary AC’97 is not ready for operation 1 = Primary AC’97 is ready for operation |  |  |
| 2 | Primary ADC Status: Read Only 0 = Digital Audio Data from the Primary AC’97 ADC is not valid (Record off) 1 = Digital Audio Data from the Primary AC’97 ADC is valid (Record on) |  |  |
| 1 | Output Slot 3&4 Enable 0 = Digital Audio Data to the Slot 3&4 AC’97 DAC is not valid (Playback off) 1 = Digital Audio Data to the Slot 3&4 AC’97 DAC is valid (Playback on) If ‘1’ (Enabled), Main data from mixer goes into this slot. |  |  |
| 0 | Warm Reset Control 0 = No “warm” reset 1 = Warm reset AC’97 Codec |  |  |

#### ACR1	Primary & Secondary AC’97 Codec Write Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 44h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:16 | Write Data for the AC’97 Codec |  |  |
| 15:12 | Reserved = “0000”. Lower nibble write data for future codec operation |  |  |
| 11 | AC’97 Write Command/Status Bit Read: 0 = Write has completed; Ready to write the AC’97 Codec register 1 = Busy writing AC’97 Codec (indexed by bits [6:0]) Write: 0 = Do Nothing 1 = Write AC’97 Codec register (indexed by bits [6:0]) with bits [31:16] |  |  |
| 10 | Reserved |  |  |
| 9:8 | Codec ID for AC’97 2.0 Codecs 00 = Primary Codec use slot 3 & 4 tag bits (ID field = 00) 01 = Secondary Codec use ID field (slot 3 & 4 tag bits = 0) 10 or 11 = Reserved |  |  |
| 7 | Reserved |  |  |
| 6:0 | Index of the AC’97 Codec register to be written |  |  |

#### ACR2	Primary AC’97 Codec Read Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 48h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:16 | Read Only: Read data return by the AC’97 Codec from last read command. |  |  |
| 15:12 | Reserved = “0000”. Lower nibble read data for future codec operation. |  |  |
| 11 | AC’97 Read Command/Status Bit Read: 0 = Ready to read the AC’97 Codec Register; Last Read command sent 1 = Busy sending read command to AC’97 Codec Write: 0 = Do Nothing 1 = Send read command to AC’97 Codec (indexed by bits [6:0]) |  |  |
| 10 | AC’97 Read Data/Status Bit Read Only: 0 = Data has been received from previous read (Up to software to keep track of which command this is related to.) 1 = Waiting for data to be received from AC’97 Codec; Read command has been sent or is pending (Wait for Codec data indexed by bits [6:0]) |  |  |
| 9:8 | Codec ID = This is implied to be a “00” for the Primary Codec. Read Only. |  |  |
| 7 | Reserved = “0” |  |  |
| 6:0 | Index of the AC’97 Codec register to be read. |  |  |

#### ACR3	Secondary AC’97 Codec Read Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 4Ch | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:16 | Read Only: Read data return by the AC’97 Codec from last read command. |  |  |
| 15:12 | Reserved = “0000”. Lower nibble read data for future codec operation. |  |  |
| 11 | AC’97 Read Command/Status Bit Read: 0 = Ready to read the AC’97 Codec Register; Last Read command sent 1 = Busy sending read command to AC’97 Codec Write: 0 = Do Nothing 1 = Send read command to AC’97 Codec (indexed by bits [6:0]) |  |  |
| 10 | AC’97 Read Data/Status Bit Read Only: 0 = Data has been received from previous read (Up to software to keep track of which command this is related to.) 1 = Waiting for data to be received from AC’97 Codec; Read command has been sent or is pending (Wait for Codec data indexed by bits [6:0]) |  |  |
| 9:8 | Codec ID = This is implied to be a “01” for the Secondary Codec. Read Only. |  |  |
| 7 | Reserved = “0” |  |  |
| 6:0 | Index of the AC’97 Codec register to be read. |  |  |

### NX - General

#### ASR0	4DWave Status Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 50h | 32-bits | Read | 00000000h |
| Bit | Description |  |  |
| 31:28 | Reserved |  |  |
| 27 | SB ESP is at special DMA mode |  |  |
| 26:25 | 00 = SB ESP is at get operator state 01 = SB ESP is at get first operand state 11 = SB ESP is at get second operand state 10 = SB ESP is at get third operand state |  |  |
| 24 | SB Mixer Soft-Reset (If a ‘1’) |  |  |
| 23 | SB PRO Command Captured Most Recently (Non-Bx or Cx Type Command Captured) (If a ‘1’) |  |  |
| 22 | Sb16 Command Captured Most Recently (Bx or Cx Type Command Captured) (If a ‘1’) |  |  |
| 21 | SB Engine Sample Rate Set By Frequency Most Recently (If a ‘1’) |  |  |
| 20 | SB Engine Sample Rate Set By Time Constant Most Recently (If a ‘1’) |  |  |
| 19 | SB16 Mixer Register Update (If a ‘1’) |  |  |
| 18 | SB PRO Mixer Register Update (If a ‘1’) |  |  |
| 17 | OPL3 Bank1 Key On/Off (If a ‘1’) |  |  |
| 16 | OPL3 Bank0 Key On/Off (If a ‘1’) |  |  |
| 15 | This bit corresponds to the AC’97 Codec (Primary or Secondary) which is currently connected to the Legacy Record Engine as controlled by ACR0 bit[9]. 0 = AC-97 codec is not ready 1 = AC-97 codec is ready |  |  |
| 14 | 0 = SB Mixer Register MX0E.1 is 0 1 = SB Mixer Register MX0E.1 is 1 |  |  |
| 13 | 0 = SB ESP is not at Direct Recording Mode 1 = SB ESP is at Direct Recording Mode |  |  |
| 12 | 0 = SB ESP has no ack byte 1 = SB ESP has ack byte that needs to be read out |  |  |
| 11 | 0 = SB ESP DMA Command is not valid 1 = SB ESP DMA command is valid |  |  |
| 10 | 0 = SB ESP Engine at Digital Audio Off State 1 = SB ESP Engine at Digital Audio On State |  |  |
| 9:8 | 00 = SB ESP Engine Command Port Not Busy 01 = SB ESP Engine Command Port Busy 10 = SB ESP DMA Test Busy 11 = SB ESP Command Buffer Full |  |  |
| 7 | 0 = 8 bit data format 1 = 16 bit data format |  |  |
| 6 | 0 = mono 1 = stereo |  |  |
| 5 | 0 = unsigned data format 1 = signed data format |  |  |
| 4 | 0 = playback 1 = recording |  |  |
| 3 | 0 = SB DMA loop disable 1 = SB DMA loop enable |  |  |
| 2:0 | Legacy Command 000 = stop: No operation. No contribution to Digital Mixer 001 = run: Normal operation. 010 = silent_DMA: SBCL will count; CA, CBC won’t count. No data fetching. No interpolation. No contribution to Digital Mixer 011 = reserved 100 = silent_SB: SBCL, CA & CBC will count as the same as run mode.No data fetching. No interpolation. No contribution to Digital Mixer 101 = pause: SBCL, CA & CBC don’t change; let SBALPHA unchanged, CACHE_HIT=1; drive current LD (or LD_L, LD_R) to Digital Mixer 110 = reserved 111 = direct play: SBCL, CA & CBC don’t change. drive SBDD to Digital Mixer |  |  |

Bits [24:16] will be cleared to a ‘0’ after this register is read.

Only one bit of Bit 21 and Bit 20 can be set 1 by implemented SB ESP Engine at any time.

Only one bit of Bit 23 and Bit 22 can be set 1 by implemented SB ESP Engine at any time.

#### ASR1  Legacy Sound Blaster Frequency Read Back Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 54h | 16-bits | Read | AC44h |
| Bit | Description |  |  |
| 15:0 | Sample Frequency Set by SB Command 41h or 42h |  |  |

#### ASR2 Legacy Sound Blaster Time Constant Read Back Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 56h | 8-bits | Read | F5h |
| Bit | Description |  |  |
| 7:0 | Time Constant Value Set by SB Command 40h |  |  |

#### ASR3 4DWave Miscellaneous Legacy Control

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 58h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:2 | General Purpose Registers |  |  |
| 1 | Midi State IRQ Enable 0 = A Midi interrupt will not be generated when the Midi state machine transitions from PASSTHROUGH to UART or when in PASSTHROUGH and a Midi RESET command is received. 1 = A Midi interrupt will be generated according to the above state transitions. |  |  |
| 0 | DMA RD/WR Snoop Disable 0 = NX will snoop all DMA reads and writes. 1 = NX will not snoop any DMA reads or writes (0x00-0x0F). |  |  |

#### ASR4 4DWave Version Control Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 5Ch | 8-bits | Read | 11h |
| Bit | Description |  |  |
| 7:0 | 4DWave Revision: Changed at each tape out of the chip |  |  |

#### ASR5 SB ESP Version High Byte Control Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 5Eh | 8-bits | Read/Write | 04h |
| Bit | Description |  |  |
| 7:4 | Reserved (0’s) |  |  |
| 3:0 | SoundBlaster Revision – High Byte |  |  |

#### ASR6 SB ESP Version Low Byte Control Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 5Fh | 8-bits | Read/Write | 02h |
| Bit | Description |  |  |
| 7:4 | Reserved (0’s) |  |  |
| 3:0 | SoundBlaster Revision – Low Byte |  |  |

### NX - OPL3 Channel Status Register

#### AOPLSR0 OPL3 Emulation Channel Keyon/off Trace Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 60h | 32-bits | Read | 00h |
| Bit | Description |  |  |
| 31:25 | Reserved |  |  |
| 24:16 | Bank1 channel 8-0 key on/off event captured (B0h-B8h) |  |  |
| 15:14 | Reserved |  |  |
| 13:9 | Bank0 rhythm channel 4-0 key on/off event captured (BDh) |  |  |
| 8:0 | Bank0 channel 8-0 key on/off event captured (B0h-B8h) |  |  |

### NX - SPDIF Channel Status Register

#### SPCSTATUS

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 64h | 32-bits | Read/Write | xxxxxxxxh |
| Bit | Description |  |  |
| 31:0 | SPDIF Channel status. These register bits are output as the SPDIF channel status bits: bit 0 in FRAME 0 … bit 31 in FRAME 31 |  |  |

### NX - SERR IO Trap Register

#### SERRIO

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 68h | 16-bits | Read/Write | 0xxxh |
| Bit | Description |  |  |
| 15 | DMA IO Trap Enable 1 = Enable SERR generation on DMA IO addresses (0-F). 0 = SERR generation on DMA addresses is disabled. (default) |  |  |
| 14 | ADLIB IO Trap Enable 1 = Enable SERR generation on ADLIB IO addresses (338-33B or 38C-38F depending on which base is selected in the Legacy IO Base register). 0 = SERR generation on ADLIB addresses is disabled. (default) |  |  |
| 13 | SoundBlaster IO Trap Enable 1 = Enable SERR generation on SoundBlaster IO addresses (220-22F or 240-24F depending on which base is selected in the Legacy IO Base register). 0 = SERR generation on SoundBlaster addresses is disabled. (default) |  |  |
| 12 | MPU401 IO Trap Enable 1 = Enable SERR generation on MPU401 IO addresses (330-333 or 300-303 depending on which base is selected in the Legacy IO Base register). 0 = SERR generation on MPU401 addresses is disabled. (default) |  |  |
| 11 | Game Port IO Trap Enable 1 = Enable SERR generation on Game Port IO addresses (200-207 or 208-20F depending on which base is selected in the Legacy IO Base register). 0 = SERR generation on Game Port addresses is disabled. (default) |  |  |
| 10 | IO trap R/W. This bit is captured from the PCI bus to inform the driver if the cycle that caused the SERR was a read or write cycle. (read only) 1 = Trapped on a Write cycle 0 = Trapped on a Read cycle |  |  |
| 9:0 | IO trap address. This address is captured from the PCI bus to inform the driver which address caused the SERR generated by NX. For an SERR to be generated 1 of the IO SERR enable bits must be set ([15:11]) , the global IO SERR enable must be set in the Legacy DMA register and the main SERR enable bit must be set in the PCI Command register. (read only) |  |  |

### NX - Channel Registers

There are two banks of 32 channels: A & B. The 32 voice channels in Bank A are optimized for MIDI. The 32 voice channels in Bank B are optimized for DirectX Buffers, WDM Streaming Audio, and WAV files. Both Banks support most of the same operations, the chief difference is that Bank A includes the Envelope Slope control that is needed for MIDI. Bank B does not include any Amplitude Slope control. Bank A is available at all times. Bank B is only available when the SoundBlaster register space is not enabled for DOS compatibility, which is the typical case while in Windows.

- Bank A = Channels 0 – 31

- Bank B = Channels 32 – 63

#### TLBC - TLB Control Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 6Ch | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:14 | TLB Base address points to the 16KB aligned TLB used for virtual address mapping to the physical address bases for each 4KB page. |  |  |
| 13:2 | Reserved: Always write a “0” |  |  |
| 1 | TLB Debug bit that force the TLB Cache Address to always be invalid. 0 = TLB operates normally 1 = Force the TLB to always be invalid; This will cause the TLB address to always be fetched before data is transferred. |  |  |
| 0 | TLB Mode: 0 = Physical: Physical_Address = Virtual_Address[31:0] (No translation) 1 = Virtual: Physical_Address = {Physical_Base[31:12], Virtual_Address[11:0]} |  |  |

#### RCI 0, 1, 2, 3 Record Channel Index 0 (Mixer), 1 (Reverb), 2 (Chorus), & 3 (AC’97)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 70h | 32-bits | Read | 00000000h |
| Bit | Description |  |  |
| 31 | RCI3 Enable (Second AC’97 Record Channel) 0 = Disabled; Record channel inactive 1 = Enabled; The channel select by |  |  |
| 30 | Reserved |  |  |
| 29:24 | RCI3 Index – These 6-bits select one of the 64 channels & it’s respective stream buffer as the record channel for this stream. |  |  |
| 23 | RCI2 Enable (Chorus) 0 = Disabled; Record channel inactive 1 = Enabled; The channel select by |  |  |
| 22 | Reserved |  |  |
| 21:16 | RCI2 Index – These 6-bits select one of the 64 channels & it’s respective stream buffer as the record channel for this stream. |  |  |
| 15 | RCI1 Enable (Reverb) 0 = Disabled; Record channel inactive 1 = Enabled; The channel select by |  |  |
| 14 | Reserved |  |  |
| 13:8 | RCI1 Index – These 6-bits select one of the 64 channels & it’s respective stream buffer as the record channel for this stream. |  |  |
| 7 | RCI0 Enable (WDM – Dry Mixer) 0 = Disabled; Record channel inactive 1 = Enabled; The channel select by |  |  |
| 6 | Reserved |  |  |
| 5:0 | RCI0 Index – These 6-bits select one of the 64 channels & it’s respective stream buffer as the record channel for this stream. |  |  |

There are four record channel index registers for Mixer, Reverb, Chorus, and the secondary AC’97 codec PCM record data. These registers specify what “channel”, if enabled, is selected as a record channel. (Note. The respective channel’s START bit must also be set.)

#### LCI Loopback Channel Index (I2S)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 74h | 32-bits | Read | 00000000h |
| Bit | Description |  |  |
| 31:8 | Reserved |  |  |
| 7 | LCI Enable (I2S) 0 = Disabled; Loopback channel inactive 1 = Enabled; Loopback enabled |  |  |
| 6 | Reserved |  |  |
| 5:0 | LCI Index – These 6-bits select one of the 64 channels & it’s respective stream buffer as the loopback channel for this stream. |  |  |

There is on loopback channel and it is used for the I2S data stream. This register specifies what “channel”, if enabled, is selected as a loopback channel. (Note. The respective channel’s START bit must also be set and the address engine correctly configured.)

#### PSBVLD_A Bank A PCI Stream Buffer Valid Flags

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 78h | 32-bits | Read/Write (For testing only) | 00000000h |
| Bit | Description |  |  |
| 31:0 | PCI Stream Buffer (PSB) Valid Status; Bit [n] = Channel [n] 0 = PSB Channel Data Invalid 1 = PSB Channel Data Valid |  |  |

#### PSBVLD_B Bank B PCI Stream Buffer Valid Flags

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 7Ch | 32-bits | Read/Write (For testing only) | 00000000h |
| Bit | Description |  |  |
| 31:0 | PCI Stream Buffer (PSB) Valid Status; Bit [n] = Channel [n] 0 = PSB Channel Data Invalid 1 = PSB Channel Data Valid |  |  |

#### START_A Bank A Start Command and Status Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 80h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:0 | Bit [n] = Channel [n] Read: Channel Status 0 = Stopped 1 = Running Write: Channel Start 0 = Do Nothing; Doesn’t effect start / running operation 1 = Start Channel (Also sets the Channel status bit) |  |  |

This register and the STOP_A register are used as Bank A channel start/stop command registers when they are written, and used as channel running/stopped status register when they are read. Bit [n] is for channel [n].

Stopped

When bit [n] is read as ‘0’, it means that any operation of channel [n], including address generation, sample data fetching, interpolation, and envelope calculation is stopped. And this channel has no contribution to the digital mixer.

This bit will be reset from ‘1’ to ‘0’ in four cases.

1) When a ‘1’ is written to the corresponding bit in register STOP.

2) When out of data, i.e. when sample loop disabled and CSO (Current Sample Offset) >= ESO (End Sample Offset).

3) When Ec (current envelope) drops down to -63.984375 dB.

4) When current envelope buffer is in delay-stop mode, and EDLY count down to ‘0’.

Running

When bit [n] is read as ‘1’, it means channel [n] is working.

This bit will be set from ‘0’ to ‘1’ only when a ‘1’ is written to the corresponding bit in register START_A.

#### STOP_A Bank A STOP Command and Status register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 84h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:0 | Bit [n] = Channel [n] Read: Channel Status (Same as START_A) 0 = Stopped 1 = Running Write: Channel Stop 0 = Do Nothing; Doesn’t effect start / running operation 1 = Stop Channel (Also clears the Channel status bit) |  |  |

#### DLY_A Delay flag register (Only used with Bank A channels)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 88h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:0 | Bit [n] = Channel [n] Read: Delay Status 0 = Normal 1 = Channel is currently in delay mode This bit will toggle from ‘0’ to ‘1’ only when the envelope engine begins to deal with a delay mode buffer. Write: 0 = Do Nothing; Doesn’t change 1 = Set Channel to Delay Mode |  |  |

#### SIGN_CSO_A Sign bit of CSO  (Only used with Bank A channels)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 8Ch | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:0 | Channel Sign Bit for CSO; Bit [n] = Channel [n] Read: 0 = Current Sample Offset (CSO) is greater than or equal to the Loop Begin Address (LBA) 1 = Current Sample Offset (CSO) is less than the Loop Begin Address (LBA) Write: 0 = Do Nothing; Doesn’t change 1 = Set Channel CSO bit to ‘1’ When channel [n] is stopped (STOP_A), bit [n] will be reset to ‘0’. |  |  |

#### CSPF_A Bank A Current Sample Position Flag

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 90h | 32-bits | Read | 00000000h |
| Bit | Description |  |  |
| 31:0 | Channel Current Sample Position; Bit [n] = Channel [n] 0 = Current Sample Offset (CSO) is less than the End Sample Offset/2 (ESO/2) {CSO < ESO/2} 1 = Current Sample Offset (CSO) is greater than or equal to End Sample Offset/2 (ESO/2) {ESO/2 <= CSO} When channel [n] is stopped (STOP_A), bit [n] will be reset to ‘0’. |  |  |

#### CEBC_A Current Envelope Buffer Control (Only used with Bank A channels)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 94h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:0 | Current Envelope Buffer; Bit [n] = Channel [n] Read: 0 = Buffer 1 (EBUF1) 1 = Buffer 2 (EBUF2) Write: 0 = Do Nothing; Doesn’t change 1 = Toggle to other Buffer |  |  |

#### AIN_A Bank A Address Engine Interrupt Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 98h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:0 | Address Engine Interrupts; Bit [n] = Channel [n]; Any bit [n] that changes from a ‘0’ to a ‘1’ (rising edge) with result in and IRQ, if enabled. Read: Address Engine Channel Interrupt Status 0 = No Interrupt 1 = Interrupt This bit will be set in 2 cases: When CSO (current sample offset) >= ESO (end sample offset), and ENDLP_IE (end of loop INT enable bit in Global Control register) =1 and AINTEN_A bit n is set 1 for channel n. When CSO (current sample offset) >= ESO/2 (middle of ESO), and MIDLP_IE (middle of loop INT enable bit in Global Control register) =1 and AINTEN_A bit n is set 1 for channel n. Write: (Write ‘1’ to clear) 0 = Do Nothing; Doesn’t change 1 = Reset bit to ‘0’ |  |  |

#### EINT_A Envelope Engine Interrupt Register (Only used with Bank A channels)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + 9Ch | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:0 | Envelope Engine Interrupts; Bit [n] = Channel [n]; Any bit [n] that changes from a ‘0’ to a ‘1’ (rising edge) with result in and IRQ, if enabled. Read: Address Engine Channel Interrupt Status 0 = No Interrupt 1 = Interrupt This bit will be set in 2 cases: When envelope buffer toggled, and ETOG_IE (envelope toggle INT enable bit in Global Control register) =1. When Ec (current envelope) <= FFFh (-63.984375 dB), and EDROP_IE (envelope dropping to -63.984375dB INT enable bit in Global Control register) =1. Write: (Write ‘1’ to clear) 0 = Do Nothing; Doesn’t change 1 = Reset bit to ‘0’ |  |  |

#### LFO_A & GC & CIR: Bank A Low Frequency Oscillator & Global Control & Channel Index

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + A0h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:30 | Operation Control when recording to mono sample. 00 = left 01 = right 10 = (left+right+1)/2 11= reserved. |  |  |
| 29:28 | DMA Status Reg (I/O 0008h) read handling control bits. 00 = Never assert StatusRDY 01 = StatusRDY = DMATCReached 10 = StatusRDY = DMATCReached ¦ LegacyDRQ 11 = In this case, handshaking with StatusWR and manipulation of return byte should been done. StatusRDY keep ‘0’ when initialization. If(StatusWR ==1) { StatusRDY = 1; if(DMAChannel==0) { ReturnByte[7:0] = {InputByte[7:5], DMAR8[4], InputByte[3:1], DMAR8[0]}; } else { ReturnByte[7:0] = {InputByte[7:6], DMAR8[5], InputByte[4], InputByte[3:2], DMAR8[1], InputByte[0]}; } } if(DMASNOOPCS_==0 & AD[7:0] = 8 & Data_rdy_ == 0 & StatusRDY==1) StatusRDY = 0; |  |  |
| 27 | Test Loopback: This bit is used for wave engine loopback testing. 0 = Normal 1 = Force recording engine get new data from playback FIFO instead of AC’Link. |  |  |
| 26 | Bank A LFO enable bit. 0 = disabled 1 = enabled |  |  |
| 25:24 | Clock rate select of Bank A LFO counter. 00 = LFO counter clock rate is 48kHz 01 = LFO counter clock rate is 48kHz/4 10 = LFO counter clock rate is 48kHz/16 11 = LFO counter clock rate is 48kHz/64 |  |  |
| 23:16 | Initial value of the Bank A LFO counter which will count down to 0 then reload |  |  |
| 15 | EDROP_IE is the INT enable bit for current envelope dropping to -63.984375dB 0 = disable 1 = enable |  |  |
| 14 | ETOG_IE is the INT enable bit for envelope buffer toggling. (Only Bank A) 0 = disable 1 = enable |  |  |
| 13 | MIDLP_IE is the INT enable bit for middle of loop. 0 = disable 1 = enable |  |  |
| 12 | ENDLP_IE is the INT enable bit for end of loop. 0 = disable 1 = enable |  |  |
| 11 | UNDERUN_IE is the INT enable bit for playback underrun 0 = disable 1 = enable When playback FIFO is empty, if this bit is set as ‘1’, an IRQ will be issued. |  |  |
| 10 | OVERUN_IE is the INT enable bit for recording overrun. 0 = disable 1 = enable When recording FIFO is full, if this bit is set as ‘1’, an IRQ will be issued. |  |  |
| 9 | PAUSE is pause command/status bit. Reading from this bit will return the paused/normal status. 0 = Normal 1 = Paused This bit will toggle from ‘1’ to ‘0’ when a ‘0’ is written in; and will toggle from ‘0’ to ‘1’ when a ‘1’ is written in and current bus master cycle has finished. Writing to this bit is the pause/resume command. 0 = Resume 1 = Pause Address engine and envelope engine will be paused after current bus master cycle complete. |  |  |
| 8 | RST_Stimer is used to reset playback sample timer counter. Read : Always return 0; Write: 0 = Do Nothing 1 = Reset Stimer |  |  |
| 7 | PSB/TLB RAM Test Enable 0 = Normal – test mode disabled; E0-ECh & FFh address their defined space 1 = Test Mode; E0-ECh address the 4 Dwords/Channel of PCI Stream Buffering; F0h address the TLB Dword/Channel; CIR is still used as the channel index |  |  |
| 6 | PSB Valid Test Enable 0 = Normal – test mode disabled; Channel Buffer Valid Flags are unaffected 1 = Test Mode; All channel valid flags are always seen as invalid. They are never seen as valid. This forces all bus cycles onto the PCI Bus for debug purposes. |  |  |
| 5:0 | Channel Index: This is used to select a channel for access. For Bank A, 00h selects channel 0 and 1Fh selects channel 31. For Bank B, 20h selects channel 32 and 3F selects channel 63. |  |  |

#### AINTEN_A Bank A Address Engine Interrupt Enable Control

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + A4h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:0 | Address Engine Channel Interrupt Enable; Bit [n] = Channel [n] 0 = Disable address engine interrupt for channel [n] 1 = Enable address engine interrupt for channel [n] |  |  |

#### MUSICVOL & WAVEVOL: Global Music Volume Control & Global Wave Volume Control

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + A8h | 32-bits | Read/Write | 00008080h |
| Bit | Description |  |  |
| 31:24 | Music Right Volume (format of 6.2) 0 0dB (no attenuation) FEh -63.5dB FFh Mute |  |  |
| 23:16 | Music Left Volume (format of 6.2) 0 0dB (no attenuation) FEh -63.5dB FFh Mute |  |  |
| 15:8 | Wave Right Volume (format of 6.2) 0 0dB (no attenuation) FEh -63.5dB FFh Mute |  |  |
| 7:0 | Wave Left Volume (format of 6.2) 0 0dB (no attenuation) FEh -63.5dB FFh Mute |  |  |

#### SBDELTA & DELTA_R: Sample Change Step for Legacy Sound Blaster Voice In/Out & Sample change step for recording

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + ACh | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:16 | RSVD |  |  |
| 15:0 | Delta Change Step for Record (format of 4.12) 48KHz/(record frequency) Delta Change Step for Playback (format 4.12) (playback frequency)/48KHz |  |  |

#### MISCINT Recording/Playback Underrun/Recording Overrun Interrupt Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + B0h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:18 | Reserved (0’s) |  |  |
| 17 | OPLTIMER_IE is the OPL3 timer interrupt enable bit. 0 = disable 1 = enable |  |  |
| 16 | PB_24K_MODE is playback 48k/24k mode control bit. 0 = Wave engine drives sample to CODEC at 48Khz (Default) 1 = Wave engine drives sample to CODEC at 24Khz (In this mode, Delta should be programmed twice as that in 48Khz mode). |  |  |
| 15:13 | Reserved |  |  |
| 12 | SoundBlaster Record Interrupt Control 0 = Generate Interrupt when recording block length expired 1 = Don’t generate Interrupt when recording block length expired |  |  |
| 11 | Mixer_overflow_flag is a flag which indicates the result of mixer accumulator exceeds 7FFFFh. This bit will be set to ‘1’ once accumulator overflows. Write ‘1’ will clear this bit. |  |  |
| 10 | Mixer_underflow_flag is a flag which indicates that the result of mixer accumulator is less than 80000h. This bit will be set to ‘1’ once accumulator underflows. Write ‘1’ will clear this bit. |  |  |
| 9 | REC_OVERUN is recording overrun status bit. Active high. This bit will be set to ‘1’ if the recording is running & rec_req_ is active & data_rdy is not active. |  |  |
| 8 | PB_UNDERUN is the playback FIFO underrun status bit. Active high. This bit will be set to ‘1’ if playback is running & the FIFO is empty & the 48kHz clock is coming. |  |  |
| 7 | Reserved |  |  |
| 6 | ENVELOPE_IRQ is the Wave-table Envelope Engine IRQ bit. Active high. Read Only. Bit[6] = ¦ EINT[31:0] |  |  |
| 5 | ADDRESS_IRQ is the Wave-table Address Engine IRQ bit. Active high. Read Only. Bit[5] = (¦ AINT_A[31:0] ) ¦ (¦AINT_B[31:0]) |  |  |
| 4 | OPL3_IRQ is the legacy OPL3 IRQ bit. Active high. Read Only. Bit[4] = timerirq (signal from Legacy Audio block) |  |  |
| 3 | MPU401_IRQ is the MPU401 IRQ bit. Active high. Read Only. Bit[3] = mpu401irq (signal from Legacy Audio block) |  |  |
| 2 | SB_IRQ is the SoundBlaster IRQ bit. Active high. Read Only. Bit[2] = sbirq (signal from Legacy Audio block) |  |  |
| 1 | REC_OVERUN_IRQ) is recording overrun IRQ bit. Active high. Read Only. Bit[1] = OVERUN_IE & Bit[9] |  |  |
| 0 | PB_UNDERUN_IRQ is playback FIFO underrun IRQ bit. Active high. Read Only. Bit[0] = UNDERUN_IE & Bit[8] |  |  |

Bits [6:0] (read only) are the interrupt request bits. All of these six bits form signal AUDIO_INT.

AUDIO_INT = 	PB_UNDERUN_IRQ | REC_OVERUN_IRQ | SB_IRQ | MPU401_IRQ |

OPL3_IRQ | ADDRESS_IRQ | ENVELOPE_IRQ

#### START_B Bank B Start Command and Status Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + B4h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:0 | Bit [n] = Channel [n] Read: Channel Status 0 = Stopped 1 = Running Write: Channel Start 0 = Do Nothing; Doesn’t effect start / running operation 1 = Start Channel (Also sets the Channel status bit) |  |  |

This register and the STOP_B register are used as Bank B channel start/stop command registers when they are written, and used as channel running/stopped status register when they are read. Bit [n] is for channel [n].

Stopped

When bit [n] is read as ‘0’, it means that any operation of channel [n], including address generation, sample data fetching, interpolation, and envelope calculation is stopped. And this channel has no contribution to the digital mixer.

This bit will be reset from ‘1’ to ‘0’ in four cases.

1) When a ‘1’ is written to the corresponding bit in register STOP.

2) When out of data, i.e. when sample loop disabled and CSO (Current Sample Offset) >= ESO (End Sample Offset).

3) When Ec (current envelope) drops down to -63.984375 dB.

4) When current envelope buffer is in delay-stop mode, and EDLY count down to ‘0’.

Running

When bit [n] is read as ‘1’, it means channel [n] is working.

This bit will be set from ‘0’ to ‘1’ only when a ‘1’ is written to the corresponding bit in register START_B.

#### STOP_B Bank B STOP Command and Status register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + B8h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:0 | Bit [n] = Channel [n] Read: Channel Status (Same as START_B) 0 = Stopped 1 = Running Write: Channel Stop 0 = Do Nothing; Doesn’t effect start / running operation 1 = Stop Channel (Also clears the Channel status bit) |  |  |

#### CSPF_B Bank B Current Sample Position Flag

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + BCh | 32-bits | Read | 00000000h |
| Bit | Description |  |  |
| 31:0 | Channel Current Sample Position; Bit [n] = Channel [n] 0 = Current Sample Offset (CSO) is less than the End Sample Offset/2 (ESO/2) {CSO < ESO/2} 1 = Current Sample Offset (CSO) is greater than or equal to End Sample Offset/2 (ESO/2) {ESO/2 <= CSO} When channel [n] is stopped (STOP_B), bit [n] will be reset to ‘0’. |  |  |

#### SBBL & SBCL: SoundBlaster DMA Base Length & SoundBlaster DMA Current Length

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + C0h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:16 | SBBL: SoundBlaster DMA block length. This corresponds to the number of SAMPLES (8 or 16 bit). 8/16 bit stereo = 2 samples (1 left & 1 right). |  |  |
| 15:0 | SBCL: Current value of the SoundBlaster DMA block length counter |  |  |

If SoundBlaster DMA loop is enabled (SBCTRL[3]=1), every time when SBCL changed from 0 to FFFFh, a INT will be issued, the contents of SBCL is reloaded from SBBL, and DMA operation continues.

If sound blaster DMA loop is not enabled (SBCTRL[3]=0), every time when SBCL changed from 0 to FFFFh, a INT will be issued, the contents of SBCL is reloaded from SBBL, and set LegacyCMD to 101(pause).

SBCTRL bit 7 is used to determine the counter operation mode (8 or 16 bit). The counter is a count down counter.

#### SBCTRL & SBE2R & SBDD: SoundBlaster Control & SoundBlaster DMA Testing Byte Data & SoundBlaster Direct Playback Data

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + C4h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:24 | SBE2R is the SoundBlaster DMA testing byte command data port (write only). Any time after bits [31:24] have been written, E2Status (source from wave engine) will be set high. E2Status will be cleared after the testing byte has been sent to the system location. |  |  |
| 23:16 | Reserved |  |  |
| 15:8 | SBDD is the SoundBlaster direct mode playback data port. |  |  |
| 7 | 0 = 8 bit data format 1 = 16 bit data format |  |  |
| 6 | 0 = mono 1 = stereo |  |  |
| 5 | 0 = unsigned data format 1 = signed data format |  |  |
| 4 | 0 = playback 1 = recording |  |  |
| 3 | SoundBlaster DMA loop enable control 0 = loop disabled. 1 = loop enabled |  |  |
| 2:0 | LegacyCMD 000 = stop: No any operation. No contribution to Digital Mixer 001 = run: Normal operation. 010 = silent_DMA: SBCL will count; CA, CBC won’t count. No data fetching. No interpolation. No contribution to Digital Mixer 011 = reserved 100 = silent_SB: SBCL, CA & CBC will count as the same as run mode. No data fetching. No interpolation. No contribution to Digital Mixer 101 = pause: SBCL, CA & CBC don’t change. Let SBALPHA unchanged, CACHE_HIT=1; drive current LD (or LD_L, LD_R) to Digital Mixer 110 = reserved 111 = Direct_playback: SBCL, CA & CBC don’t change. Drive SBDD to Digital Mixer |  |  |

#### STIMER Sample Timer

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + C8h | 32-bits | Read | 00000000h |
| Bit | Description |  |  |
| 31:24 | Reserved |  |  |
| 23:0 | STIMER will show current state of the sample timer counter. The STIMER will count up every 48kHz clock and will be reset when RST_Stimer bit is being written. |  |  |

#### ROM Test

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + CCh | 16-bits | Read | xxxxh |
| Bit | Description |  |  |
| 15:12 | Reserved (0’s) |  |  |
| 11:0 | Internal ROM Data [11:0] |  |  |

#### LFO_B Bank B Low Frequency Oscillator

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + CEh | 16-bits | Read/Write | 0000h |
| Bit | Description |  |  |
| 15:11 | Reserved (0’s) |  |  |
| 10 | Bank B LFO enable bit. 0 = disabled 1 = enabled |  |  |
| 9:8 | Clock rate select of Bank B LFO counter. 00 = LFO counter clock rate is 48kHz 01 = LFO counter clock rate is 48kHz/4 10 = LFO counter clock rate is 48kHz/16 11 = LFO counter clock rate is 48kHz/64 |  |  |
| 7:0 | Initial value of the Bank B LFO counter which will count down to 0 then reload |  |  |

#### Test Mixer FIFO

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + D0h | 32-bits | Read | xxxxxxxxh |
| Bit | Description |  |  |
| 31:16 | Mixer FIFO Audio Left Sample [19:4] |  |  |
| 15:0 | Mixer FIFO Audio Right Sample [19:4] |  |  |

#### Test Mixer Accumulator

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + D4h | 32-bits | Read | xxxxxxxxh |
| Bit | Description |  |  |
| 31:16 | Mixer Accumulator Audio Left Sample [19:4] |  |  |
| 15:0 | Mixer Accumulator Audio Right Sample [19:4] |  |  |

#### AIN_B Bank B Address Engine Interrupt Register

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + D8h | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:0 | Address Engine Interrupts; Bit [n] = Channel [n]; Any bit [n] that changes from a ‘0’ to a ‘1’ (rising edge) with result in and IRQ, if enabled. Read: Address Engine Channel Interrupt Status 0 = No Interrupt 1 = Interrupt This bit will be set in 2 cases: When CSO (current sample offset) >= ESO (end sample offset), and ENDLP_IE (end of loop INT enable bit in Global Control register) =1 and AINTEN_B bit n is set 1 for channel n. When CSO (current sample offset) >= ESO/2 (middle of ESO), and MIDLP_IE (middle of loop INT enable bit in Global Control register) =1 and AINTEN_B bit n is set 1 for channel n. Write: (Write ‘1’ to clear) 0 = Do Nothing; Doesn’t change 1 = Reset bit to ‘0’ |  |  |

#### AINTEN_B

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + DCh | 32-bits | Read/Write | 00000000h |
| Bit | Description |  |  |
| 31:0 | Address Engine Channel Interrupt Enable; Bit [n] = Channel [n] 0 = Disable address engine interrupt for channel [n] 1 = Enable address engine interrupt for channel [n] |  |  |

### NX - Indexed Channel Registers

There are 64 independent voice channels. The first 32 voice channels are designated Bank A. The second 32 voice channels are designated Bank B. Every voice channel has it’s own parameter register set. Bank A and Bank B parameters differ. These 64 register sets will share the same I/O space. The global register CIR is used to select the current accessible channel.

#### DELTA & CSO: Delta Sample Rate Ratio (LSB) & Current Sample Offset (Bank A & B)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + E0h | 32-bits | Read/Write | xxxxxxxxh |
| Bit | Description |  |  |
| [31:24] | LSB of DELTA[7:0]: DELTA is sample change step in format 4.12 (four bits integer, 12 bits fraction), which stands for the frequency ratio: Fs/48KHz, while Fs is the sum of sample rate and pitch shifting rate. |  |  |
| 23:0 | CSO is the offset of current sample relative to loop begin sample |  |  |

#### PPTR & LBA: PSB Pointer & Loop Begin Address (Bank A & B)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + E4h | 32-bits | Read/Write | xxxxxxxxh |
| Bit | Description |  |  |
| 31:30 | PCI Stream Buffer address pointer; This is used by the hardware to store the current pointer location into each channel’s 4-Dword stream buffer. It can be ignored by software. |  |  |
| 29:0 | Linear address of the loop begin sample. Bit 0 will always be zero in 16-bit sample data mode. When in scatter-gather mode (TLB is enabled), bits [31:24] are ignored |  |  |

#### DELTA & ESO: Delta Sample Rate Ratio (MSB) & End Sample Offset (Bank A & B)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + E8h | 32-bits | Read/Write | xxxxxxxxh |
| Bit | Description |  |  |
| [31:24] | MSB of DELTA[15:8]: DELTA is sample change step in format 4.12 (four bits integer, 12 bits fraction), which stands for the frequency ratio: Fs/48KHz, while Fs is the sum of sample rate and pitch shifting rate. |  |  |
| 23:0 | ESO is the offset of loop end sample relative to loop begin sample. |  |  |

#### ALPHA & FMS & FMC & RVOL & CVOL: Alpha, FM Step, FM Control, Reverb Volume & Chorus Volume (Bank A & B)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + ECh | 32-bits | Read/Write | xxxxxxxxh |
| Bit | Description |  |  |
| 31:20 | ALPHA is sample interpolation coefficient, which stands for the linear interpolation ratio between current sample and the next one. |  |  |
| 19:16 | FMS is Frequency Modulation Step. |  |  |
| 15:14 | FMC - FM Control 00 = (FMS*SIN) >> 3 01 = (FMS*SIN) >> 2 10 = (FMS*SIN) >> 1 11 = (FMS*SIN) >> 0 |  |  |
| 13:7 | RVOL -- Reverb send volume format: 5.2, (0- -31.5dB) 7Fh as mute |  |  |
| 6:0 | CVOL -- Chorus send volume format: 5.2, (0- -31.5dB) 7Fh as mute |  |  |

#### GVSEL & PAN & VOL & CTRL & Ec: Global Volume Select & Positioning Attenuation & Volume Attenuation & Control & Current Envelope (Bank A & B)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + F0h | 32-bits | Read/Write | xxxxxxxxh |
| Bit | Description |  |  |
| 31 | GVSEL is global volume select bit. 0 = select MUSICVOL 1 = select WAVEVOL |  |  |
| 30 | PAN Control - Positioning attenuation control selects attenuated channel. 0 = left 1 = right |  |  |
| 29:24 | PAN Volume - Positioning attenuation value in format of 4.2. Special value is 3Fh, which means mute. |  |  |
| 23:16 | VOL is channel volume attenuation in format of 5.3. 00h stands for 0 dB attenuation, FEh stands for -31.875 dB attenuation. FFh stands for Mute. |  |  |
| 15 | Selects 8/16 bit sample data 0 = 8-bit data 1 = 16-bit data |  |  |
| 14 | Selects mono/stereo sample data 0 = mono 1 = stereo |  |  |
| 13 | Selects unsigned/signed sample data 0 = unsigned 1 = signed |  |  |
| 12 | Loop mode enable bit. 0 = disable 1 = enable |  |  |
| 11:0 | Ec is current envelope in format of 6.6 (Six bits integer and six bits fraction). 000h stands for 0dB, FFFh stands for -63.984375 dB. Note: Ec for Bank A is controlled by EBUF1 or EBUF2. Ec for Bank B is static (does not inc/dec per sample period) and is used for better volume control. |  |  |

#### EBUF1 (Bank A Only)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + F4h | 32-bits | Read/Write | xxxxxxxxh |
| Bit | Description |  |  |
| 31:30 | AMS_H is Amplitude Modulation Step High part. AMS[3:2] |  |  |
| 29:28 | EMOD (Envelope Mode) defines the mode of operation 00 = DEC Mode (ramp from 0dB to –64dB) 01 = INC Mode (ramp from –64dB to 0dB) 10 = DELAY Mode (See Sub-Mode [27:26]) 11 = STILL Mode (Ec is static, buffer never toggles) |  |  |
| 27:26 | DEC Mode = EMAT[11:10] INC Mode = EMAT[11:10] DELAY Mode = Sub-Mode 00 = Delay_hold 01 = Delay_start 10 = Delay_stop 11 = Reserved STILL Mode = Reserved |  |  |
| 25:20 | DEC Mode = EAMT[9:4] INC Mode = EMAT[9:4] Delay Mode = Reserved STILL Mode = Reserved |  |  |
| 19:16 | DEC Mode = EMAT[3:0] INC Mode = EMAT[3:0] Delay Mode = EDLY[19:16] STILL Mode = Reserved |  |  |
| 15:8 | DEC Mode = EINIT[7:0] Provides the initial value of the 8-bit counter ECNT INC Mode = EINIT[7:0] Provides the initial value of the 8-bit counter ECNT Delay Mode = EDLY[15:8] Delay counter STILL Mode = Reserved |  |  |
| 7:0 | DEC Mode = ECNT[5:0] 8-bit Counter INC Mode = ECNT[5:0] Delay Mode = EDLY[7:0] STILL Mode = Reserved |  |  |

EAMT[11:0] = Absolute ramping amount with a range from 0dB to 63.984375dB.

Every time EAMT = 000h, the Envelope Engine will toggle the CEBC flag (Select the other buffer)

ECNT[7:0] = 8-bit Counter; Decrements every 48kHz clock; Whenever it reaches 00h:

ECNT reloads with EINT

EAMT decreases by 1

Ec decreases by 1

EINIT[7:0] = Initial value of ECNT[7:0]

EDLY[19:0] = 20-bit Delay Counter; Decrements every 48kHz clock; Whenever it reaches 00000h:

If Sub-Mode = Delay_hold: Engine will toggle the CEBC flag

If Sub-Mode = Delay_start: Engine with reset the DLY flag

If Sub-Mode = Delay_stop: Engine will reset the START/STOP flag (to stop)

#### EBUF2 (Bank A Only)

| I/O Port | Size | Access | Default |
| --- | --- | --- | --- |
| AudioBase + F8h | 32-bits | Read/Write | xxxxxxxxh |
| Bit | Description |  |  |
| 31:30 | AMS_L is Amplitude Modulation Step Low part. AMS[1:0] |  |  |
| 29:28 | EMOD (Envelope Mode) defines the mode of operation 00 = DEC Mode (ramp from 0dB to –64dB) 01 = INC Mode (ramp from –64dB to 0dB) 10 = DELAY Mode (See Sub-Mode [27:26]) 11 = STILL Mode (Ec is static, buffer never toggles) |  |  |
| 27:26 | DEC Mode = EMAT[11:10] INC Mode = EMAT[11:10] DELAY Mode = Sub-Mode 00 = Delay_hold 01 = Delay_start 10 = Delay_stop 11 = Reserved STILL Mode = Reserved |  |  |
| 25:20 | DEC Mode = EAMT[9:4] INC Mode = EMAT[9:4] Delay Mode = Reserved STILL Mode = Reserved |  |  |
| 19:16 | DEC Mode = EMAT[3:0] INC Mode = EMAT[3:0] Delay Mode = EDLY[19:16] STILL Mode = Reserved |  |  |
| 15:8 | DEC Mode = EINIT[7:0] Provides the initial value of the 8-bit counter ECNT INC Mode = EINIT[7:0] Provides the initial value of the 8-bit counter ECNT Delay Mode = EDLY[15:8] Delay counter STILL Mode = Reserved |  |  |
| 7:0 | DEC Mode = ECNT[5:0] 8-bit Counter INC Mode = ECNT[5:0] Delay Mode = EDLY[7:0] STILL Mode = Reserved |  |  |

EBUF2 is totally as same as EBUF1except bits 31-30 are AMS_L (Amplitude Modulation Step Low part).

EAMT[11:0] = Absolute ramping amount with a range from 0dB to 63.984375dB.

Every time EAMT = 000h, the Envelope Engine will toggle the CEBC flag (Select the other buffer)

ECNT[7:0] = 8-bit Counter; Decrements every 48kHz clock; Whenever it reaches 00h:

ECNT reloads with EINT

EAMT decreases by 1

Ec decreases by 1

EINIT[7:0] = Initial value of ECNT[7:0]

EDLY[19:0] = 20-bit Delay Counter; Decrements every 48kHz clock; Whenever it reaches 00000h:

If Sub-Mode = Delay_hold: Engine will toggle the CEBC flag

If Sub-Mode = Delay_start: Engine with reset the DLY flag

If Sub-Mode = Delay_stop: Engine will reset the START/STOP flag (to stop)